| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0088-v1 |
| --- | --- |

| Title: | CE4.2.3: Affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add affine merge candidates in a unified merge list. The affine merge candidates include spatial inherited affine candidates and constructed affine candidates. The spatial inherited affine candidates are inherited from neighboring blocks and reuse neighboring blocks’ affine model, and the constructed affine candidates use neighboring motion vectors (MVs) as control point MVs to derive affine model. A method to reduce the line buffer requirement for affine merge mode is also added to this proposal. Only one MV row above the current CU is used for deriving affine candidates when the top boundary of the current CU is also a CTU boundary. It is reported that this proposal achieves -0.17% and -0.26% luma BD-rates with +1% and +1% encoding time changes and +7% and +5% decoding time changes for RA and LB, respectively, compared against VTM2.0.1. When compared against the affine merge common base, this proposal reportedly achieves +0.58% and +0.17% luma BD-rates with -5% and -6% encoding time changes for RA and LB, respectively, and it is asserted that decoding time changes are negligible.

# Introduction

In the JVET-K meeting, affine merge mode was discussed and planned as CE4.2 in JVET-K1024 [1]. Each test has to be implemented on top of the common base. The common base will be used as a second anchor for comparison with each test. This contribution includes tests for spatial inherited affine candidates, constructed affine candidates, and line buffer reduction for affine merge mode. The affine merge mode in this contribution is based on a unified merge list.

# Proposed method

In all our tests, the affine merge candidate list in the common base is removed, and the affine candidates are added to the merge candidate list. There are two kinds of affine candidates. The first one is spatial inherited affine candidate. The second one is constructed affine candidates.

## Spatial inherited affine candidates and constructed affine candidates

Spatial inherited affine candidates use affine model from neighboring CUs located at A1, B1, B0, A0, and B2 to derive current CU’s affine model as shown in Figure 1. The maximum number of spatial inherited affine candidates is equal to 5. For the same neighboring block position, spatial inherited affine candidate is always checked first. If the affine candidate is added to the merge list, normal spatial candidate will not be checked. If the affine candidate is available but is pruned by other affine candidates, the normal spatial candidate will be inserted to the merge list without pruning if the MV is available.

To obtain constructed affine candidates, it is proposed to use neighboring MVs as control point MVs. In this contribution, two different constructed affine candidates are evaluated. The first candidate’s control point MVs of (CP1, CP2, CP3) are from MV set of blocks (B2, B1, A1) as shown in Figure 1. The second constructed affine candidate contains temporal MV in the collocated block RB, in which the control point MVs of (CP1, CP2, CP4) are from MV set of blocks (B2, B1, RB) as shown in Figure 1. Available motion triplets of spatial neighbors with the same reference picture are inserted. The second constructed affine candidate will be evaluated only if the first constructed affine candidate is not added to the merge list. The maximum number of constructed affine candidates is equal to 1.

The pruning process for spatial inherited affine candidates and constructed affine candidates are separated. Spatial inherited affine candidates will only be pruned by existing spatial inherited affine candidates. The pruning process for constructed affine candidates is unnecessary since there is at most one constructed affine candidate in the merge list.

The pruning process of spatial inherited affine candidates is done by checking the neighboring affine block’s top-left position. For affine candidate in position A1 and B1, they will not be checked for pruning. For affine candidate in B0, its top-left position will be checked; if the position is the same as the affine block in position B1, the candidate will be pruned. For affine candidate in A0, its top-left position will be checked; if the position is the same as the affine block in position A1, the candidate will be pruned. For affine candidate in B2, its top-left position will be checked, if the position is the same as the affine block in position B1 or position A1, the candidate will be pruned.

In this proposal, spatial inherited affine candidates and constructed affine candidates only consider 6-parameter affine model. Constructed affine candidates are inserted after spatial candidate at location B2.

Figure 1. Position for spatial inherited affine and constructed affine candidates

## Line buffer reduction

Instead of allowing to use all MVs in the upper CTU row, it is proposed to reduce the line buffer requirement by using only one MV row above the current CTU row, as shown in Figure 2.

When deriving the affine candidates from the neighboring blocks, two control point MVs or two sub-block MVs of the neighboring blocks are used. For example, in Figure 2, for block A, the VA2 and VA3 are used to derive the 4-parameter affine model for current block. For block B, the VB2 and VB3 are used to derive the 4-parameter affine model for the current block.

Figure 2. MV buffer used in affine candidate derivation

## Complexity analysis

| Number of | Max number of |  |  | Operations |  |  |  |  | Buffer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| candidates in the final list | inherited affine candidate | constructed affine candidate | candidate comparison |  | add | sht. | cmp. | scal. | additional buffers |
| 6, 8, 10 | 5 | 1 | 4 | Total | 470 | 150 | 232 | 2 | 0 |
|  |  |  |  | inherited | 450 | 150 | 145 | 2 | 0 |
|  |  |  |  | constructed | 20 | 0 | 62 | 0 | 0 |
|  |  |  |  | pruning | 0 | 0 | 25 | 0 | 0 |

# Experiment results

The proposed modifications have been implemented on top of the common base affine merge software [2]. The common test conditions [3] are used for evaluation. Since affine merge in the unified merge list possesses a very different behavior of separate list affine merge, all of the tests are simulated under the condition that the affine merge mode in the common base is turned off.

Tables 1, 2, and 3 show the results of affine merge mode in a unified merge list with spatial inherited affine candidates only (Test A) with maximum number of merge candidates equal to 6, 8, and 10, respectively. Tables 4, 5, and 6 show the results of affine merge mode in a unified merge list with constructed affine candidates only (Test B) with maximum number of merge candidates equal to 6, 8, and 10, respectively. Tables 7, 8, and 9 show the results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates (Test C) with maximum number of merge candidates equal to 6, 8, and 10, respectively. Tables 10, 11, and 12 show the results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates and applying the buffer reduction technique for affine merge mode (Test D) with maximum number of merge candidates equal to 6, 8, and 10, respectively.

Table 13 shows the results of Table 11, but the anchor is changed from VTM2.0.1 to the affine merge common base. The decoding time numbers are not accurate in Table 13. It is asserted that decoding time changes are negligible in this case.

# Conclusions

In this contribution, spatial inherited affine candidates and constructed affine candidates are simplified and added to the normal merge list. A line buffer reduction method is also proposed. The combined results show -0.17% and -0.26% luma BD-rates with +1% and +1% encoding time changes and +7% and +5% decoding time changes for RA and LB, respectively, compared against VTM2.0.1. When compared against the affine merge common base, this proposal reportedly achieves +0.58% and +0.17% luma BD-rates with -5% and -6% encoding time changes and +2% and 0% decoding time changes for RA and LB, respectively.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-K1024, August 2018.

[2]Common Base Software for affine merge in VTM2.0.1: https://vcgit.hhi.fraunhofer.de/chenhuanbang/VVCSoftware_VTM/tree/AffineMergeCommonBase

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, August 2018.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1. Results of affine merge mode in a unified merge list with spatial inherited affine candidates only (Test A) with maximum number of merge candidates equal to 6

Table 2. Results of affine merge mode in a unified merge list with spatial inherited affine candidates only (Test A) with maximum number of merge candidates equal to 8

Table 3. Results of affine merge mode in a unified merge list with spatial inherited affine candidates only (Test A) with maximum number of merge candidates equal to 10

Table 4. Results of affine merge mode in a unified merge list with constructed affine candidates only (Test B) with maximum number of merge candidates equal to 6

Table 5. Results of affine merge mode in a unified merge list with constructed affine candidates only (Test B) with maximum number of merge candidates equal to 8

Table 6. Results of affine merge mode in a unified merge list with constructed affine candidates only (Test B) with maximum number of merge candidates equal to 10

Table 7. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates (Test C) with maximum number of merge candidates equal to 6

Table 8. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates (Test C) with maximum number of merge candidates equal to 8

Table 9. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates (Test C) with maximum number of merge candidates equal to 10

Table 10. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates and applying the buffer reduction technique for affine merge mode (Test D) with maximum number of merge candidates equal to 6

Table 11. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates and applying the buffer reduction technique for affine merge mode (Test D) with maximum number of merge candidates equal to 8

Table 12. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates and applying the buffer reduction technique for affine merge mode (Test D) with maximum number of merge candidates equal to 10

Table 13. Results of affine merge mode in a unified merge list with both spatial inherited affine candidates and constructed affine candidates and applying the buffer reduction technique for affine merge mode with maximum number of merge candidates equal to 8 (Table 11) but the anchor is common base affine merge (the decoding time is not accurate)