| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0099-v1 |
| --- | --- |

| Title: | CE9-related: BIO simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Yu-Chi Su, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, yu-chi.su, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes three simplifications on bi-prediction optical flow (BIO). The first simplification is to reduce operation bit depth by applying bit depth constraints. The second simplification is to reduce the worst case number of extra interpolated samples by disabling BIO for small CUs. The third simplification is to reduce data dependency by applying one fixed filter to intermediate results after horizontal interpolation in MC process for gradient calculation. Simulation results are shown as follows, where the anchor disables BIO.

BIO:
VTM2.0.1-RA: {Y-BD-rate = -1.41%, EncT=105%, DecT=119%}

BIO with simplification 1: Apply bit depth constraints
VTM2.0.1-RA: {Y-BD-rate = -1.38%, EncT=105%, DecT=119%}

BIO with simplification 2: Disable BIO for small CUs
VTM2.0.1-RA: {Y-BD-rate = -1.20%, EncT=103%, DecT=114%}

BIO with simplification 3: Apply one fixed filter to intermediate results after horizontal interpolation in MC process for gradient calculation
VTM2.0.1-RA: {Y-BD-rate = -1.13%, EncT=106%, DecT=123%}

BIO with simplification 1 and simplification 2
VTM2.0.1-RA: {Y-BD-rate = -1.17%, EncT=103%, DecT=116%}

BIO with simplification 1, simplification 2, and simplification 3
VTM2.0.1-RA: {Y-BD-rate = -1.03%, EncT=103%, DecT=116%}

# Introduction

BIO is proposed in VCEG-AZ07 and adopted in JEM. The concept of BIO is to utilize the assumption of optical flow and steady motion to further refine the final predictor in bi-prediction. In this contribution, simplifications to BIO are tested.

# Proposed methods

## Apply bit depth constraints

In the current design of BIO, operations with bit depth higher than 32 are needed. For example, in BIO displacement derivation, several sums of products (s1, s2, …, s6) between “per sample difference between predictor of list 0 (L0) and predictor of list 1 (L1) (predDiff),” “per sample sum of the horizontal gradients of L0 and L1 (gradXSum),” and “per sample sum of vertical gradient of L0 and L1 (gradYSum)” are accumulated in one 6x6 window, as shown in the following equations, where Ix0i',j' and Ix1i',j' represent horizontal gradient values in L0 and L1, Iy0i',j' and Iy1i',j' represent vertical gradient values in L0 and L1, and p0i',j' and p1i',j' represent sample values after MC interpolation in L0 and L1 before right shift to be consistent with input sample bit depth, i.e., 10 bits in common test conditions (CTC). In these equations, the bit depth of predDiff is 15; the bit depths of gradXSum and gradYSum are 12. Therefore, the bit depths of the products are up to 27, and the bit depths of the sums of the products in one 6x6 window are up to 33. To allow 32-bit operations for BIO, it is proposed to right shift 4 bits for three intermediate values in BIO process, including predDiff, gradXSum, and gradYSum.

predDiffi',j'= p(0)i',j'-p1i',j',

gradXSumi',j'=Ix0i',j'+Ix1i',j'

gradYSumi',j'=Iy(0)i',j'+Iy(1)[i',j']

s1=-ΩgradXSum[i', j']2

s2=s4=ΩgradXSum[i', j']gradYSum[i', j']

s3=-ΩpredDiffi',j'gradXSum[i', j']

s5=Ω(gradYSum[i', j'])2

s6=-ΩpredDiffi',j'gradYSum[i', j']

Ω:a 6x6 window

## Disable BIO for small CUs

In the current design of BIO, the worst case number of interpolated samples per 4x4 block is increased from 4x4 to 8x8 when BIO is enabled. The worst case happens when the current CU is 4x4. In order to reduce the worst case number of extra interpolated samples, it is proposed to disable BIO when the CU height or width is smaller than 8. With this CU size constraint, the worst case number of interpolated samples per 4x4 block becomes 5x5 (derived from (20x20) / (4x4)), and the worst case happens when the current CU is 16x16. The worst case number of extra interpolated samples per 4x4 block is reduced from 300% to 56%.

## Apply one fixed filter to intermediate results in MC process for gradient calculation

JVET-K0255 proposed to calculate gradients directly on interpolated samples. In this way, the additional filter sets in JEM7.0 for gradient calculation can be saved, but data dependency between sample interpolation and gradient calculation is introduced. In order to remove this data dependency, it is proposed to apply one fixed filter to intermediate results after horizontal interpolation in MC process.

# Experimental results

The proposed methods have been integrated into BMS2.1 [1]. All the tests were conducted according to the RA common test conditions [2], where the anchor disables BIO. Table 1 shows the results of the current BIO. Table 2 shows the results of BIO with simplification 1 (applying bit depth constraints). Table 3 shows the results of BIO with simplification 2 (disabling BIO for small CUs). Table 4 shows the results of BIO with simplification 3 (applying one fixed filter to intermediate results after horizontal interpolation in MC process for gradient calculation). Table 5 shows the results of BIO with simplification 1 and simplification 2. Table 6 shows the results of BIO with the three simplifications.

Table 1.  Results of BIO

Table 2.  Results of BIO with simplification 1

Table 3.  Results of BIO with simplification 2

Table 4.  Results of BIO with simplification 3

Table 5.  Results of BIO with simplification 1 and simplification 2

Table 6.  Results of BIO with the three simplifications

# Conclusions

Three BIO simplifications are proposed. Applying bit depth constraints, disabling BIO for small CUs, and applying one fixed filter to intermediate results after horizontal interpolation in MC process for gradient calculation are tested and reported.

# References

- BMS2.1: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-2.1

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).