| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0111 |
| --- | --- |

| Title: | Transform Skip Condition on Transform Block Size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution reports that the transform skip (TS) condition in VTM-2.0.1 (TS is possible when the product of TU width and height is equal to or less than 16) does not match with that in VVC draft 2 (TS is possible when both TU width and height are equal to or less than 4), and provides performance comparison of the two different conditions. With VTM-2.0.1 as the anchor, the condition in VVC draft 2 shows that an average of Y(0.00%), U(-0.01%) and V(0.01%) coding gain for AI with run-time change of encoder (76%) and decoder(82%), respectively.

Introduction

In UnitTools.cpp of VTM-2.0.1, TransformSkipFlag(const CodingStructure&cs, const CompArea&area) is used to check whether the transform skip flag should be parsed or not. This function checks the transform skip flag in a different way from the VVC draft 2[1].

Transform Skip Condition on Transform Block size

The current VVC draft 2.0 specifies that transform skip (TS) is possible when both block width and height are equal to or less than 4 (that is, (( log2TbWidth  <=  2 )  &&  ( log2TbHeight  <=  2 )) is true). However, the recently updated reference software code in VTM-2.0.1 parses the transform skip flag when the product of TU width and height is equal to or less than 16. We note that since the minimum TU size of luma signal is 4x4, the two different condition affects the choma signal only. To resolve this ambiguity and see which one is favorable in coding performance, this contribution conducts comparison experiment.

Experiment Result

Experiment is conducted to evaluate the test method under CTC condition as below.

- Anchor:  area.width * area.height  <=16  	(as in VTM-2.0.1)

- Test:  area.width <=4 && area.height <=4  	(as in VVC draft 2.0)

(test results are in the attached excel file JVET-L0111.xlsm)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.02% | 71% | 76% |
| Class A2 | 0.00% | -0.02% | -0.07% | 65% | 70% |
| Class B | 0.01% | 0.04% | 0.06% | 62% | 86% |
| Class C | 0.00% | -0.03% | 0.03% | 98% | 90% |
| Class E | 0.00% | -0.02% | 0.03% | 100% | 81% |
| Overall | 0.00% | -0.01% | 0.01% | 76% | 82% |
| Class D | -0.02% | 0.19% | -0.28% | 101% | 89% |
| Class F (optional) | 0.02% | -0.09% | 0.00% | 83% | 76% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.03% | 0.03% | -0.04% | 67% | 74% |
| Class C | -0.04% | -0.05% | 0.02% | 89% | 94% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | 0.06% | 0.44% | 84% | 92% |
| Class F (optional) | -2.15% | 0.30% | -0.02% | 66% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.01% | 0.03% | 0.16% | 83% | 88% |
| Class E | -0.05% | 0.21% | 0.34% | 85% | 94% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | 0.04% | 0.07% | 88% | 88% |
| Class F (optional) | -0.01% | -1.39% | -1.08% | 79% | 95% |

Conclusion

This proposal provides performance comparison related to transform skip based on transform block size. With VTM-2.0.1 as the anchor, the tested condition shows that an average Y(0.00%), U(-0.01%) and V(0.01%) coding gain for AI with run-time change of encoder (76%) and decoder (82%), respectively.
It is recommended that the VTM-2.0.1 be modified to the tested condition.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” JVET-K1001, 11th meeting, Ljubljana, SI, 10–18 July 2018.

Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).