| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0127 |
| --- | --- |

| Title: | On VVC tile design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution suggests to consider supporting load balanced parallelism and MCTS-based viewport-dependent 360° streaming in VVC tile design. Several tile partition examples are suggested to be considered when evaluating tile design contributions. This contribution also suggests to re-consider HEVC constraint on the relationship between slices and tiles to enable flexible tile partitioning, and consider supporting temporal motion-constrained tile set (MCTS) in VVC tile design .

# Introduction

Tile is specified in HEVC as a partitioning tool to divides a picture horizontally and/or vertically into rectangular regions that can be processed in parallel. The tile syntax format supports both even and uneven tiles.  Compared to slices, tile based parallelism design provides benefits for managing the computational complexity and reducing the memory bandwidth and on-chip memory requirements [1].  In addition, temporal motion-constrained tile sets (MCTS) SEI message allows a decoder to correctly decode the specified MCTS within the pictures of a coded video sequence without needing to decode the entire content of each picture. MCTS enables a form of complexity scalability for region-of-interest decoding and display, provides improved loss robustness, and enables enhanced decoder parallelism [2].

## 360° video viewport-dependent streaming

Several MCTS-based methods for viewport-dependent operation enabled by the OMAF video media profiles are described in OMAF Annex D [3].

OMAF D.4.2 presents an equal-resolution MCTS-based sub-picture approach. Figure 1 shows an example of such tile-based video streaming. A 4×2 tile grid is used in forming of the motion-constrained tile sets. Two HEVC bitstreams originating from the same source content are encoded at different picture qualities and bitrates, but with the same MCTS grid. An extractor track is formed by merging different quality or bitrate HEVC MCTS-based sub-picture tracks of the same resolution based on the viewing orientation into a single bitstream that could be decoded with a single HEVC decoder.

Figure 1 Example of merging of HEVC MCTS-based sub-picture tracks of the same resolution

OMAF D.5.2 presents a MCTS-based approach for achieving 5K effective ERP resolution.  Figure 2 shows such processing example. Four MCTS tiles from high resolution bitstream and 2 MCTS tiles from low resolution bitstream are selected based on the viewing orientation. An extractor track is created for each distinct viewport-adaptive MCTS selection. In the presented example, the bitstreams resolved from the extractor tracks have resolution 2560×3200. Consequently, a 4K-capable decoder could decode content where the viewport is extracted from a coded bitstream with 5K (5120×2560) resolution.

Figure 2 Example of extractor track combining MCTSs from bitstreams of different resolution

OMAF D.6.3 presents a MCTS-based approach for achieving 6K effective ERP resolution for viewport dependent streaming.  Figure 3 shows such example where the source content is resampled at 3 spatial resolutions, namely 6K, 3K and 1,5K. Based on the viewing orientation, an extractor selects four adjacent tiles from the cropped 6K bitstream, four tiles from the cropped 3K bitstream complementing those extracted from the cropped 6K bitstream, the top or bottom stripe MCTS from the 3K resolution depending on whether the targeted viewing orientation is above or below the equator, respectively, and the top or bottom stripe bitstream from the 1.5K resolution depending on whether the viewing orientation is below or above the equator, respectively. The picture size of the bitstream resolved from the extractor track is 3840×2304.

Figure 3 Pre-processing and encoding for achieving 6K effective ERP resolution

OMAF D6.4 presents a MCTS-based approach for achieving 6K effective cubemap resolution for viewport-dependent streaming. In Figure 4, the hatched tiles illustrate an example of the selected high- and low-resolution tiles suitable for a particular range of viewing orientations. An extractor track is created for each distinct viewport-adaptive MCTS selection. This results into 24 extractor tracks being created. Each extractor track uses the same tile grid of 3×6 tiles with tile column widths equal to 768, 768, and 384 luma samples, and a constant tile row height of 768 luma samples. The bitstreams resolved from the extractor tracks have resolution 1920×4608.

Figure 4 Pre-processing and encoding for achieving 6K effective cubemap resolution

## ROI-based load balanced partition for 360° video ERP

A region of interest (ROI) based partition for load balancing is shown in Figure 5. Each tile is a MCTS and encoded independently from each other.

Figure 5 ROI-based partition for load balancing

Table 1 shows each tile’s real processing time using NVidia HEVC NVENC encoder with constant QP configuration running on a i9-7900X 10-core CPU with GTX1080Ti graphic card.  The tile partition in Figure 5 is not supported in HEVC.

Table 1 system time usage for ROI-based partition

| system time (sec.) | tile#1 | tile#2 | tile#3 | tile#4 | tile#5 |
| --- | --- | --- | --- | --- | --- |
| KiteFlite | 3.563 | 4.794 | 4.735 | 5.362 | 3.738 |
| Harbor | 4.250 | 5.059 | 4.532 | 4.617 | 3.281 |
| Trolley | 3.709 | 4.797 | 3.460 | 4.847 | 3.163 |
| Gaslamp | 3.875 | 4.602 | 4.429 | 4.444 | 3.337 |

## Coding parameter signaling on tile

HEVC signals tile configuration information such as column and row locations and loop filter control in the PPS. The coding parameters of the coding blocks within each tile refers to the slice header of its respective slice. Even though slice boundaries need not coincide with tile boundaries, the combination of slices and tiles is constrained to manage decoder implementation complexity, The constraint is that either all coded blocks in a tile must belong to the same slice, or all coded blocks in a slice must belong to the same tile. Figure 6 shows two tile/slice combination examples from [1]. In Figure 6 (a), all coded blocks within the three tiles belong to a single slice; while in Figure 6 (b), all the coded blocks in a slice belong to the same tile. A slice that does not start at the beginning of a tile cannot span multiple tiles. The combination of tile and slice, and the constraints make tile and MCTS partition more complicated.  When designing VVC tile, it is suggested to reconsider the HEVC constraint on the relationship between slices and tiles.

Figure 6 tile and slice combination example

# Proposal

In order to form a HEVC compliant extractor bitstream for the partitioning examples in Figure 3, Figure 4 and Figure 5, slice headers need to be inserted or rewritten which increases the overhead cost and complicates the tile combination process of extractor. It is desirable for VVC tile design to enable MCTS merge and reordering without slice header insertion or rewritten.

In order to progress the work on VVC tile design, we suggest to take the following into account:

- To consider the partitioning examples described in this proposal when evaluating the performance of VVC tile design.

- To re-consider the HEVC constraint on the relationship between slices and tiles to enable flexible tile partitioning.

- To consider directly supporting MCTS in VVC tile design to enable common use cases such as 360° video viewport-dependent streaming.

# Reference

- K. Misra, A. Segall, M. Horowitz, Shilin Xu, A. Fuldseth, Minhua Zhou, "An Overview of Tiles in HEVC", Selected Topics in Signal Processing IEEE Journal of, vol. 7, no. 6, Dec 2013, pp. 969-977.

- JCTVC-M0235, “Motion-constrained tile set SEI message”, April, 2013

- ISO/IEC JTC1/SC29/WG11 N17827, “WD2 of ISO/IEC 23090-2 OMAF 2nd edition”, July, 2018

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).