| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0129_r1 |
| --- | --- |

| Title: | CE1-related: Chroma block coding and size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au andrew.dorrell@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

Prior to VTM-2.0, a single coding tree was used in VVC, resulting in chroma blocks and luma block sizes being related by chroma format. This results in a very small chroma blocks (e.g. 2x2, 2x4, and 4x2) and a corresponding very short feedback path for intra reconstruction. Since the adoption of separate trees in intra, it is possible to apply different constraints in the chroma coding tree compared to the luma coding tree. This contribution proposes to restrict split options in the chroma coding tree for intra such that 2x2, 2x4, and 4x2 blocks do not occur. Moreover, since the remaining chroma blocks of width or height of two have minimum opposing dimension of eight, it is proposed to use a 2x8 or 8x2 coefficient group size instead of a 2x2 coefficient group size. In AI config the results are 0.03%, 0.32%, 0.44% in Y, Cb, and Cr channels, respectively. In RA the corresponding results are 0.02%, 0.32%, and 0.37% and in LDB the corresponding results are -0.04%, -0.12%, 0.00%.

# Introduction

Prior to VTM-2.0, a single coding tree was used in VVC, resulting in chroma blocks and luma block sizes being related by chroma format. This results in a very small chroma blocks (e.g. 2x2, 2x4, and 4x2) and a corresponding very short feedback path for intra reconstruction and intra prediction mode derivation. In particular, the minimum number of samples to process in this feedback loop is reduced to four, from 16 for the case of HEVC. Since the adoption of separate trees in intra, it is possible to apply different constraints in the chroma coding tree compared to the luma coding tree.

In VTM-2.0 and earlier, a 2x2 coefficient group size is used for chroma blocks of width or height of two.

# Description of the proposed method

This proposal has two aspects:

- In chroma, restrictions in the chroma coding tree of intra slices prohibits splits that would result in any 2x2, 2x4, or 4x2 chroma blocks.

- In chroma, blocks of width or height of 2 use a 2x8 or 8x2 coefficient group size, respectively. As a consequence, coefficient groups always contain 16 coefficients (excepting the coefficient group containing the last significant coefficient).

# Experimental results

Simulation results under the JVET common test conditions [1] are presented in Table 1.

Table 1. Chroma block size restriction results.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.18% | -0.03% | 95% | 93% |
| Class A2 | 0.04% | 0.26% | 0.19% | 93% | 95% |
| Class B | 0.01% | 0.17% | 0.31% | 91% | 84% |
| Class C | 0.08% | 0.88% | 1.42% | 90% | 74% |
| Class E | -0.01% | 0.01% | 0.07% | 89% | 96% |
| Overall | 0.03% | 0.32% | 0.44% | 91% | 87% |
| Class D | 0.11% | 1.52% | 1.94% | 89% | 75% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.09% | -0.02% | 101% | 102% |
| Class A2 | 0.02% | 0.18% | 0.04% | 101% | 105% |
| Class B | 0.03% | 0.18% | 0.37% | 100% | 106% |
| Class C | 0.05% | 0.77% | 0.92% | 92% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.32% | 0.37% | 98% | 103% |
| Class D | 0.06% | 0.80% | 1.47% | 88% | 84% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1_jvetk1010.tlist |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.19% | 96% | 102% |
| Class C | 0.03% | 0.08% | 0.19% | 90% | 87% |
| Class E | -0.20% | -0.67% | -0.57% | 90% | 88% |
| Overall | -0.04% | -0.12% | 0.00% | 93% | 93% |
| Class D | -0.03% | 0.56% | 0.23% | 90% | 87% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This input document proposes removal of the chroma blocks of under 16 samples in size, i.e. 2x2, 2x4, and 4x2, and replacing the 2x2 coefficient group with a 2x8 or 8x2 coefficient group for the remaining narrow chroma block sizes.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).