| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0132 |
| --- | --- |

| Title: | CE 6-1.1 (c,d): Fast DST-7/DCT-8 based on DFT and 32 point MTS based on skipping high frequency coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-K0096 [1] AMT replacement and restriction, a simplified MTS was introduced. This contribution reports test results of CE 6-1.1 (c,d), and explains details of algorithm and architecture applied in the proposed MTS simplification, which is composed of three tests: 1) 32-point MTS based on skipping high frequency coefficients (RMTS32), 2) DFT-based DST-7/DCT-8 for both 16-point (DFT16) and 32-point (DFT32), and 3) DFT16 + RMTS32. Additionally, DFT16 + DFT32 on top of RMTS32 and the combinations of MTS candidates selection simplification methods [4], i.e. CE 6-1.1 (a,b), are experimented.

As first test (CE 6-1.1d) results, with 95% (AI), 99% (RA), and 100% (LD) encoding time compared to VTM anchor, BD-rate degradation is 0.11% (AI), 0.03% (RA), and 0.02% (LD).

As second test (CE 6-1.1c) results, with 97% (AI), 99% (RA), and 99% (LD) encoding time compared to VTM anchor, BD-rate degradation is 0.01% (AI), 0.01% (RA), and -0.01% (LD).

As third test results, with 95% (AI), 98% (RA), and 100% (LD) encoding time compared to VTM anchor, BD-rate difference is 0.11% (AI), 0.04% (RA), and -0.04% (LD).

Proposed Method

  - 32-point MTS based on skipping high frequency coefficients (RMTS32):

On top of existing zero-out that up-to top-left 32x32 region is kept, only left (top) half of coefficients are kept if block width (height) is greater than or equal to 32 when MTS flag is 1. Because this zero-out is applied to horizontal and vertical directions independently, RMTS32 is applied to all block shapes. The computational complexity of 32-point MTS in terms of operation count can be reduced by half with RMTS32. From the perspective of memory usage, only half of transform basis vectors in 32-point MTS matrices needs to be stored.

For the region where must be zero, residual coding is skipped in such a way that the associated subblock flags are implicitly derived to be 0, and truncated unary binarization of last coefficient position is adjusted considering maximum feasible position.

1.2) Fast DST-7/DCT-8 based on DFT:

This method is already introduced in JVET-K0096 [1]. If DFT-based DST-7/DCT-8 is employed, multiplication counts of 16-point MTS and 32-point MTS can be reduced by 68% and 77% relative to direct multiply counterpart, respectively.

1.3) MTS Candidates Selection Simplifications:

For notational convenience, Simplifed_1 and Simplied_2 correspond to CE 6-1.1b and CE 6-1.1a, respectively [4]. The combination tests along with these two are also performed.

Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration according to common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1026 [3].

2.1) RMTS32

The applied 16-point MTS and MTS candidate selection method are the same as VTM.

Anchor is BMS 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.02% | -0.02% | 97% | 91% |
| Class A2 | 0.16% | -0.05% | -0.07% | 98% | 95% |
| Class B | 0.10% | 0.02% | 0.03% | 98% | 94% |
| Class C | 0.08% | 0.04% | 0.06% | 97% | 98% |
| Class E | 0.12% | 0.00% | -0.03% | 98% | 96% |
| Overall | 0.11% | 0.01% | 0.01% | 98% | 95% |
| Class D | 0.07% | 0.12% | 0.00% | 97% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | 0.00% | 99% | 99% |
| Class A2 | 0.01% | -0.10% | -0.14% | 98% | 99% |
| Class B | 0.05% | 0.04% | 0.08% | 99% | 100% |
| Class C | 0.05% | 0.13% | 0.07% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | 0.02% | 99% | 100% |
| Class D | -0.01% | 0.11% | -0.06% | 99% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | 0.05% | 100% | 100% |
| Class C | 0.09% | 0.16% | 0.13% | 100% | 101% |
| Class E | -0.07% | 0.03% | -0.80% | 100% | 102% |
| Overall | 0.02% | 0.04% | -0.14% | 100% | 101% |
| Class D | -0.02% | 0.51% | -0.26% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.1.1) Results with inter MTS on

The same RMTS32 is applied to inter MTS.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.02% | 95% | 97% |
| Class A2 | 0.02% | 0.01% | -0.13% | 96% | 99% |
| Class B | 0.06% | -0.08% | -0.03% | 96% | 99% |
| Class C | 0.07% | 0.06% | 0.08% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.01% | 96% | 98% |
| Class D | 0.04% | 0.12% | -0.01% | 97% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.02% | 0.05% | 95% | 97% |
| Class C | 0.09% | 0.24% | 0.14% | 96% | 98% |
| Class E | -0.07% | -0.03% | -0.49% | 97% | 99% |
| Overall | 0.03% | 0.07% | -0.05% | 96% | 98% |
| Class D | 0.10% | 0.21% | 0.13% | 97% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.2) DFT16 + DFT32

The applied MTS candidate selection method are the same as VTM.

Anchor is BMS 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.04% | 96% | 86% |
| Class A2 | 0.01% | 0.00% | -0.03% | 97% | 89% |
| Class B | 0.02% | -0.03% | 0.06% | 97% | 89% |
| Class C | -0.01% | 0.02% | 0.06% | 97% | 94% |
| Class E | 0.00% | -0.01% | -0.01% | 97% | 92% |
| Overall | 0.01% | 0.00% | 0.03% | 97% | 90% |
| Class D | 0.00% | 0.00% | -0.21% | 97% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.00% | 99% | 98% |
| Class A2 | -0.01% | 0.13% | -0.11% | 99% | 98% |
| Class B | 0.03% | -0.04% | 0.18% | 99% | 99% |
| Class C | 0.02% | 0.11% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.04% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.11% | 99% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.05% | 99% | 100% |
| Class C | 0.07% | -0.06% | 0.02% | 99% | 100% |
| Class E | -0.16% | -0.22% | -0.08% | 100% | 102% |
| Overall | -0.01% | -0.06% | 0.01% | 99% | 100% |
| Class D | 0.06% | 0.82% | 0.51% | 99% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.2.1) Results with inter MTS on

The DFT16 and DFT32 are also applied to inter MTS.

Anchor is BMS 2.0.1 with inter MTS on under VTM configuration:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | -0.02% | 96% | 96% |
| Class A2 | 0.01% | 0.00% | -0.02% | 97% | 98% |
| Class B | 0.00% | -0.15% | 0.10% | 97% | 97% |
| Class C | 0.00% | 0.07% | 0.08% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | 0.05% | 97% | 97% |
| Class D | -0.01% | 0.07% | 0.42% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.14% | 96% | 96% |
| Class C | 0.03% | 0.06% | 0.11% | 97% | 95% |
| Class E | -0.03% | -0.01% | 0.61% | 97% | 97% |
| Overall | 0.00% | 0.04% | 0.25% | 97% | 96% |
| Class D | -0.06% | 0.61% | 0.38% | 98% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) DFT16 + RMTS32

The applied MTS candidate selection method are the same as VTM.

Anchor is VTM 2.0.1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.05% | -0.02% | 95% | 89% |
| Class A2 | 0.16% | -0.08% | -0.05% | 96% | 91% |
| Class B | 0.11% | 0.01% | 0.05% | 95% | 91% |
| Class C | 0.07% | 0.07% | 0.18% | 95% | 95% |
| Class E | 0.14% | 0.06% | 0.09% | 96% | 93% |
| Overall | 0.11% | 0.00% | 0.06% | 95% | 92% |
| Class D | 0.08% | 0.12% | -0.17% | 95% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | 0.02% | 98% | 99% |
| Class A2 | 0.02% | 0.06% | -0.08% | 98% | 99% |
| Class B | 0.05% | 0.02% | 0.17% | 99% | 100% |
| Class C | 0.07% | 0.18% | 0.17% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.09% | 98% | 99% |
| Class D | 0.03% | 0.09% | 0.27% | 99% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.14% | 0.20% | 99% | 100% |
| Class C | 0.02% | 0.11% | 0.18% | 99% | 100% |
| Class E | -0.17% | 0.26% | -0.03% | 100% | 102% |
| Overall | -0.04% | 0.05% | 0.14% | 100% | 100% |
| Class D | 0.01% | 0.37% | -0.53% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3.1) Results with inter MTS on

The same DFT16 and RMTS32 are applied to inter MTS.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | 0.05% | 93% | 96% |
| Class A2 | 0.03% | 0.10% | -0.03% | 94% | 97% |
| Class B | 0.10% | -0.08% | 0.06% | 95% | 97% |
| Class C | 0.09% | 0.03% | 0.10% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.02% | 0.05% | 94% | 97% |
| Class D | 0.02% | 0.22% | 0.40% | 96% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.04% | 0.55% | 94% | 96% |
| Class C | 0.09% | 0.14% | 0.04% | 94% | 94% |
| Class E | 0.06% | 0.18% | -0.18% | 96% | 98% |
| Overall | 0.08% | 0.11% | 0.20% | 94% | 96% |
| Class D | 0.07% | 0.31% | -0.06% | 97% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) Combination tests with Simplified_1 and Simplfied_2

Only AI results are provided in the below table.

| DFT16 | DFT32 | RMTS32 | MTS cand. selection | BD-rate |  |  | Enc. time | Dec. time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V |  |  |
| Y | Y | N | VTM | 0.01% | 0.00% | 0.03% | 96.67% | 90.38% |
| Y | Y | N | Simplified_1 | 0.32% | -0.18% | -0.19% | 59.95% | 88.94% |
| Y | Y | N | Simplified_2 | 0.13% | 0.19% | 0.20% | 76.52% | 90.22% |
| Y | N | Y | VTM | 0.11% | 0.00% | 0.06% | 95.41% | 92.07% |
| Y | N | Y | Simplified_1 | 0.40% | -0.16% | -0.15% | 59.51% | 90.03% |
| Y | N | Y | Simplified_2 | 0.22% | 0.18% | 0.21% | 75.97% | 91.80% |
| Y | Y | Y | VTM | 0.12% | 0.03% | 0.05% | 94.93% | 90.24% |
| Y | Y | Y | Simplified_1 | 0.41% | -0.19% | -0.16% | 59.50% | 87.92% |
| Y | Y | Y | Simplified_2 | 0.22% | 0.19% | 0.24% | 75.75% | 90.27% |

Conclusion

The DFT-based DST-7/DCT-8 design still shows little BD-rate loss as JVET-K0096 [1] while enabling quite amount of complexity reduction. Furthermore, the 32-point MTS based on skipping high frequency coefficients also help reduce complexity additively with reasonable BD-rate loss. It is recommended that one of combinations presented in this contribution should be adopted in the next VTM.

Reference

- M. Koo et al., “CE 6.1.11: AMT replacement and restriction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0096, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Salehifar et al., “CE 6-1.1 (a,b): Selection of MTS Candidates”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-LXXXX, 12th meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).