| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0134-v2 |
| --- | --- |

| Title: | CE6-related: Shape adaptive transform selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a tool that selects the horizontal and vertical transforms for intra blocks based on the shape of the transform block. For square blocks the proposed tool selects DCT2 for both directions. For non-square blocks DST7 is used for the direction of shorter dimension of the block and DCT2 is used for the direction of the larger dimension. In the case multiple transform selection (MTS) is enabled, the shape adaptiveness is applied when MTS flag is zero and VTM-2 would use DCT2 for both horizontal and vertical directions. In the case MTS flag is one, the VTM-2 transform selection process is followed.

The proposed tool provides -1.17 %, -0.47 % and -0.09 % bitrate impact for AI, RA and LDB configurations, respectively when MTS is off. When MTS is on, the bitrate impacts are -0.13 %, 
-0.01 % and -0.01 %, respectively.

# Proposed Method

As distribution of the intra prediction error signal appears to correlate with the shape of the block, it is proposed to select different transforms for different directions based on the width and height of the transform block. More precisely, it is proposed to use DST7 for the shorter side of the transform block and DCT2 for the longer side of the block. For square blocks it is proposed to use DCT2 for both horizontal and vertical transforms.

If multiple transform selection (MTS) is enabled, the shape adaptiveness is applied in the case when MTS flag is zero and VTM-2 would use DCT2 for both horizontal and vertical directions. In the case MTS flag is one, the VTM-2 transform selection process is followed. All the three transforms used (DCT2, DST7 and DCT8) are identical to the ones defined in VTM-2.

The table below summarizes the operation when MTS is applied together with the proposed tool.

Table 1. Proposed transform derivation and signalling table with differences to VTM-2 highlighted in green.

| MTS_CU_flag | MTS_Hor_flag | MTS_Ver_flag | Intra |  | Inter |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |
| 0 |  |  | DST7 if width < height, else DCT2 | DST7 if height < width, else DCT2 | DCT2 |  |
| 1 | 0 | 0 | DST7 | DST7 | DCT8 | DCT8 |
|  | 0 | 1 | DCT8 | DST7 | DST7 | DCT8 |
|  | 1 | 0 | DST7 | DCT8 | DCT8 | DST7 |
|  | 1 | 1 | DCT8 | DCT8 | DST7 | DST7 |

# Experimental Results

Full results are provided in the attached Excel sheets. In the first set of results multiple transform selection (MTS) of VTM-2 is switched off for both anchor and test. In the second set of results MTS is enabled and thus generation of results fully follow JVET common test condition. Also result with tool enabled for all slices and only I-slices are provided separately. The table below summarizes the results.

|  | MTS: | Off | Off | On | On |
| --- | --- | --- | --- | --- | --- |
|  | Tool in: | All slices | I-slices | All slices | I-slices |
|  | Y | -1.17 % | -1.17 % | -0.13 % | -0.13 % |
|  | U | -0.62 % | -0.62 % | 0.72 % | 0.72 % |
| AI | V | -0.65 % | -0.65 % | 0.75 % | 0.75 % |
|  | Enc | 106 % | 106 % | 99 % | 99 % |
|  | Dec | 101 % | 101 % | 99 % | 99 % |
|  | Y | -0.47 % | -0.26 % | -0.01 % | -0.01 % |
|  | U | 0.45 % | 0.13 % | 0.77 % | 0.30 % |
| RA | V | 0.39 % | 0.04 % | 0.83 % | 0.30 % |
|  | Enc | 102 % | 101 % | 100 % | 100 % |
|  | Dec | 103 % | 101 % | 98 % | 97 % |
|  | Y | -0.09 % | -0.01 % | -0.01 % | 0.00 % |
|  | U | 0.50 % | 0.31 % | 0.57 % | 0.24 % |
| LB | V | 0.40 % | 0.26 % | 0.63 % | 0.33 % |
|  | Enc | 100 % | 100 % | 100 % | 99 % |
|  | Dec | 102 % | 100 % | 97 % | 95 % |

## Results when MTS is off - tool on in all slices

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43 % | -0.47 % | -0.66 % | 110 % | 104 % |
| Class A2 | -1.15 % | -0.73 % | -0.91 % | 110 % | 101 % |
| Class B | -1.16 % | -0.73 % | -0.65 % | 104 % | 100 % |
| Class C | -0.80 % | -0.39 % | -0.21 % | 104 % | 102 % |
| Class E | -1.45 % | -0.76 % | -0.93 % | 103 % | 99 % |
| Overall | -1.17 % | -0.62 % | -0.65 % | 106 % | 101 % |
| Class D | -0.64 % | -0.19 % | -0.09 % | 103 % | 103 % |
| Class F (optional) | -0.51 % | -0.33 % | -0.47 % | 104 % | 104 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50 % | 0.90 % | 0.65 % | 101 % | 101 % |
| Class A2 | -0.36 % | 0.59 % | 0.13 % | 103 % | 100 % |
| Class B | -0.54 % | 0.22 % | 0.47 % | 102 % | 101 % |
| Class C | -0.45 % | 0.29 % | 0.28 % | 101 % | 110 % |
| Class E |  |  |  |  |  |
| Overall | -0.47 % | 0.45 % | 0.39 % | 102 % | 103 % |
| Class D | -0.31 % | 0.28 % | 0.06 % | 101 % | 104 % |
| Class F (optional) | -0.27 % | 0.07 % | 0.03 % | 100 % | 105 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18 % | 0.20 % | -0.19 % | 101 % | 102 % |
| Class C | -0.12 % | -0.10 % | -0.09 % | 99 % | 100 % |
| Class E | 0.09 % | 1.81 % | 2.05 % | 99 % | 104 % |
| Overall | -0.09 % | 0.50 % | 0.40 % | 100 % | 102 % |
| Class D | -0.04 % | 0.56 % | 0.99 % | 100 % | 97 % |
| Class F (optional) | -0.02 % | 0.14 % | 0.46 % | 98 % | 100 % |

## Results when MTS is off - tool on in I-slices

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30 % | 0.19 % | 0.02 % | 101 % | 98 % |
| Class A2 | -0.26 % | 0.29 % | -0.05 % | 101 % | 98 % |
| Class B | -0.24 % | 0.01 % | 0.13 % | 101 % | 100 % |
| Class C | -0.24 % | 0.11 % | 0.02 % | 102 % | 108 % |
| Class E |  |  |  |  |  |
| Overall | -0.26 % | 0.13 % | 0.04 % | 101 % | 101 % |
| Class D | -0.18 % | 0.11 % | -0.06 % | 102 % | 101 % |
| Class F (optional) | -0.15 % | -0.08 % | -0.13 % | 102 % | 105 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06 % | 0.03 % | -0.26 % | 101 % | 100 % |
| Class C | 0.01 % | -0.13 % | -0.17 % | 99 % | 98 % |
| Class E | 0.04 % | 1.36 % | 1.69 % | 100 % | 103 % |
| Overall | -0.01 % | 0.31 % | 0.26 % | 100 % | 100 % |
| Class D | 0.02 % | 0.12 % | 0.04 % | 100 % | 99 % |
| Class F (optional) | -0.13 % | -0.18 % | 0.34 % | 98 % | 100 % |

## Results when MTS is on (CTC results) – tool on in all slices

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23 % | 1.05 % | 0.80 % | 100 % | 95 % |
| Class A2 | -0.14 % | 0.58 % | 0.37 % | 101 % | 98 % |
| Class B | -0.10 % | 0.62 % | 0.82 % | 95 % | 100 % |
| Class C | -0.06 % | 0.48 % | 0.81 % | 100 % | 99 % |
| Class E | -0.14 % | 1.00 % | 0.87 % | 99 % | 103 % |
| Overall | -0.13 % | 0.72 % | 0.75 % | 99 % | 99 % |
| Class D | -0.07 % | 0.61 % | 0.56 % | 99 % | 104 % |
| Class F (optional) | 0.08 % | 0.29 % | 0.30 % | 100 % | 101 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07 % | 1.09 % | 1.07 % | 99 % | 96 % |
| Class A2 | 0.00 % | 0.90 % | 0.45 % | 99 % | 102 % |
| Class B | 0.00 % | 0.60 % | 0.99 % | 101 % | 98 % |
| Class C | 0.01 % | 0.65 % | 0.72 % | 101 % | 96 % |
| Class E |  |  |  |  |  |
| Overall | -0.01 % | 0.77 % | 0.83 % | 100 % | 98 % |
| Class D | 0.02 % | 0.39 % | 0.60 % | 100 % | 97 % |
| Class F (optional) | 0.07 % | 0.56 % | 0.53 % | 101 % | 95 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05 % | 0.37 % | 0.81 % | 100 % | 93 % |
| Class C | 0.08 % | 0.20 % | 0.13 % | 100 % | 98 % |
| Class E | -0.23 % | 1.42 % | 0.99 % | 101 % | 103 % |
| Overall | -0.01 % | 0.57 % | 0.63 % | 100 % | 97 % |
| Class D | 0.01 % | 0.90 % | 0.54 % | 100 % | 100 % |
| Class F (optional) | 0.13 % | 0.82 % | 0.31 % | 99 % | 99 % |

## Results when MTS is on (CTC results) – tool on in I-slices

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | 0.37 % | 0.21 % | 99 % | 94 % |
| Class A2 | -0.02 % | 0.39 % | 0.16 % | 100 % | 99 % |
| Class B | 0.01 % | 0.23 % | 0.43 % | 101 % | 97 % |
| Class C | 0.00 % | 0.25 % | 0.33 % | 100 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | -0.01 % | 0.30 % | 0.30 % | 100 % | 97 % |
| Class D | 0.00 % | 0.14 % | 0.30 % | 100 % | 99 % |
| Class F (optional) | 0.04 % | 0.31 % | 0.30 % | 100 % | 99 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03 % | 0.02 % | 0.25 % | 99 % | 89 % |
| Class C | 0.05 % | 0.03 % | -0.09 % | 100 % | 97 % |
| Class E | -0.09 % | 0.90 % | 1.01 % | 99 % | 104 % |
| Overall | 0.00 % | 0.24 % | 0.33 % | 99 % | 95 % |
| Class D | 0.01 % | 0.40 % | 0.23 % | 100 % | 94 % |
| Class F (optional) | -0.08 % | 0.61 % | 0.11 % | 99 % | 98 % |

# Conclusion

Selecting transforms based on the dimensions of the transform block seems to be beneficial especially when MTS tool is not used. With a fraction of the encoder runtime increase of MTS, the proposed tool appears to be able to improve intra coding efficiency with more that 1 %. It is proposed to test the impact of the tool in a core experiment during the next meeting cycle.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).