| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3-12 Oct. 2018 | Document: JVET-L0136-v2 |
| --- | --- |

| Title: | CE3 : CCLM with line buffer restriction (Test 5.2.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jaehyun.lim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CCLM line buffer restriction, which is test CE3-5.2.7. This method restricts the line buffer increase at the upper CTU boundary in the CCLM prediction. Experimental results from All Intra configuration for CE3-5.2.7 reportedly show 0.01%, 0.01%, and 0.04% bit-rate increase compared to VTM2.0.1 on Y, Cb, and Cr components, respectively.

Proposed method

The CCLM (cross-component linear model) method in the current VVC specification always uses 2 lines of reconstructed luma reference samples to get the down-sampled collocated luma [1].

In this proposal, to avoid the line buffer increase in the intra prediction, only one luma line (general line buffer in intra prediction) is used to make the downsampled luma samples when the upper reference line is at the CTU boundary.

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1023 [3]. For the VTM based test, the VTM configuration was enabled.

Table 1 and Table 2 show the test results in AI and RA configuration.

Table 1 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.04% | 100% | 99% |
| Class A2 | 0.01% | 0.06% | -0.03% | 100% | 100% |
| Class B | 0.01% | 0.03% | 0.07% | 100% | 98% |
| Class C | 0.00% | 0.01% | 0.09% | 100% | 101% |
| Class E | 0.01% | -0.04% | 0.01% | 100% | 101% |
| Overall | 0.01% | 0.01% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.03% | 100% | 96% |

Table 2 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.17% | 0.07% | 100% | 100% |
| Class A2 | 0.01% | -0.08% | -0.01% | 99% | 99% |
| Class B | 0.03% | 0.04% | 0.04% | 100% | 100% |
| Class C | 0.02% | 0.16% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.16% | -0.16% | 99% | 94% |

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1001, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1021, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).