| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3-12 Oct. 2018 | Document: JVET-L0136-v3 |
| --- | --- |

| Title: | CE3 : CCLM with line buffer restriction (Test 5.2.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jaehyun.lim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CCLM line buffer restriction, which is test CE3-5.2.7. This method restricts the line buffer increase at the upper CTU boundary in the CCLM prediction. Experimental results from All Intra configuration for CE3-5.2.7 reportedly show 0.01%, 0.01%, and 0.04% bit-rate increase compared to VTM2.0.1 on Y, Cb, and Cr components, respectively.

Proposed method

The CCLM (cross-component linear model) method in the current VVC specification always uses 2 lines of reconstructed luma reference samples to get the down-sampled collocated luma [1]. In this proposal, to avoid the line buffer increase in the intra prediction, only one luma line (general line buffer in intra prediction) is used to make the downsampled luma samples when the upper reference line is at the CTU boundary.

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1023 [3]. For the VTM based test, the VTM configuration was enabled.

Table 1 and Table 2 show the test results in AI and RA configuration.

Table 1 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.04% | 100% | 99% |
| Class A2 | 0.01% | 0.06% | -0.03% | 100% | 100% |
| Class B | 0.01% | 0.03% | 0.07% | 100% | 98% |
| Class C | 0.00% | 0.01% | 0.09% | 100% | 101% |
| Class E | 0.01% | -0.04% | 0.01% | 100% | 101% |
| Overall | 0.01% | 0.01% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.03% | 100% | 96% |

Table 2 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.17% | 0.07% | 100% | 100% |
| Class A2 | 0.01% | -0.08% | -0.01% | 99% | 99% |
| Class B | 0.03% | 0.04% | 0.04% | 100% | 100% |
| Class C | 0.02% | 0.16% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.16% | -0.16% | 99% | 94% |

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1001, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1021, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Specification of INTRA_CCLM intra prediction mode with proposed method (section 8.2.4.2.8 in VVC specification)

Inputs to this process are:

- a chroma location ( xCbC, yCbC ) of the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..nTbH − 1 and x = 0..nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << 1, yTbC << 1 )		(8-48)

The variables availL, availT and availTL are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY − 1, yTbY ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY, yTbY − 1 ) as inputs, and the output is assigned to availT.

- The availability of top-left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY − 1, yTbY − 1 ) as inputs, and the output is assigned to availTL.

The variable bCTUboudary is derived as follows:

- bCTUboudary = yCbC & ( ( 1 << ( CtbLog2SizeY − 1 ) − 1) = = 0

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both availL and availT are equal to FALSE, the following applies:

predSamples[ x ][ y ] = 1 << ( BitDepthC − 1 )		(8-49)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * 2 − 1, y= 0..nTbH * 2 − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples samples pY[ x ][ y ] are derived as follows:

    - When availL is equal to TRUE, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..2 * nTbH − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When availT is equal to TRUE, the neighbouring top luma samples pY[ x ][ y ] with x = 0..2 * nTbW − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x − 1 ][ 2 * y ] + 	pY[ 2 * x − 1 ][ 2 * y + 1 ] + 
	2* pY[ 2 * x ][ 2 * y ] +	 2*pY[ 2 * x ][ 2 * y + 1 ] + 	(8-50)
		pY[ 2 * x + 1 ][ 2 * y ] + 	pY[ 2 * x + 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ 0 ][ 2 * y ] + 2*pY[ 0 ][ 2*y + 1 ] + 	(8-51)
		pY[ 1 ][ 2 * y ] + 	pY[ 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 1 ) >> 1	(8-52)

  - When availL is equal to TRUE, the down-sampled neighbouring left luma samples pLeftDsY[ y ] with y = 0..nTbH − 1 are derived as follows:

pLeftDsY[ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ −2 ][ 2 * y ] + 2*pY[ −2 ][ 2 * y + 1 ] + 	(8-53)
		pY[ −3 ][ 2 * y ] + 	pY[ −3 ][ 2 * y + 1 ] + 4 ) >> 3

  - When availT is equal to TRUE and bCTUboudary is equal to FALSE, the down-sampled neighbouring top luma samples pTopDsY[ x ] with x = 0..nTbW − 1 are specified as follows:

    - pTopDsY[ x ] with x = 1..nTbW − 1 is derived as follows:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −2 ] + pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −2 ] +	 2*pY[ 2 * x ][ −1 ] + 	(8-54)
		pY[ 2 * x + 1 ][ −2 ] + pY[ 2 * x + 1 ][ −1 ] + 4 ) >> 3

    - If availTL is equal to TRUE, pTopDsY[ 0 ] is derived as follows:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −2 ] + pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −2 ] + 2*pY[ 0 ][ −1 ] + 	(8-55)
		pY[ 1 ][ −2 ] + pY[ 1 ][ −1 ] + 4 ) >> 3

    - Otherwise, pTopDsY[ 0 ] is derived as follows:

pTopDsY[ 0 ] = ( pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 1 ) >> 1	(8-56)

  - When availT is equal to TRUE and bCTUboudary is equal to TRUE, the down-sampled neighbouring top luma samples pTopDsY[ x ] with x = 0..nTbW − 1 are specified as follows:

    - pTopDsY[ x ] with x = 1..nTbW − 1 is derived as follows:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 	2*pY[ 2 * x ][ −1 ] + pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2	(8-54)

    - If availTL is equal to TRUE, pTopDsY[ 0 ] is derived as follows:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −1 ] + 2*pY[ 0 ][ −1 ] + pY[ 1 ][ −1 ] + 2 ) >> 2	(8-55)

    - Otherwise, pTopDsY[ 0 ] is derived as follows:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ] 		(8-56)

  - The variables nS, xS, yS, k0, k1 are derived as follows:

nS = ( ( availL && availT ) ? Min( nTbW, nTbH ) : ( availL ? nTbH : nTbW ) ) 	(8-57)

xS = 1 << ( ( ( nTbW > nTbH ) && availL && availT ) ? ( Log2( nTbW) − Log2( nTbH ) ) : 0 ) 	(8-58)

yS = 1 << ( ( ( nTbH > nTbW ) && availL && availT ) ? ( Log2( nTbH) − Log2( nTbW ) ) : 0 ) 	(8-59)

k1 = ( ( availL && availT ) ? Log2( nS ) + 1 : Log2( nS ) ) 	(8-60)

k0 = BitDepthC + k1 − 15		(8-61)

  - Variables l, c, ll, lc and k1 are derived as follows:

l = 	( availL ? y=0nS-1pLeftDsYy*yS : 0 ) + 	( availT ? x=0nS-1pTopDsYx*xS : 0 )	(8-62)

c = 	( availL ? y=0nS-1p-1][y*yS : 0 ) + 	( availT ? x=0nS-1px*xS][-1 : 0 )	(8-63)

ll = 	( availL ? y=0nS-1pLeftDsYy*yS2 : 0 ) + 	( availT ? x=0nS-1pTopDsYx*xS2 : 0 )	(8-64)

lc = 	( availL ? y=0nS-1pLeftDsYy*yS*p-1][y*yS : 0 ) + 	(8-65)
	( availT ? x=0nS-1pTopDsYx*xS*px*xS][-1 : 0 )

  - When k0 is greater than 0, the variable l, c, ll, lc and k1 are modified as follows

l = ( l + ( l << ( k0 − 1 ) ) ) >> k0
c = ( c + ( c << ( k0 − 1 ) ) ) >> k0
ll = ( ll + ( ll << ( k0 − 1 ) ) ) >> k0		(8-66)
lc = ( lc + ( lc << ( k0 − 1 ) ) ) >> k0
k1 = k1 − k0

  - The variables a, b, and k are derived as follows:

    - If k1 is equal to 0, the following applies:

k = 0		(8-67)

a = 0		(8-68)

b = 1 << ( BitDepthC − 1) 		(8-69)

    - Otherwise, the following applies:

avgY = l >> k1
errY = l & ( ( 1 << k1 ) − 1 ) 
avgC = c >> k1
errC = c & ( ( 1 << k1 ) − 1 
a1 = lc − ( ( avgY * avgC ) << k1 + avgY * errC + avgC * errY )
a2 = ll − ( ( avgY2 ) << k1 + 2 * avgY * errY )
k2 = ( a1 = = 0 ) ? 0 : Max( 0, Floor( Log2( Abs( a1 ) ) ) − BitDepthC + 2 )
k3 = ( a2 = = 0 ) ? 0 : Max( 0, Floor( Log2( Abs( a2 ) ) ) − 5 )
k4 = k3 − k2 + BitDepthC − 2
a1s = a1 >> k2
a2s = a2 >> k3 		(8-70)
a2t = ( a2s < 32 ) ? 0 : ( ( 1 << ( BitDepthY + 4 ) ) + a2s / 2 ) / a2s
if( a2s < 32 )
  a3 = 0
else if( a2s >= 32 && k4 >= 0 )
  a3 = ( a1s * a2t ) >> k4 
else
  a3 = ( a1s * a2t ) << ( −k4 )
a4 = Clip3( − 28, 28 − 1, a3 )
a5 = a4 << 7
k5 = ( a5 = = 0 ) ? 0 : Floor( Log2( Abs( a5 ) + ( Sign2( a5 ) − 1 ) / 2 ) ) − 5

k = 13 − k5		(8-71)

a = a5 >> k5		(8-72)

b = avgC − ( ( a * avgY ) >> k ) 		(8-73)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-74)