| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3-12 Oct. 2018 | Document: JVET-L0138-v2 |
| --- | --- |

| Title: | CE3-related : Reduced number of reference samples for CCLM parameter calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The CCLM (cross-component linear model) method in the current VTM2 requires complex process to derive the linear model parameter α and β based on the least mean square algorithm. Considering that the derivation process of the linear model parameters is also required in decoder side, it is necessary to reduce the required complexity. In this proposal, the maximum number of reference sample pairs, which are used in the CCLM parameter determination is restricted to predefined number so that complexity reduction factors relative to worst and average cases become 2 and 8, respectively. The modified subsampling position is also introduced in this proposal, and it can improve the proposed reference sample limitation method.

Experimental results show 0.05%, 0.14%, and 0.10% BD-rate losses on Y, Cb, and Cr components respectively, compared to VTM2.0.1 in All Intra configuration, and the experimental results with the modified subsampling position show 0.01%, -0.05%, and -0.16% BD-rate losses on Y, Cb, and Cr components respectively, compared to VTM2.0.1 in All Intra configuration.

Introduction

The CCLM (cross-component linear model) method in the current VTM2.0.1 software obtains the linear model parameter α and β based on the least mean square algorithm as follows[1]:

α= N·∑Ln·Cn-∑Ln·∑CnN·∑Ln·Ln-∑Ln·∑Lnα= N·∑Ln·Cn-∑Ln·∑CnN·∑Ln·Ln-∑Ln·∑Ln                	  	(1)

β= ∑Cn-α·∑LnNβ= ∑Cn-α·∑LnN					(2)

where LnLn represents the down-sampled top and left neighbouring reconstructed luma samples, CnCn indicates the top and left neighbouring reconstructed chroma samples, and value of N N  is equal to twice of the minimum of width and height of the current chroma coding block. For a coding block with a square shape, the above two equations are applied directly. For a non-square coding block, the neighbouring samples of the longer boundary are first subsampled to have the same number of samples as for the shorter boundary.

Table 1 shows the number of operations which are used to derive the linear model parameter α and β based on the least mean square.

Table 1 Number of operations used in the least mean square algorithm

| Operation type | Number of operations in equation (1) and (2) |
| --- | --- |
| Multiplications | 2N+7 |
| Addition | 4N+18 |
| Division | Two operations are required but they are replaced by look up table and shift in VTM2 |

As shown in table 1, the number of operations which are needed to derive the CCLM parameters are increased almost proportionally to the number of N.

The derivation of the parameter α and β are required in both encoding and decoding process of the CCLM prediction. And this process becomes burden in the decoder implementation.

Proposed method

This contribution proposes a simplified CCLM parameter calculation method, which uses the smaller number of reference samples in the determination of the CCLM parameters so that the required computational complexity can be reduced significantly. Please note that the required complexity is proportional to the number of pixels which are involved in CCLM parameter generation process. In this method, the maximum number of N is restricted by Nth, so the reference sample pairs are selected with subsampling if it is required.

Table 2shows the selected number of N in the various chroma block sizes with Nth:

Table 2 Selected number of N in the various chroma block sizes with Nth

| Chroma block size | N in VTM 2.0.1 | Restricted Nth in proposed method |
| --- | --- | --- |
| 2 x 2 | 4 | 2 |
| 2 x n / n x 2 (n > 2) | 4 | 4 |
| 4 x 4 / 4 x n / n x 4 (n > 4) | 8 | 8 |
| 8 x 8 / 8 x n / n x 8 (n > 8) | 16 | 8 |
| 16x16, 16x32, 32x16 | 32 | 8 |
| 32x32 | 64 | 8 |

With the proposed method, the number of N is reduced by half for worst case (i.e. 2x2 block) and up to by eight (i.e. 32x32) for average case.

The modified subsampling pixel position is also proposed. In this proposal, the subsampling pixels can be selected in the inverse positions compared to the previous VTM2, as shown in Figure 1.

Figure 1 The example of the modified subsampling pixel position selection

With this method, the various position of the reference pixels can be considered in the CCLM parameter calculation.

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1023 [3]. For the VTM based test, the VTM configuration was enabled.

Test 1: Nth with Table 2

In this test, the maximum reference sample pair number is restricted as shown in Table 2 in the CCLM parameter calculation. Table 3 and Table 4 show the test results in AI and RA configuration.

Table 3 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.09% | -0.02% | 101% | 102% |
| Class A2 | 0.09% | 0.37% | 0.12% | 102% | 102% |
| Class B | 0.03% | 0.04% | 0.01% | 101% | 103% |
| Class C | 0.02% | 0.16% | 0.30% | 101% | 107% |
| Class E | 0.03% | 0.10% | 0.08% | 101% | 104% |
| Overall | 0.05% | 0.14% | 0.10% | 101% | 104% |
| Class D | 0.03% | 0.32% | -0.10% | 101% | 108% |

Table 4 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.33% | 0.18% | 101% | 101% |
| Class A2 | 0.04% | 0.14% | 0.02% | 101% | 101% |
| Class B | 0.03% | -0.06% | -0.05% | 101% | 103% |
| Class C | 0.03% | 0.21% | 0.34% | 102% | 108% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.13% | 0.11% | 101% | 104% |
| Class D | 0.02% | 0.04% | 0.25% | 102% | 115% |

Test 2: Nth with Table 2 and modified subsampling position

In this test, the maximum reference sample pair number is restricted as shown in Table 2 in the CCLM parameter calculation, and the modified subsampling position is also applied. Table 5 and Table 6 show the test results in AI and RA configuration.

Table 5 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | -0.31% | 101% | 102% |
| Class A2 | 0.04% | 0.19% | -0.03% | 101% | 102% |
| Class B | 0.01% | -0.11% | -0.24% | 101% | 102% |
| Class C | -0.02% | -0.03% | 0.09% | 101% | 106% |
| Class E | -0.01% | -0.11% | -0.35% | 101% | 103% |
| Overall | 0.01% | -0.05% | -0.16% | 101% | 103% |
| Class D | 0.01% | 0.10% | -0.14% | 101% | 108% |

Table 6 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.10% | -0.13% | 101% | 101% |
| Class A2 | 0.05% | 0.06% | -0.15% | 101% | 101% |
| Class B | 0.02% | -0.15% | -0.23% | 101% | 102% |
| Class C | 0.03% | -0.03% | -0.04% | 102% | 108% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.07% | -0.15% | 101% | 103% |
| Class D | 0.03% | -0.31% | 0.09% | 102% | 115% |

Conclusions

To reduce the complexity of CCLM parameter derivation, the limitation of reference sample number for CCLM parameter calculation is proposed. This method can reduce the number of operations in the CCLM prediction with the negligible coding performance changes while the computation complexity is reduced significantly. The modified subsampling position is also introduced in this proposal, and it can bring synergy effect with the proposed reference sample limitation method. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- J. Chen, et al., “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1021, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).