| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0146-v2 |
| --- | --- |

| Title: | CE7-related: Context variable reduction for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reduce context variables for dependent quantization. The add operation to derive the context variable index is also removed. In test1, the number of context variables for significant flag, parity flag, and greater flags are reduced to 74%. In total 48 context variables are saved, which results in 0.09%, 0.11% and 0.18% luma BD-rates with negligible encoding time and decoding time changes for AI, RA, and LB, respectively, compared against VTM2.0.1. In test2, the number of context variables for significant flag, parity flag, and greater flags are reduced to 63%. In total 68 context variables are saved, which results in 0.19%, 0.16% and 0.26% luma BD-rates with negligible encoding time and decoding time changes for AI, RA, and LB, respectively, compared against VTM2.0.1.

Two additional results combining this proposal and context reduction for the position of the last coefficient in JVET-L0096 are provided. Based on the test1, 62 context models are reduced. The combined test results in 0.12%, 0.13% and 0.17% luma BD-rates with negligible encoding time and decoding time changes for AI, RA, and LB, respectively, compared against VTM2.0.1. Based on the test2, 82 context models are reduced. The combined test results in 0.23%, 0.18% and 0.28% luma BD-rates with negligible encoding time and decoding time changes for AI, RA, and LB, respectively, compared against VTM2.0.1.

# Introduction

In JVET-K-meeting, transform coefficient coding and quantization presented in JVET-K0072 [1] were adopted into the VTM2. In this proposal, complexity reduction methods for significant flag, parity flag, greater flags, and Rice parameter derivation are proposed.

# Proposed method

The context modeling design is modified accordingly. The context modelling and binarization depend on the following measures for the local neighbourhood:

•	state:	state of the dependent quantizer

•	sumAbs1: the sum of partially reconstructed absolute levels (absLevel1) after the first pass in the local neighbourhood;

•	sumAbs: the sum of reconstructed absolute levels in the local neighbourhood

Let x and y represent the coordinates of a current scan position inside the transform block.
The selected probability models further depend on the diagonal position d, where d = x + y.

Based on the values of sumAbs1, and d, the probability models for coding sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag are selected. The Rice parameter for binarizing abs_remainder is selected based on the values of sumAbs.

Context modelling for the significance flag

The probability model used for coding the sig_coeff_flag is determined as follows.

In test1,

- If the current colour component represents the luma component, the probability model used is selected among a set of 36 probability models (3×12). The context index ctxIdSig (specifying one of the 54 probability models) is determined according to

ctxIdSig = 12 * max(0, state−1) + min( sumAbs1, 3 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

- Otherwise (the current colour component represents a chroma component), the probability model used is selected among a set of 24 probability models (3×8). The context index ctxIdSig is determined according to

ctxIdSig = 8 * max(0, state−1) + min(sumAbs1, 3 ) + ( d < 2 ? 6 : 0 ) )

In test2,

- If the current colour component represents the luma component, the probability model used is selected among a set of 24 probability models (2×12). The context index ctxIdSig (specifying one of the 54 probability models) is determined according to

ctxIdSig = 12 *(state >> 1) + min( sumAbs1, 3 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

- Otherwise (the current colour component represents a chroma component), the probability model used is selected among a set of 16 probability models (2×8). The context index ctxIdSig is determined according to

ctxIdSig = 8 * (state >> 1) + min(sumAbs1, 3 ) + ( d < 2 ? 6 : 0 ) )

Context modelling for the parity flag

Similar as for the significance flag, different sets of probability models are used for luma and chroma. The same set of probability models is used for both chroma components. For the luma component, one of 17 probability models is selected; for the chroma component, one of 9 probability models is selected. Hence, in total, 26 context models (17 for luma and 9 for chroma) are used for coding the par_level_flag.

In both test1 and test2, the context index ctxIdPar specifying the probability model used is determined as follows:

- If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Otherwise, if the current colour component is the luma component, the context index is set to

ctxIdPar = 1 + min(sumAbs1>>1, 3 ) + ( d == 0 ? 15 : ( d < 3 ? 10 : ( d < 10 ? 5 : 0 ) ) )

- Otherwise (the current colour component is the chroma component), the context index is set to

ctxIdPar = 1 + min(sumAbs1>>1, 3 ) + ( d == 0 ? 5 : 0 )

Context modelling for the greater flags

Similar as for the parity flag, one of 26 context models (17 for luma and 9 for chroma) is used for coding the rem_abs_gt1_flag and rem_abs_gt2_flag. Different sets of context models are used for the rem_abs_gt1_flag and rem_abs_gt2_flag.

In both test1 and test2, the context indexes ctxIdGt1 and ctxIdGt1 specifying the probability models used are determined in the same way as the context index ctxIdPar (see above):

ctxIdGt1 = ctxIdPar
			ctxIdGt2 = ctxIdPar

It should be noted that the presence of the rem_abs_gt1_flag and its context variable do not depend on the value of the directly preceding parity flag.

Rice parameter for binarization of abs_remainder

The non-binary syntax element abs_remainder is binarized using the same class of Rice codes as in HEVC and VTM1. The resulting bins are coded in the bypass mode of the arithmetic coding engine.

In both test1 and test2, the Rice parameter ricePar is determined as follows:

- If sumAbs is less than 15, ricePar is set equal to 0;

- Otherwise, if sumAbs is less than 28, ricePar is set equal to 1;

- Otherwise, ricePar is set equal to 2.

# Experiment results

The proposed modifications have been implemented on top of VTM2.0.1 [2]. The common test conditions [3] are used for evaluation. Table 1 shows the results of this proposal. Table 1 show the result of test1, which is reducing the number of context models to 74%. Table 2 show the result of test2, which is reducing the number of context models to 63%.

Two additional results combining this proposal and the context reduction for the position of the last coefficient in JVET-L0096 [4] are provided. Table 3 shows the results of combining test1 and JVET-L0096. Table 4 shows the results of combining test2 and JVET-L0096.

# Conclusions

This proposal reduces the number of context variables for significant flag, parity flag, and greater flags to 74% with 0.09%, 0.11% and 0.18% luma BD-rates in VTM2.0.1-AI, VTM2.0.1-RA, and VTM2.0.1-LB, respectively. If the number of context variables is further reduced to 63%, in total 68 context variables are saved, which results in 0.19%, 0.16% and 0.26% luma BD-rates. It is reported that the encoding time and decoding time changes for AI, RA, and LB compared against VTM2.0.1 are all negligible.

# References

[1] H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative entropy coding for dependent quantization,” JVET-K0072, July 2018.

[2] VTM-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, August 2018.

[4] M. G. Sarwer, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) “CE7-related: Context modeling of the position of last significant coefficient coding,” JVET-L0096, September 2018

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1. Results of reducing the number of context variables to 74%

Table 2. Results of reducing the number of context variables to 63%

Table 3. Results of combined test for reducing the number of context variables to 74% and JVET-L0096

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.06% | 98% | 98% |
| Class A2 | 0.12% | 0.11% | 0.12% | 98% | 97% |
| Class B | 0.16% | 0.29% | 0.15% | 97% | 94% |
| Class C | 0.15% | 0.21% | 0.23% | 97% | 100% |
| Class E | 0.09% | 0.17% | 0.07% | 97% | 100% |
| Overall | 0.12% | 0.17% | 0.13% | 97% | 98% |
| Class D | 0.24% | 0.43% | 0.03% | 97% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 399767% | 447% |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.01% | 0.08% | 98% | 100% |
| Class A2 | 0.12% | 0.07% | 0.24% | 98% | 100% |
| Class B | 0.12% | 0.07% | 0.06% | 99% | 99% |
| Class C | 0.21% | 0.29% | 0.12% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.12% | 0.11% | 98% | 99% |
| Class D | 0.25% | -0.11% | 0.27% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 884961% | 725% |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | 0.43% | 0.29% | 98% | 101% |
| Class C | 0.29% | 0.43% | 0.03% | 97% | 93% |
| Class E | 0.02% | 0.39% | -0.05% | 97% | 96% |
| Overall | 0.17% | 0.42% | 0.12% | 97% | 97% |
| Class D | 0.31% | 0.40% | 0.06% | 97% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 946201% | 684% |

Table 4. Results of combined test for reducing the number of context variables to 63% and JVET-L0096

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | -0.23% | -0.18% | 99% | 101% |
| Class A2 | 0.32% | -0.14% | -0.16% | 99% | 101% |
| Class B | 0.23% | 0.17% | 0.01% | 98% | 99% |
| Class C | 0.17% | 0.28% | 0.23% | 97% | 100% |
| Class E | 0.12% | 0.22% | 0.01% | 98% | 101% |
| Overall | 0.23% | 0.08% | 0.00% | 98% | 101% |
| Class D | 0.28% | 0.50% | 0.01% | 97% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 403166% | 454% |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | -0.17% | -0.18% | 99% | 102% |
| Class A2 | 0.04% | -0.25% | -0.26% | 98% | 97% |
| Class B | 0.24% | 0.02% | 0.00% | 99% | 100% |
| Class C | 0.22% | 0.30% | 0.18% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.00% | -0.04% | 99% | 99% |
| Class D | 0.31% | 0.11% | 0.33% | 98% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 885260% | 743% |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM-2.0.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.34% | 0.13% | -0.29% | 99% | 99% |
| Class C | 0.36% | 0.32% | -0.05% | 99% | 100% |
| Class E | 0.07% | 0.28% | -0.61% | 99% | 101% |
| Overall | 0.28% | 0.23% | -0.29% | 99% | 100% |
| Class D | 0.43% | 0.34% | -0.97% | 99% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 964656% | 727% |