| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0147 |
| --- | --- |

| Title: | CE2: Subsampled Laplacian calculation (Test 6.1, 6.2, 6.3, and 6.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Jungwon Kang Hahyun Lee Jinho Lee Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr hykim5@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE2.6.1, CE2.6.2, CE2.6.3, and CE2.6.4 tests. In the CE2.6.1 to CE2.6.4 tests, 4x4 block classification methods based on various subsampled Laplacian calculations were tested in order to reduce the computational complexity for block classification of the adaptive loop filter (ALF) in the VVC test model (VTM), especially for the worst case operation count. Compared to VTM, it is reported that the CE2.6.1 to CE2.6.4 tests reduce the operation count of 44.17% in the block classification and the operation count of 17.02% in the total operation of the ALF. Compared to VTM with SIMD disabled, it is reported that the CE2.6.1 test shows average BD-rates of 0.06%, 0.07%, and 0.04% with decoding times of 97%, 97%, and 98% for AI, RA, and LDB cases, respectively. And, the CE2.6.2 test shows average BD-rates of 0.03%, 0.04%, and 0.00% with decoding times of 98%, 97%, and 97% for AI, RA, and LDB cases, respectively. And, the CE2.6.3 test shows average BD-rates of 0.03%, 0.04%, and -0.01% with decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases, respectively. Finally, the CE2.6.4 test shows average BD-rates of 0.09%, 0.11%, and 0.06% with decoding times of 97%, 97%, and 97% for AI, RA, and LDB cases, respectively.

Test descriptions

In the 11th JVET meeting, to reduce complexity of adaptive loop filter (ALF), low-complexity ALF methods including the subsampled sum-modified-Laplacian with 4×4 block classification and 2×2 block classification [1,2] were evaluated in the core experiment 2 (CE2) on in-loop filters [3]. And, for block classification method, 4x4 block classification without the subsampled Laplacian calculation was adopted into VVC test model (VTM) [4]. In order to reduce complexity of 4x4 block classification, four subsampled Laplacian calculation methods were suggested to be evaluated in the CE2 on adaptive loop filter [5] in the 12th JVET meeting.

In the CE2.6.1, CE2.6.2, CE2.6.3, and CE2.6.4 tests, 4x4 block classification using the subsampled Laplacian calculation is performed to classify each 4x4 luma block where filtering operation is performed, thus ALF coefficients may switch for each 4x4 luma block. And, each 4x4 luma block is classified into one out of up to 25 classes. The classification index corresponding to filter index for each luma 4x4 block is derived based on its directionality and a quantized value of activity. To calculate directionality and the quantized value of activity for each 4x4 luma block, each gradient of vertical, horizontal, diagonal 1, and diagonal 2 directions is calculated by summing up the calculated 1-D Laplacian values on the subsampled positions within 8x8 window.

The CE2.6.1 test uses the subsampled 1-D Laplacian calculation where the subsampled positions for calculating vertical, horizontal, diagonal 1, and diagonal 2 gradients are different according to each direction of gradient as shown in Fig. 1. And, V, H, D1, and D2 are the calculated 1-D Laplacian values for vertical, horizontal, diagonal 1, and diagonal 2 directions, respectively. And, the CE2.6.2, CE2.6.3, and CE2.6.4 tests use the unified subsampled 1-D Laplacian calculation positions for all gradients as shown in Fig. 2, Fig. 3, and Fig. 4, respectively.

| (a) Subsampled positions for vertical gradient | (b) Subsampled positions for horizontal gradient |
| --- | --- |
| (c) Subsampled positions for diagonal gradient | (d) Subsampled positions for diagonal gradient |

Fig. 1	Subsampled Laplacian calculation for CE2.6.1

| (a) Subsampled positions for vertical gradient | (b) Subsampled positions for horizontal gradient |
| --- | --- |
| (c) Subsampled positions for diagonal gradient | (d) Subsampled positions for diagonal gradient |

Fig. 2	Subsampled Laplacian calculation for CE2.6.2

| (a) Subsampled positions for vertical gradient | (b) Subsampled positions for horizontal gradient |
| --- | --- |
| (c) Subsampled positions for diagonal gradient | (d) Subsampled positions for diagonal gradient |

Fig. 3	Subsampled Laplacian calculation for CE2.6.3

| (a) Subsampled positions for vertical gradient | (b) Subsampled positions for horizontal gradient |
| --- | --- |
| (c) Subsampled positions for diagonal gradient | (d) Subsampled positions for diagonal gradient |

Fig. 4	Subsampled Laplacian calculation for CE2.6.4

The rest of derivation process for directionality and the quantized value of activity with the calculated gradients is the same as ALF. Since 1-D Laplacian values, V, H, D1, and D2 are calculated at the subsampled positions within 8x8 window, it can be claimed that the computational complexity in calculating gradients is reduced by using the CE2.6.1 to CE2.6.4 tests.

Experiment results

The CE2.6.1 to CE2.6.4 tests were performed using CE2.6.1 software [6], based on VTM 2.0.1 software under JVET common test condition [7]. The experiments are conducted on the platform of Intel Xeon E5-1620 v2 (3.7GHz) and 32GB RAM with GCC 4.7.2 compiler under CentOS Linux. The default setting for the current VTM software is SIMD enabled. But, the experiments with SIMD disabled were also performed by using a command line parameter “--SIMD=SCALAR” to show the decoding time reduction of the CE2.6.1 to CE2.6.4 tests. In this SIMD disabled setting, all of the SIMD instructions including instructions related to the ALF are disabled.

The experimental results of CE2.6.1 test with SIMD disabled and enabled compared to VTM 2.0.1 anchor with SIMD disabled and enabled are summarized in Table I, respectively. The average BD-rates of 0.06%, 0.07%, and 0.04% are observed for AI, RA, and LDB cases, respectively. In the case of SIMD disabled, decoding times of 97%, 97%, and 98% for AI, RA, and LDB cases are observed, respectively.

Table I. Experimental results of CE2.6.1 over VTM 2.0.1 anchor

| All Intra Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.03%; 0.00%; 0.00%; 100%; 97%. Class A2; 0.03%; 0.00%; 0.00%; 100%; 97%. Class B; 0.04%; 0.00%; 0.00%; 100%; 97%. Class C; 0.12%; -0.01%; -0.01%; 100%; 98%. Class E; 0.07%; -0.01%; -0.01%; 100%; 97%. Overall; 0.06%; 0.00%; 0.00%; 100%; 97%. Class D; 0.09%; -0.02%; -0.02%; 100%; 98%. Random Access Main 10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.03%; 0.06%; 0.00%; 99%; 97%. Class A2; 0.04%; -0.02%; 0.02%; 99%; 97%. Class B; 0.09%; 0.02%; -0.07%; 99%; 97%. Class C; 0.09%; 0.09%; 0.01%; 100%; 98%. Class E. Overall; 0.07%; 0.04%; -0.02%; 99%; 97%. Class D; 0.09%; -0.12%; -0.02%; 100%; 98%. Low delay B Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.06%; -0.04%; -0.15%; 99%; 97%. Class C; 0.12%; 0.00%; -0.11%; 100%; 98%. Class E; -0.09%; -0.49%; -0.22%; 99%; 98%. Overall; 0.04%; -0.14%; -0.16%; 99%; 98%. Class D; 0.02%; 0.54%; 0.00%; 100%; 98%. | All Intra Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.03%; 0.00%; 0.00%; 100%; 100%. Class A2; 0.03%; 0.00%; 0.00%; 100%; 100%. Class B; 0.04%; 0.00%; 0.00%; 100%; 100%. Class C; 0.12%; -0.01%; -0.01%; 100%; 100%. Class E; 0.07%; -0.01%; -0.01%; 100%; 100%. Overall; 0.06%; 0.00%; 0.00%; 100%; 100%. Class D; 0.09%; -0.02%; -0.02%; 100%; 100%. Random Access Main 10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.03%; 0.06%; 0.00%; 99%; 101%. Class A2; 0.04%; -0.02%; 0.02%; 100%; 100%. Class B; 0.09%; 0.02%; -0.07%; 100%; 100%. Class C; 0.09%; 0.09%; 0.01%; 100%; 100%. Class E. Overall; 0.07%; 0.04%; -0.02%; 100%; 100%. Class D; 0.09%; -0.12%; -0.02%; 100%; 100%. Low delay B Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.06%; -0.04%; -0.15%; 100%; 100%. Class C; 0.12%; 0.00%; -0.11%; 100%; 100%. Class E; -0.09%; -0.49%; -0.22%; 100%; 100%. Overall; 0.04%; -0.14%; -0.16%; 100%; 100%. Class D; 0.02%; 0.54%; 0.00%; 100%; 100%. |
| --- | --- |

The experimental results of CE2.6.2 test with SIMD disabled and enabled compared to VTM 2.0.1 anchor with SIMD disabled and enabled are summarized in Table II, respectively. The average BD-rates of 0.03%, 0.04%, and 0.00% are observed for AI, RA, and LDB cases, respectively. In the case of SIMD disabled, decoding times of 98%, 97%, and 97% for AI, RA, and LDB cases are observed, respectively.

Table II. Experimental results of CE2.6.2 over VTM 2.0.1 anchor

| All Intra Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.00%; 0.00%; 100%; 97%. Class A2; 0.01%; 0.00%; 0.00%; 100%; 97%. Class B; 0.02%; 0.00%; 0.00%; 100%; 98%. Class C; 0.08%; -0.01%; -0.01%; 100%; 98%. Class E; 0.03%; -0.01%; -0.01%; 100%; 97%. Overall; 0.03%; 0.00%; 0.00%; 100%; 98%. Class D; 0.06%; -0.01%; -0.01%; 100%; 98%. Random Access Main 10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.15%; -0.04%; 100%; 97%. Class A2; 0.04%; 0.08%; -0.01%; 100%; 97%. Class B; 0.06%; -0.03%; -0.14%; 100%; 97%. Class C; 0.05%; -0.02%; -0.12%; 100%; 98%. Class E. Overall; 0.04%; 0.03%; -0.09%; 100%; 97%. Class D; 0.08%; 0.00%; -0.17%; 100%; 98%. Low delay B Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.04%; -0.07%; -0.14%; 100%; 97%. Class C; 0.09%; -0.10%; -0.02%; 100%; 98%. Class E; -0.19%; -0.21%; -0.22%; 100%; 98%. Overall; 0.00%; -0.11%; -0.12%; 100%; 97%. Class D; 0.05%; 0.30%; -0.68%; 100%; 98%. | All Intra Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.00%; 0.00%; 100%; 100%. Class A2; 0.01%; 0.00%; 0.00%; 100%; 100%. Class B; 0.02%; 0.00%; 0.00%; 100%; 100%. Class C; 0.08%; -0.01%; -0.01%; 100%; 100%. Class E; 0.03%; -0.01%; -0.01%; 100%; 100%. Overall; 0.03%; 0.00%; 0.00%; 100%; 100%. Class D; 0.06%; -0.01%; -0.01%; 100%; 100%. Random Access Main 10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.15%; -0.04%; 99%; 101%. Class A2; 0.04%; 0.08%; -0.01%; 100%; 100%. Class B; 0.06%; -0.03%; -0.14%; 100%; 100%. Class C; 0.05%; -0.02%; -0.12%; 100%; 100%. Class E. Overall; 0.04%; 0.03%; -0.09%; 100%; 100%. Class D; 0.08%; 0.00%; -0.17%; 100%; 100%. Low delay B Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.04%; -0.07%; -0.14%; 100%; 100%. Class C; 0.09%; -0.10%; -0.02%; 100%; 100%. Class E; -0.19%; -0.21%; -0.22%; 99%; 100%. Overall; 0.00%; -0.11%; -0.12%; 100%; 100%. Class D; 0.05%; 0.30%; -0.68%; 100%; 100%. |
| --- | --- |

The experimental results of CE2.6.3 test with SIMD disabled and enabled compared to VTM 2.0.1 anchor with SIMD disabled and enabled are summarized in Table III, respectively. The average BD-rates of 0.03%, 0.04%, and -0.01% are observed for AI, RA, and LDB cases, respectively. In the case of SIMD disabled, decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases are observed, respectively.

Table III. Experimental results of CE2.6.3 over VTM 2.0.1 anchor

| All Intra Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.00%; 0.00%; 100%; 97%. Class A2; 0.01%; 0.00%; 0.00%; 100%; 97%. Class B; 0.02%; 0.00%; 0.00%; 100%; 98%. Class C; 0.07%; 0.00%; 0.00%; 100%; 98%. Class E; 0.02%; 0.00%; 0.00%; 100%; 97%. Overall; 0.03%; 0.00%; 0.00%; 100%; 98%. Class D; 0.06%; -0.01%; -0.01%; 100%; 98%. Random Access Main 10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.00%; -0.02%; -0.04%; 100%; 96%. Class A2; 0.04%; 0.01%; -0.06%; 100%; 97%. Class B; 0.05%; 0.05%; 0.01%; 100%; 97%. Class C; 0.06%; 0.13%; 0.05%; 100%; 98%. Class E. Overall; 0.04%; 0.05%; 0.00%; 100%; 97%. Class D; 0.05%; -0.05%; -0.04%; 100%; 98%. Low delay B Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.05%; 0.03%; 0.14%; 100%; 97%. Class C; 0.04%; 0.10%; -0.15%; 100%; 98%. Class E; -0.20%; 0.28%; -0.17%; 100%; 98%. Overall; -0.01%; 0.11%; -0.03%; 100%; 98%. Class D; 0.04%; 0.58%; -0.10%; 100%; 98%. | All Intra Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.01%; 0.00%; 0.00%; 100%; 100%. Class A2; 0.01%; 0.00%; 0.00%; 100%; 100%. Class B; 0.02%; 0.00%; 0.00%; 100%; 100%. Class C; 0.07%; 0.00%; 0.00%; 100%; 100%. Class E; 0.02%; 0.00%; 0.00%; 100%; 100%. Overall; 0.03%; 0.00%; 0.00%; 100%; 100%. Class D; 0.06%; -0.01%; -0.01%; 100%; 100%. Random Access Main 10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.00%; -0.02%; -0.04%; 100%; 100%. Class A2; 0.04%; 0.01%; -0.06%; 100%; 100%. Class B; 0.05%; 0.05%; 0.01%; 100%; 100%. Class C; 0.06%; 0.13%; 0.05%; 100%; 100%. Class E. Overall; 0.04%; 0.05%; 0.00%; 100%; 100%. Class D; 0.05%; -0.05%; -0.04%; 99%; 101%. Low delay B Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.05%; 0.03%; 0.14%; 100%; 100%. Class C; 0.04%; 0.10%; -0.15%; 100%; 100%. Class E; -0.20%; 0.28%; -0.17%; 99%; 100%. Overall; -0.01%; 0.11%; -0.03%; 100%; 100%. Class D; 0.04%; 0.58%; -0.10%; 100%; 100%. |
| --- | --- |

The experimental results of CE2.6.4 test with SIMD disabled and enabled compared to VTM 2.0.1 anchor with SIMD disabled and enabled are summarized in Table IV, respectively. The average BD-rates of 0.09%, 0.11%, and 0.06% are observed for AI, RA, and LDB cases, respectively. In the case of SIMD disabled, decoding times of 97%, 97%, and 97% for AI, RA, and LDB cases are observed, respectively.

Table IV. Experimental results of CE2.6.4 over VTM 2.0.1 anchor

| All Intra Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.07%; 0.00%; 0.00%; 100%; 97%. Class A2; 0.05%; 0.00%; 0.00%; 100%; 97%. Class B; 0.08%; -0.01%; -0.01%; 100%; 97%. Class C; 0.10%; -0.02%; -0.02%; 100%; 98%. Class E; 0.17%; -0.02%; -0.02%; 100%; 97%. Overall; 0.09%; -0.01%; -0.01%; 100%; 97%. Class D; 0.07%; -0.02%; -0.02%; 100%; 98%. Random Access Main 10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1; 0.04%; 0.00%; -0.04%; 100%; 96%. Class A2; 0.13%; 0.05%; 0.01%; 100%; 97%. Class B; 0.14%; 0.05%; -0.12%; 100%; 97%. Class C; 0.11%; 0.05%; -0.11%; 100%; 97%. Class E. Overall; 0.11%; 0.04%; -0.07%; 100%; 97%. Class D; 0.04%; -0.02%; -0.14%; 100%; 98%. Low delay B Main10. Over VTM-2.0.1 (SIMD disabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.11%; -0.04%; -0.15%; 100%; 97%. Class C; 0.08%; 0.00%; -0.07%; 100%; 98%. Class E; -0.05%; -0.07%; -0.31%; 100%; 97%. Overall; 0.06%; -0.03%; -0.17%; 100%; 97%. Class D; 0.08%; 0.66%; -0.21%; 100%; 98%. | All Intra Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.07%; 0.00%; 0.00%; 100%; 100%. Class A2; 0.05%; 0.00%; 0.00%; 100%; 100%. Class B; 0.08%; -0.01%; -0.01%; 100%; 100%. Class C; 0.10%; -0.02%; -0.02%; 100%; 100%. Class E; 0.17%; -0.02%; -0.02%; 100%; 100%. Overall; 0.09%; -0.01%; -0.01%; 100%; 100%. Class D; 0.07%; -0.02%; -0.02%; 100%; 100%. Random Access Main 10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1; 0.04%; 0.00%; -0.04%; 99%; 100%. Class A2; 0.13%; 0.05%; 0.01%; 100%; 100%. Class B; 0.14%; 0.05%; -0.12%; 100%; 100%. Class C; 0.11%; 0.05%; -0.11%; 100%; 99%. Class E. Overall; 0.11%; 0.04%; -0.07%; 100%; 100%. Class D; 0.04%; -0.02%; -0.14%; 100%; 100%. Low delay B Main10. Over VTM-2.0.1 (SIMD enabled). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.11%; -0.04%; -0.15%; 100%; 100%. Class C; 0.08%; 0.00%; -0.07%; 100%; 100%. Class E; -0.05%; -0.07%; -0.31%; 99%; 100%. Overall; 0.06%; -0.03%; -0.17%; 100%; 100%. Class D; 0.08%; 0.66%; -0.21%; 100%; 100%. |
| --- | --- |

Computational complexity analysis

It is claimed that the CE2.6.1 to CE2.6.4 tests reduce the computational complexity compared to the ALF in the VTM as described in the following:

In the 4x4 block classification based on subsampled Laplacian calculation, 1-D Laplacian values, V, H, D1, and D2, calculated at the subsampled positions within 8x8 window are summed up for each 4x4 luma block to derive the gradients of vertical, horizontal, diagonal 1, and diagonal 2 directions. Thus, for example of 8x8 block, 720+240 additions, 288 comparisons, and 144 shifts are required to calculate all of the gradients. On the other hand, in the 4x4 block classification of the ALF in the VTM, 1-D Laplacian values calculated at every positions within 8x8 window are summed up for each 4x4 luma block to derive the all of the gradients. For example of 8x8 block, 1586+240 additions, 576 comparisons, and 144 shifts are required to calculate all of the gradients. And, the rest of derivation process for directionality and the quantized value of activity with the calculated gradients requires 8 additions, 28 comparisons, 8 multiplications, and 20 shifts. Thus, in the block classification process, the CE2.6.1 to CE2.6.4 methods requires 968 additions, 316 comparisons, 8 multiplications, and 164 shifts within 8x8 block. And, the operations per sample are 15.125 additions, 4.9375 comparisons, 0.125 multiplications, and 2.5625 shifts. On the other hand, in the block classification process, the ALF in the VTM requires 1832 additions, 604 comparisons, 8 multiplications, and 164 shifts within 8x8 block. And, the operations per sample are 28.625 additions, 9.4375 comparisons, 0.125 multiplications, and 2.5625 shifts.

If the computational complexity for a given block size (e.g., 8x8 block) is considered, the CE2.6.1 to CE2.6.4 tests reduce 44.17% of worst case operation count compared to the ALF for the total operation in the block classification, and the CE2.6.1 to CE2.6.4 tests reduce 17.02% of worst case operation count compared to the ALF for the total operation of the ALF.

Table V. Computational complexity comparison

| Test | Operations of classification process per luma sample | Operations of filtering process per luma sample | Operations of filtering process per chroma sample | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) | Memory for storing filters | Summary (total operations per sample) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor (VTM 2.0.1) | Add: 28.625 Comp: 9.4375 Mul: 0.125 Shift: 2.5625 | Add: 25 Comp: 4 Mult: 13 Shift: 1 | Add: 13 Comp: 1 Mult: 7 Shift: 1 | 4x4, slice, slice | 6, 4 | 0 | Add: 66.63 Comp: 14.44 Mult: 20.13 Shift: 4.56 |
| CE2.6.1, CE2.6.2, CE2.6.3, CE2.6.4 | Add: 15.125 Comp: 4.9375 Mult: 0.125 Shift: 2.5625 | Add: 25 Comp: 4 Mult: 13 Shift: 1 | Add: 13 Comp: 1 Mult: 7 Shift: 1 | 4x4, slice, slice | 6, 4 | 0 | Add: 53.13 Comp: 9.94 Mult: 20.13 Shift: 4.56 |
| Complexity Ratio (Test / Anchor) | Add: 52.84% Comp: 52.32% Mult: 100.00% Shift: 100.00% Total: 55.83% | Same as anchor | Same as anchor | Same as anchor | Same as anchor | Same as anchor | Add: 79.74% Comp: 68.83% Mult: 100.00% Shift: 100.00% Total: 82.98% |

Conclusions

This contribution reports the experimental results of CE2.6.1, CE2.6.2, CE2.6.3, and CE2.6.4 tests. 4x4 block classification methods based on various subsampled Laplacian calculations were tested in order to reduce the worst case computational complexity for the block classification of the ALF in the VTM. Compared to the VTM, the CE2.6.1 to CE2.6.4 tests reduce the operation count of 44.17% in the block classification and the operation count of 17.02% in the total operation of the ALF. Compared to VTM with SIMD disabled, the CE2.6.1 test shows average BD-rates of 0.06%, 0.07%, and 0.04% with decoding times of 97%, 97%, and 98% for AI, RA, and LDB cases, respectively. And, the CE2.6.2 test shows average BD-rates of 0.03%, 0.04%, and 0.00% with decoding times of 98%, 97%, and 97% for AI, RA, and LDB cases, respectively. And, the CE2.6.3 test shows average BD-rates of 0.03%, 0.04%, and -0.01% with decoding times of 98%, 97%, and 98% for AI, RA, and LDB cases, respectively. Finally, the CE2.6.4 test shows average BD-rates of 0.09%, 0.11%, and 0.06% with decoding times of 97%, 97%, and 97% for AI, RA, and LDB cases, respectively.

References

- J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim, N.-U. Kim, and Y.-L. Lee, “Description of SDR video coding technology proposal by ETRI and Sejong University,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0013, Apr. 2018

- S.-C. Lim, J. Kang, H. Lee, J. Lee, S. Cho, and H. Y. Kim, “CE2: Subsampled sum-modified-Laplacian (Test 4.1.1), ” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0164, Jul. 2018

- L. Zhang, K. Andersson, and C.-Y. Chen, “CE2: Summary Report on In-Loop Filters,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0022, Jul. 2018

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- V. Seregin and C.-Y. Chen, “Description of Core Experiment 2 (CE2): Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1022, Jul. 2018

- https://vcgit.hhi.fraunhofer.de/JVET-K-CE2/VVCSoftware_VTM.git

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1010, Jul. 2018

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).