CE2.6.1 based on Versatile Video Coding (Draft 2) v7

      - Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-435)

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-436)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ], varTempD0[ x ][ y ] and varTempD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

If ( Abs( y ) % 2 ) is equal to 0, then

varTempH[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-437)
	− recPicture[ hxC+x−1, vyC+y ] − recPicture[ hxC+x+1, vyC+y ]  )

If ( Abs( x ) % 2 ) is equal to 0, then

varTempV[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-438)
	− recPicture[ hxC+x, vyC+y−1 ] − recPicture[ hxC+x, vyC+y+1 ]  )

If ( ( Abs( x ) + Abs( y ) ) % 2 ) is equal to 0, then

varTempD0[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-439)
	− recPicture[ hxC+x−1, vyC+y−1 ] − recPicture[ hxC+x+1, vyC+y+1 ]  )

If ( ( Abs( x ) + Abs( y ) ) % 2 ) is equal to 1, then

varTempD1[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-440)
	− recPicture[ hxC+x+1, vyC+y−1 ] − recPicture[ hxC+x−1, vyC+y+1 ]  )

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-441)

varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-442)

varTempD01[ x ][ y ] = ij varTempD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-443)

varTempD11[ x ][ y ] = ij varTempD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-444)

varTemp[ x ][ y ] = varTempH1[ x ][ y ] + varTempV1[ x ][ y ]	(8-445)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and directionHV are derived as follows:

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = varTempV1[ x >> 2 ][ y >> 2 ]	(8-446)

hv0 = varTempH1[ x >> 2 ][ y >> 2 ] 	(8-447)

directionHV = 1		(8-448)

- Otherwise, the following applies:

hv1 = varTempH1[ x >> 2 ][ y >> 2 ]	(8-449)

hv0 = varTempV1[ x >> 2 ][ y >> 2 ] 	(8-450)

directionHV = 3		(8-451)

- The variables d1, d0 and directionD are derived as follows:

- If varTempD0[ x >> 2 ][ y >> 2 ] is greater than varTempD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = varTempD0[ x >> 2 ][ y >> 2 ]	(8-452)

d0 = varTempD1[ x >> 2 ][ y >> 2 ] 	(8-453)

directionD = 0		(8-454)

- Otherwise, the following applies:

d1 = varTempD1[ x >> 2 ][ y >> 2 ]	(8-455)

d0 = varTempD0[ x >> 2 ][ y >> 2 ] 	(8-456)

directionD = 2		(8-457)

- The variables hvd1, hvd0,   are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-458)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-459)

- The variables directionStrength, mainDirection and secondaryDirectionare derived as follows:

mainDirection = ( d1 * hv0 > hv1 * d0 )  ?  directionD  :  directionHV	(8-460)

secondaryDirection = ( d1 * hv0 > hv1 * d0 )  ?  direction  :  HVdirectionD	(8-461)

directionStrength = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-462)

- The variable avgVar is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-463)

avgVar = varTab[ Clip3( 0, 15, ( varTemp[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-464)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ mainDirection * 2 + ( secondaryDirection >> 1 ) ]

filtIdx[ x ][ y ] = avgVar

When directionStrength is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( mainDirection & 0x1 ) << 1 ) + directionStrength ) * 5	(8-465)

CE2.6.2 based on Versatile Video Coding (Draft 2) v7

      - Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-435)

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-436)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ], varTempD0[ x ][ y ] and varTempD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

If ( ( Abs( x ) + Abs( y ) ) % 2 ) is equal to 0, then

varTempH[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-437)
	  − recPicture[ hxC+x−1, vyC+y ] − recPicture[ hxC+x+1, vyC+y ]  )

varTempV[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-438)
	  − recPicture[ hxC+x, vyC+y−1 ] − recPicture[ hxC+x, vyC+y+1 ]  )

varTempD0[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-439)
	− recPicture[ hxC+x−1, vyC+y−1 ] − recPicture[ hxC+x+1, vyC+y+1 ]  )

varTempD1[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-440)
	− recPicture[ hxC+x+1, vyC+y−1 ] − recPicture[ hxC+x−1, vyC+y+1 ]  )

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-441)

varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-442)

varTempD01[ x ][ y ] = ij varTempD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-443)

varTempD11[ x ][ y ] = ij varTempD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-444)

varTemp[ x ][ y ] = varTempH1[ x ][ y ] + varTempV1[ x ][ y ]	(8-445)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and directionHV are derived as follows:

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = varTempV1[ x >> 2 ][ y >> 2 ]	(8-446)

hv0 = varTempH1[ x >> 2 ][ y >> 2 ] 	(8-447)

directionHV = 1		(8-448)

- Otherwise, the following applies:

hv1 = varTempH1[ x >> 2 ][ y >> 2 ]	(8-449)

hv0 = varTempV1[ x >> 2 ][ y >> 2 ] 	(8-450)

directionHV = 3		(8-451)

- The variables d1, d0 and directionD are derived as follows:

- If varTempD0[ x >> 2 ][ y >> 2 ] is greater than varTempD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = varTempD0[ x >> 2 ][ y >> 2 ]	(8-452)

d0 = varTempD1[ x >> 2 ][ y >> 2 ] 	(8-453)

directionD = 0		(8-454)

- Otherwise, the following applies:

d1 = varTempD1[ x >> 2 ][ y >> 2 ]	(8-455)

d0 = varTempD0[ x >> 2 ][ y >> 2 ] 	(8-456)

directionD = 2		(8-457)

- The variables hvd1, hvd0,   are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-458)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-459)

- The variables directionStrength, mainDirection and secondaryDirectionare derived as follows:

mainDirection = ( d1 * hv0 > hv1 * d0 )  ?  directionD  :  directionHV	(8-460)

secondaryDirection = ( d1 * hv0 > hv1 * d0 )  ?  direction  :  HVdirectionD	(8-461)

directionStrength = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-462)

- The variable avgVar is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-463)

avgVar = varTab[ Clip3( 0, 15, ( varTemp[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-464)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ mainDirection * 2 + ( secondaryDirection >> 1 ) ]

filtIdx[ x ][ y ] = avgVar

When directionStrength is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( mainDirection & 0x1 ) << 1 ) + directionStrength ) * 5	(8-465)

CE2.6.3 based on Versatile Video Coding (Draft 2) v7

      - Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-435)

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-436)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ], varTempD0[ x ][ y ] and varTempD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

If ( ( Abs( x ) + Abs( y ) ) % 2 ) is equal to 1, then

varTempH[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-437)
	− recPicture[ hxC+x−1, vyC+y ] − recPicture[ hxC+x+1, vyC+y ]  )

varTempV[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-438)
	− recPicture[ hxC+x, vyC+y−1 ] − recPicture[ hxC+x, vyC+y+1 ]  )

varTempD0[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-439)
	− recPicture[ hxC+x−1, vyC+y−1 ] − recPicture[ hxC+x+1, vyC+y+1 ]  )

varTempD1[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-440)
	− recPicture[ hxC+x+1, vyC+y−1 ] − recPicture[ hxC+x−1, vyC+y+1 ]  )

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-441)

varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-442)

varTempD01[ x ][ y ] = ij varTempD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-443)

varTempD11[ x ][ y ] = ij varTempD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-444)

varTemp[ x ][ y ] = varTempH1[ x ][ y ] + varTempV1[ x ][ y ]	(8-445)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and directionHV are derived as follows:

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = varTempV1[ x >> 2 ][ y >> 2 ]	(8-446)

hv0 = varTempH1[ x >> 2 ][ y >> 2 ] 	(8-447)

directionHV = 1		(8-448)

- Otherwise, the following applies:

hv1 = varTempH1[ x >> 2 ][ y >> 2 ]	(8-449)

hv0 = varTempV1[ x >> 2 ][ y >> 2 ] 	(8-450)

directionHV = 3		(8-451)

- The variables d1, d0 and directionD are derived as follows:

- If varTempD0[ x >> 2 ][ y >> 2 ] is greater than varTempD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = varTempD0[ x >> 2 ][ y >> 2 ]	(8-452)

d0 = varTempD1[ x >> 2 ][ y >> 2 ] 	(8-453)

directionD = 0		(8-454)

- Otherwise, the following applies:

d1 = varTempD1[ x >> 2 ][ y >> 2 ]	(8-455)

d0 = varTempD0[ x >> 2 ][ y >> 2 ] 	(8-456)

directionD = 2		(8-457)

- The variables hvd1, hvd0,   are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-458)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-459)

- The variables directionStrength, mainDirection and secondaryDirectionare derived as follows:

mainDirection = ( d1 * hv0 > hv1 * d0 )  ?  directionD  :  directionHV	(8-460)

secondaryDirection = ( d1 * hv0 > hv1 * d0 )  ?  direction  :  HVdirectionD	(8-461)

directionStrength = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-462)

- The variable avgVar is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-463)

avgVar = varTab[ Clip3( 0, 15, ( varTemp[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-464)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ mainDirection * 2 + ( secondaryDirection >> 1 ) ]

filtIdx[ x ][ y ] = avgVar

When directionStrength is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( mainDirection & 0x1 ) << 1 ) + directionStrength ) * 5	(8-465)

CE2.6.4 based on Versatile Video Coding (Draft 2) v7

      - Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-435)

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-436)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ], varTempD0[ x ][ y ] and varTempD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

If ( Abs( y ) % 2 ) is equal to 0, then

varTempH[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-437)
	  − recPicture[ hxC+x−1, vyC+y ] − recPicture[ hxC+x+1, vyC+y ]  )

varTempV[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-438)
	  − recPicture[ hxC+x, vyC+y−1 ] − recPicture[ hxC+x, vyC+y+1 ]  )

varTempD0[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-439)
	− recPicture[ hxC+x−1, vyC+y−1 ] − recPicture[ hxC+x+1, vyC+y+1 ]  )

varTempD1[ x ][ y ] = Abs(  ( recPicture[ hxC+x, vyC+y ] << 1 )  	(8-440)
	− recPicture[ hxC+x+1, vyC+y−1 ] − recPicture[ hxC+x−1, vyC+y+1 ]  )

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-441)

varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-442)

varTempD01[ x ][ y ] = ij varTempD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-443)

varTempD11[ x ][ y ] = ij varTempD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-444)

varTemp[ x ][ y ] = varTempH1[ x ][ y ] + varTempV1[ x ][ y ]	(8-445)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY)

- The variables mainDirection, secondaryDirection and directionStrength with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and directionHV are derived as follows:

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = varTempV1[ x >> 2 ][ y >> 2 ]	(8-446)

hv0 = varTempH1[ x >> 2 ][ y >> 2 ] 	(8-447)

directionHV = 1		(8-448)

- Otherwise, the following applies:

hv1 = varTempH1[ x >> 2 ][ y >> 2 ]	(8-449)

hv0 = varTempV1[ x >> 2 ][ y >> 2 ] 	(8-450)

directionHV = 3		(8-451)

- The variables d1, d0 and directionD are derived as follows:

- If varTempD0[ x >> 2 ][ y >> 2 ] is greater than varTempD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = varTempD0[ x >> 2 ][ y >> 2 ]	(8-452)

d0 = varTempD1[ x >> 2 ][ y >> 2 ] 	(8-453)

directionD = 0		(8-454)

- Otherwise, the following applies:

d1 = varTempD1[ x >> 2 ][ y >> 2 ]	(8-455)

d0 = varTempD0[ x >> 2 ][ y >> 2 ] 	(8-456)

directionD = 2		(8-457)

- The variables hvd1, hvd0,   are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-458)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-459)

- The variables directionStrength, mainDirection and secondaryDirectionare derived as follows:

mainDirection = ( d1 * hv0 > hv1 * d0 )  ?  directionD  :  directionHV	(8-460)

secondaryDirection = ( d1 * hv0 > hv1 * d0 )  ?  direction  :  HVdirectionD	(8-461)

directionStrength = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-462)

- The variable avgVar is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-463)

avgVar = varTab[ Clip3( 0, 15, ( varTemp[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-464)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ mainDirection * 2 + ( secondaryDirection >> 1 ) ]

filtIdx[ x ][ y ] = avgVar

When directionStrength is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( mainDirection & 0x1 ) << 1 ) + directionStrength ) * 5	(8-465)