| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0155 |
| --- | --- |

| Title: | CE3-related: Most Frequent Mode (MFM) for Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong-Uk Yoon Do-Hyeon Park Jae-Gon Kim Jinho Lee Jungwon Kang | Tel: Email: | +82-2-300-0414 yuyoon@kau.kr dhpark@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

At the last meeting, 3 MPMs and 67 modes are adopted in VTM 2.0.1 software. As the number of intra modes are increased, more efficient coding of intra mode is require. This contribution proposes the most frequent mode (MFM) for intra mode coding. The MFM means the most frequent mode to be derived from neighbors and is indicated by MFM_FLAG. The decoder parses MFM_FLAG and derives one mode from neighbors if MFM_FLAG is true, otherwise, parses MPM_FLAG. This contribution reports the performances on three test including the MFM in the RD-check list and adjusting the number of RD-checks for the MPM. Based on the same number of RD-checks as the current VTM-2.0.1 software, the proposed method reportedly shows 0.28% bitrate saving without increase of encoding/decoding running time.

Introduction

At the last meeting, 3 MPMs and 67 modes are adopted in VTM 2.0.1 software[1],[2]. The MPM is a method of effectively representing the intra mode of the current block with the neighbor modes as candidates, assuming that the current block is highly correlated with the neighboring blocks. The MPM is signalled by MPM_FLAG and MPM_IDX and takes at least 2 bins. Since the number of intra mode is increased to 67, the MPM candidates to be derived from two locations may be restricted to accommodate diverse modes. In addition, it is observed that certain modes are likely to be dominant, then these dominant modes can be represented with fewer bins. In this contribution, the most frequent mode (MFM) is proposed to efficiently encode dominant modes with less bins, and results in more diverse modes signalled by MFM and MPM.

Proposed method

The proposed MFM is derived as one intra mode from the those of five neighboring blocks which are used for the derivation of the MERGE candidates as shown in Figure 1. The MFM is determined to be the most frequent mode at the derived positions. The searching order is as follows: L, A, LB, AR, AL, which gives priority to select MFM when there are multiple modes having the same frequency. In addition, 3 MPMs can be derived together in the MFM derivation with the same searching order. Figure 2 shows examples of deriving MFM. Additionally, the MPM list is constructed with the order of L, A, Planar, DC, LB, AR, and AL which is the same order with 6 MPM derivation. The selected MFM is signalled to MFM_FLAG, and the decoder parses MFM_FLAG and derives MFM from the neighbors. If it is not MFM, MPM_FLAG is parsed to check if the current intra mode is MPM. Beside MFM, the number of remaining modes is 63, which is encoded with truncated binary coding.

Figure 1. Positions deriving the MFM and MPM

Figure 2. Examples of MFM derivation

Experiment results

The proposed method is implemented on top of BMS-2.0.1 [3] and evaluated with VTM configuration according to the common test conditions [4]. In current VTM software, if the left and above intra modes are the same, one MPM is added to the RD-check list, otherwise two MPMs are included. This contribution report experimental results on three tests which include the MFM in the RD-check list and adjust the number of RD-checks for the MPM according to condition below.

- Test 1: (above == left) ? 0 : 1 – the same to the number of current RD-check with the VTM 2.0.1 (1 or 2 RD check including MFM)

- Test 2: MPM list 0 is included in default in RD-check list (2 RD check including MFM)

- Test 3: (above == left) ? 1 : 2 – add 1 or 2 MPMs in RD-check list (2 or 3 RD check including MFM)

Table 1. Experiment results of Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (VTM cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.30% | -0.24% | 100% | 104% |
| Class A2 | -0.24% | -0.25% | -0.32% | 100% | 100% |
| Class B | -0.26% | -0.31% | -0.32% | 100% | 100% |
| Class C | -0.32% | -0.39% | -0.19% | 100% | 99% |
| Class E | -0.35% | -0.34% | -0.39% | 101% | 97% |
| Overall | -0.28% | -0.32% | -0.29% | 100% | 100% |
| Class D | -0.15% | -0.02% | -0.26% | 101% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2. Experiment results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (VTM cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.36% | -0.32% | 104% | 104% |
| Class A2 | -0.31% | -0.33% | -0.33% | 103% | 100% |
| Class B | -0.34% | -0.32% | -0.32% | 104% | 100% |
| Class C | -0.36% | -0.40% | -0.25% | 103% | 99% |
| Class E | -0.41% | -0.42% | -0.37% | 103% | 98% |
| Overall | -0.35% | -0.36% | -0.32% | 103% | 100% |
| Class D | -0.22% | -0.08% | -0.36% | 103% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3. Experiment results of Test 3

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (VTM cfg) |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class A1 | -0.52% |  | -0.42% |  | -0.39% | 110% | 105% |
| Class A2 | -0.40% |  | -0.34% |  | -0.41% | 110% | 100% |
| Class B | -0.45% |  | -0.39% |  | -0.38% | 111% | 101% |
| Class C | -0.47% |  | -0.45% |  | -0.29% | 111% | 101% |
| Class E | -0.58% |  | -0.45% |  | -0.52% | 111% | 99% |
| Overall | -0.48% |  | -0.41% |  | -0.39% | 111% | 101% |
| Class D | -0.33% |  | -0.09% |  | -0.37% | 113% | 101% |
| Class F (optional) | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This contribution presents the most frequent mode (MFM) for intra mode coding. The proposed method encodes dominant modes with less bins, and results in more diverse modes signalled by MFM and MPM. The experimental results report 0.28% bitrate savings without increase of encoding/decoding running time based on the same number of RD-checks as the current VTM-2.0.1 software.

References

- J. Chen, Y. Ye, and S. Kim, “Algorithm description for Versatile Video Coding and Test Model 2,” JVET document, JVET-K1002, July 2018.

- VVC Test Model (VTM), https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags

- Benchmark Set (BMS), https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suhring, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-K1010, July 2018.

Patent rights declaration(s)

KAU and ETRI Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).