| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0157 |
| --- | --- |

| Title: | CE10: Uniform Directional Diffusion Filters For Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Anastasia Henkel, Jonathan Pfaff, Michael Schäfer, Heiko Schwarz, Mischa Siekmann, Martin Winken, Philipp Helle, Detlev Marpe, Thomas Wiegand | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de, anastasia.henkel@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results on the tool under test in CE10 (JVET-Jxxxx) for studying the performance of uniform directional diffusion filters on top of VTM and BMS.

In this document, diffusion filters which were introduced in [2],[3] are modified: The filter coefficients are not signal adaptive. Thus, the diffusion filters are implemented by applying a fixed filter mask to the prediction signal.  A detailed description of the filter masks as well as an analysis of the maximal number of operations (additions/bit-shifts/multiplications) per pixel are provided.

# Proposed method: Decoder algorithm description

Uniform Directional Diffusion Filters have been integrated to the test model according to the description of sub-tests CE10.4.1 [1].

## Uniform Diffusion Filter

The Uniform Diffusion Filter is realized by convolving the prediction signal with a fixed mask that is either given as hI or as hIV, defined below. 	
Besides the prediction signal itself, one line of reconstructed samples left and above of the block are used as an input for the filtered signal, where the use of these reconstructed samples can be avoided on inter blocks (CE 10.4.1).

Let pred be the prediction signal on a given block obtained by intra or motion compensated prediction. In order to handle boundary points for the filters, the prediction signal needs to be extended to a prediction signal predext. This extended prediction can be formed in two ways: Either, as an intermediate step, one line of reconstructed samples left and above the block are added to the prediction signal and then the resulting signal is mirrored in all directions. Or only the prediction signal itself is mirrored in all directions. The latter extension is used in CE 10.4.1 for inter blocks. In this case, only the prediction signal itself comprises the input for the extended prediction signal predext.

If the filter hI is to be used, it is proposed to replace the prediction signal pred by

hI*pred,

using the aforementioned boundary extension. Here, the filter mask hI is given as

hI=0.254000010000000404000006016060004024024040101603601601040240240400060160600000404000000010000.

If the filter hIV is to be used, it is proposed to replace the prediction signal pred by

hIV*pred.

Here, the filter hIV is given as

hIV=hI*hI*hI*hI.

## Directional Diffusion Filter

Instead of using signal adaptive diffusion filters as has been done in [2],[3] previously, directional filters, a horizontal filter hhor and a vertical filter hver,  are used which still have a fixed mask. More precisely, the uniform diffusion filtering corresponding to the mask hI of the previous section is simply restricted to be either applied only along the vertical or along the horizontal direction. The vertical filter is realized by applying the fixed filter mask

hver=0.54 104060401

to the prediction signal and the horizontal filter is realized by using the transposed mask

hhor=hvert.

The extension of the prediction signal is performed in exactly the same way as in the previous section, including the restriction for inter blocks that is active in CE 10.4.1.

# Implementation and Complexity

Different filter configurations, depending on the block-shape, are used. These configurations are described via the syntax element diffFilterIdx as described in the table below.

| diffFilterIdx | filter type | Filters to be used |  |
| --- | --- | --- | --- |
|  |  | Intra | inter |
| 0 | no filter | - | - |
| 1 | Uniform | hI | hI |
| 2 | Uniform | hIV | hIV |
| 3 | directional horizontal | hhor | hhor |
| 4 | directional vertical | hver | hver |

Table  Interpretation of the syntax element diffFilterIdx

It should be noted here that the filter is restricted to larger blocks as depicted in the following table.

| BlkSizes | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 16 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 32 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 64 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 128 | 0 | 0 | 0 | 0 | 0 | 0,1,2,3,4 |

Table  Restricted number of tested Filters ordered by block sizes, width versus height

The filtering is implemented in a way such that symmetries within the filter-masks are exploited. The multiplications that are factors of two are implemented using bitshifts. The maximal complexity per pixel in terms of number of additions, number of bit-shifts and number of multiplications to produce the overall prediction signal when applying the diffusion filter is depicted for different block sizes below.

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 25 | 25 | 25 | 25 | - |
| 16 | 25 | 100 | 100 | 100 | - |
| 32 | 25 | 100 | 100 | 100 | - |
| 64 | 25 | 100 | 100 | 100 | - |
| 128 | - | - | - | - | 100 |

Table  Maximal number of additions for different block sizes (case of  filter hIV if allowed)

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 3 | 3 | 3 | 3 | - |
| 16 | 3 | 12 | 12 | 12 | - |
| 32 | 3 | 12 | 12 | 12 | - |
| 64 | 3 | 12 | 12 | 12 | - |
| 128 | - | - | - | - | 12 |

Table  Maximal number of bit-shifts for different block sizes (case of filter hIV if allowed)

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 2 | 2 | 2 | 2 | - |
| 16 | 2 | 8 | 8 | 8 | - |
| 32 | 2 | 8 | 8 | 8 | - |
| 64 | 2 | 8 | 8 | 8 | - |
| 128 | - | - | - | - | 8 |

Table  Maximal number of multiplications for different block sizes (case of filter hIV if allowed)

# Results

According to the CE10 description in JVET-K1030 [1] and the common test conditions in JVET-K1010 [4], the proposed method is evaluated for intra-only, random-access and low delay configuration with BMS-1.0 software in VTM and BMS configuration.

In the following table an overview over the scheduled CE tests with their corresponding config parameters is shown.

|  | Test |  |
| --- | --- | --- |
| CE10.4.1 | All Intra, Random Access and Low Delay, VTM +BMS - No neighboring block samples used for inter blocks | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 3 NMBDRYDiffusion : 1 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |
| CE10.4.2 | Random Access and Low Delay, VTM +BMS - Using reconstructed samples for inter | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 3 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |

## CE10.4.1

In case there are no neighboring samples used, the diffusion filtering obtains the following results.

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.80% | -0.54% |  | -0.49% |  |
| Class A2 | -0.44% | -0.25% |  | -0.24% |  |
| Class B | -0.28% | -0.21% |  | -0.12% |  |
| Class C | -0.12% | -0.16% |  | 0.03% |  |
| Class E | -0.23% | -0.04% |  | -0.10% |  |
| Overall | -0.35% | -0.23% |  | -0.17% |  |
| Enc Time [%] | 152% |  |  |  |  |
| Dec Time [%] | 102% |  |  |  |  |
| Class D | -0.09% |  | 0.08% |  | -0.34% |
| Class F | -0.05% |  | -0.12% |  | 0.06% |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.70% | -0.99% |  | -0.82% |  |
| Class A2 | -0.46% | -0.89% |  | -0.69% |  |
| Class B | -0.66% | -1.00% |  | -1.05% |  |
| Class C | -0.17% | -0.32% |  | -0.31% |  |
| Overall | -0.50% | -0.79% |  | -0.73% |  |
| Enc Time [%] | 125% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.08% |  | -0.46% |  | 0.12% |
| Class F | -0.11% |  | -0.20% |  | 0.06% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.25% | -0.14% |  | -0.28% |  |
| Class C | -0.13% | 0.15% |  | 0.14% |  |
| Class E | -0.13% | 0.51% |  | 0.36% |  |
| Overall | -0.18% | 0.12% |  | 0.02% |  |
| Enc Time [%] | 123% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.02% |  | 1.00% |  | 0.08% |
| Class F | -0.09% |  | 0.59% |  | 0.07% |

|  | BMS, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.78% | -0.57% |  | -0.36% |  |
| Class A2 | -0.42% | -0.30% |  | -0.23% |  |
| Class B | -0.28% | -0.23% |  | -0.22% |  |
| Class C | -0.09% | -0.01% |  | -0.05% |  |
| Class E | -0.20% | -0.20% |  | -0.08% |  |
| Overall | -0.33% | -0.24% |  | -0.18% |  |
| Enc Time [%] | 160% |  |  |  |  |
| Dec Time [%] | 102% |  |  |  |  |
| Class D | -0.06% |  | -0.05% |  | -0.10% |
| Class F | -0.03% |  | -0.01% |  | -0.08% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.64% | -0.97% |  | -0.73% |  |
| Class A2 | -0.51% | -0.89% |  | -0.63% |  |
| Class B | -0.66% | -0.85% |  | -1.12% |  |
| Class C | -0.18% | -0.35% |  | -0.24% |  |
| Overall | -0.50% | -0.75% |  | -0.71% |  |
| Enc Time [%] | 135% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.13% |  | 0.11% |  | -0.28% |
| Class F | -0.08% |  | -0.16% |  | -0.14% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.28% | -0.01% |  | -0.15% |  |
| Class C | -0.20% | 0.14% |  | 0.18% |  |
| Class E | -0.05% | -0.07% |  | 0.46% |  |
| Overall | -0.19% | 0.03% |  | 0.11% |  |
| Enc Time [%] | 134% |  |  |  |  |
| Dec Time [%] | 99% |  |  |  |  |
| Class D | -0.03% |  | 0.67% |  | 1.07% |
| Class F | -0.14% |  | 0.44% |  | -0.24% |

## CE10.4.2

If in inter case the reconstructed samples left and above are used, the diffusion filtering obtains the following results.

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.71% | -1.16% |  | -0.90% |  |
| Class A2 | -0.54% | -1.20% |  | -0.80% |  |
| Class B | -0.77% | -1.28% |  | -1.34% |  |
| Class C | -0.27% | -0.41% |  | -0.36% |  |
| Overall | -0.58% | -1.01% |  | -0.88% |  |
| Enc Time [%] | 125% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.17% |  | -0.42% |  | 0.09% |
| Class F | -0.17% |  | -0.30% |  | -0.03% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.52% | 0.00% |  | -0.07% |  |
| Class C | -0.25% | -0.01% |  | 0.08% |  |
| Class E | -0.32% | 0.57% |  | 0.24% |  |
| Overall | -0.38% | 0.14% |  | 0.06% |  |
| Enc Time [%] | 122% |  |  |  |  |
| Dec Time [%] | 100% |  |  |  |  |
| Class D | -0.12% |  | 0.75% |  | 0.13% |
| Class F | -0.23% |  | 0.34% |  | -0.09% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.70% | -1.27% |  | -0.72% |  |
| Class A2 | -0.61% | -1.10% |  | -0.83% |  |
| Class B | -0.87% | -1.04% |  | -1.41% |  |
| Class C | -0.36% | -0.25% |  | -0.34% |  |
| Overall | -0.65% | -0.89% |  | -0.87% |  |
| Enc Time [%] | 133% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.20% |  | 0.18% |  | -0.01% |
| Class F | -0.12% |  | -0.19% |  | -0.17% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.58% | 0.07% |  | 0.16% |  |
| Class C | -0.36% | -0.07% |  | 0.26% |  |
| Class E | -0.29% | 0.28% |  | 0.85% |  |
| Overall | -0.43% | 0.08% |  | 0.37% |  |
| Enc Time [%] | 133% |  |  |  |  |
| Dec Time [%] | 100% |  |  |  |  |
| Class D | -0.18% |  | 0.35% |  | 0.51% |
| Class F | -0.34% |  | -0.44% |  | -0.58% |

# References

- C.W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” JVET-K1030, Ljubljana, SI, 10–18 July 2018.

- J. Pfaff et al., “Signal Adaptive Diffusion Filters For Video Coding,” JVET-J0038, San Diego, USA, April 2018.

- J. Rasch et al., “CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5)” JVET-K323, Ljubljana, SI, 10–18 July 2018.

- F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-K1010, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).