| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0160 |
| --- | --- |

| Title: | AHG14: Intra Refresh Test conditions and Anchors generation Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson, David Gommelet VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com david.gommelet@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution proposes encoder only modifications as well as test conditions to be used as reference for studying Intra Refresh normative tool. This is an answer to the first mandates of the AHG14 on “Low-latency random access”. After describing the encoder modifications and the proposed test conditions, results are reported against a Low-Delay reference with a comparable Intra period. Significant objective losses are reported.

Authors recommend to integrate the proposed encoder modifications in next VTM software and to use the test conditions for studying normative Intra Refresh tool.

# Introduction

At the 11th JVET meeting, an improved Intra Refresh proposal was presented, as documented in JVET-K0212 [1]. This contribution proposed a technique to improve the use of Cyclic Intra Refresh, a technique that is commonly used for real-time implementation of HEVC video encoders. Intra Refresh provides several desirable features, such as bit-rate smoothing, maintaining Intra data bit-rate under control, low delay and natural error resilience limiting temporal error propagation along frames. For video encoder’s manufacturers this feature is considered as mandatory for real-life applications.

As stated in the AHG14 mandates, it is necessary to define relevant test conditions as well as anchors for handling related proposals. This document proposes Intra Refresh modifications at encoder level and dedicated test conditions, necessary to reflect how Intra Refresh would be implanted by encoder manufacturer in a similar way as in previous standards.

# Anchor generation proposal

## Proposed software modifications

Figure 1: Intra Refresh principle.

The software modifications to support Intra Refresh tool at the encoder-level in VTM software include:

- Intra prediction mode forced on coding unit on column basis.

- Constrained Intra Prediction enabled to ensure reconstruction of Intra CU.

- Motion vectors constrained to point within the Clean area while taking into account an additional margin to avoid motion compensation filters error spreading (5 pixels for instance).

- Removing of former reference pictures when re-looping the Intra column.

- Disabling of Adaptive Loop Filter at CTU level for CTU including Intra refresh pixels.

The additional parameters added to the configuration file are as follows:

IntraRefreshType			  : 1			# Intra Refresh type flag 0: disable, 1: classical, 2: improved

IntraRefreshStart			  : 0			# Position of the first column of Intra refresh (in pixels)

IntraRefreshDirection		  : 0			# Direction of Intra Refresh 0: left to right, 1: right to left

IntraRefreshPeriod			  : 32		# Period of the Intra Refresh

## Proposed test conditions

The Low Delay configuration file is used as a basis.

The period of Intra Refresh is set at 32 to match the random access Intra Period. The size of the column for Intra Refresh is then computed automatically to match this. The sizes are reported in following table:

| Resolution | Column Size |
| --- | --- |
| 3840x2160p | 120 |
| 1920x1080p | 64 |
| 1280x720p | 40 |
| 832x480p | 32 |
| 704x576p | 24 |
| 416x240p | 16 |

The GOP size is set to 1 with a QP offset of 1 in order to be close to an Intra Refresh scenario where the goal is to keep the same picture size along the sequence.

## On Column-based Intra Refresh

This study focus on Column-based Intra Refresh which is actually the most widely used by professional encoders for former standards (HEVC, AVC, MPEG-2). While Line-based or other pattern can be relevant for given applications, the authors would like to provide several input in favor of Column-based Intra Refresh to justify its used instead of Line-based refresh for this study:

- Column-based Intra Refresh allows to easily regulate bits consumption within a picture since the encoder is not directly encoding one full row in Intra so it has the time to react if the consumption is too high.

- Line-based Intra refresh increases the minimal quantity of Intra the encoder can force so it reduces the coding efficiency. Since the goal is to get the better coding efficiency, by using row which matches a slice, it is necessary to do this at CTU level so 64x64 in HEVC and 128x128 in VVC. Thanks to the column-based pattern, the size of the Intra area can be setup with a finer granularity (down to 8x8 thanks to CU/PU properties). In addition the width is longer than the height, this can lead to a lot of Intra pixels with Line-based and consequently a significant impact on coding efficiency.

- The camera motion is commonly more horizontal so that it is easier to hide the Intra refresh pattern with column based solution.

# Results

In this section, the performances for the proposed Intra Refresh anchor are compared to a low-delay reference without Intra Refresh which is encoded with a same Intra period in order to have a fair comparison with the same amount of forced Intra blocks. The goal of this comparison is to provide input information on the coding efficiency loss due to the use of Intra Refresh technique. However, only the Intra Refresh encodings achieve the low-latency goal by allowing to maintain a constant bits consumption by picture.

The following results were generated using the VVC software VTM v2.0.1 with the proposed test conditions. As introduced previously, the proposed configuration consists to use the “encoder_lowdelay_vtm.cfg” configuration with ‘GOPSize’ set to 1. The only reference B frame has a ‘QPoffset’ set to 1 and uses the following ‘reference pictures’: -1 -2 -3 -4 .

Section 3.1 and 3.2 reports the results over the Class B, C, D, E and F. The tested reference uses the proposed configuration with ‘IntraPeriod’ set to 32 and Intra Refresh disabled (‘IntraRefreshType’ set to 0). The proposed Intra Refresh at encoder only is configured with ‘IntraPeriod’ set to -1, ‘IntraRefreshType’ set to 1, ‘IntraRefreshDirection’ set to 0 (left to right), ‘IntraRefreshStart’ set to 0 and ‘IntraRefreshPeriod’ set to 32.

In order to highlight the origin of the coding efficiency loss, two tests are reported for Intra Refresh:

- The first one brings the results when the only modification is the Intra column addition instead of the Intra picture at a given period. This test doesn’t allow reconstructing the stream in case of random access so it is not workable in practice.

- The second test adds all the constraints as described in section 2.1.

## Test 1: Intra Refresh without constraints (not random-access)

In this case, the Intra Refresh version only includes the forcing of Intra column. The size of the column depends on the frame size (refer to Section 2.2).

|  | Low delay B Intra Refresh Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_VTM2.0_CIR.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class C | 5,98% | 6,92% | 7,37% | 133% | #DIV/0! |
| Class E | 16,46% | 15,95% | 20,39% | 143% | #DIV/0! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class D | 5,92% | 11,44% | 12,52% | 130% | #DIV/0! |
| Class F (optional) | 11,85% | 12,24% | 12,60% | 148% | #DIV/0! |

Reported results demonstrate a significant loss when introducing such Intra Refresh instead of a full Intra picture, especially for simple sequences which are usually well predicted with Inter and Skip modes (Class E and F). The introduction of such Intra area indeed increases the Intra residual proportion and also breaks locally the Inter motion vector prediction. This result must be analyzed according to the results of next section which include all the Intra Refresh constraints.

## Test 2: Intra Refresh with encoder constraints (random-access)

In this case, the streams are fully decodable thanks to additional encoder constraints. The size of the column depends on the frame size (refer to Section 2.2).

|  | Low delay B Intra Refresh Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_VTM2.0_CIR.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class C | 22,14% | 32,03% | 34,76% | 130% | #DIV/0! |
| Class E | 71,91% | 98,70% | 108,36% | 159% | #DIV/0! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #DIV/0! |
| Class D | 24,57% | 41,63% | 46,01% | 140% | #DIV/0! |
| Class F (optional) | 53,14% | 58,97% | 61,11% | 164% | #DIV/0! |

The loss is significant when integrating the constraints to the encoder in order to ensure the reconstruction. In particular, the motion vector constraints which forbid the usage of several motion vector candidates close to the refreshed area, often lead to selecting Intra mode in those area. This is particularly the case on simple sequences with fixed camera and really small motion such as the Class E sequences where the loss is multiplied by more than four after addition of the constraints. Otherwise, with classical sequences, the loss is in the range of 22% for Y component and 35% for U and V components. This is an even too much significant loss for such a key application.

# Conclusion

This document proposes tests conditions and Intra Refresh anchors generation for studying the opportunity to have normative Intra Refresh tools in VVC. The encoder-only modifications studied in this document are representative of professional implementations for former standards. The reported performances have been compared to a Low-Delay B encoding with a comparable Intra period and the average loss are xx%. Such a significant loss is really undesirable for a key feature like low-latency and can be managed efficiently with normative modifications.

# Recommendation

It is recommended to integrate these encoder only modifications within the next VTM software version and to add the proposed test conditions to the CTC.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

# Patent rights declaration(s)

VITEC does not have any current or pending patent rights relating to the technology described in this contribution.