| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0162 |
| --- | --- |

| Title: | CE2: Alf with Multiplication Replaced by Bit-Shifting (Test 2.5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik Biao Wang Han Gao Anand Meher Kotra Jianle Chen | Tel: Email: | semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

In the contribution it is proposed to use logarithmic quantization of ALF filters in such a way that filter coefficients can assume only the values: {-128, -64, -32, -16, -8, -4, 0, 4, 8, 16, 32, 64, 128}. Accordingly the proposed method reportedly allows simplified hardware implementations due to the fact that 10 out of 13 luma and 4 out of 7 chroma multiplication operations per pixel sample are replaced by simple bit-shift operations. Experimental results of the method are as follows:

Over VTM Anchor (AI, RA, LDB): 0.06%, 0.04%, -0.08% difference in luma coding gain with unchanged encoding and decoding times.

# Introduction

The proposed method was first introduced in the Huawei’s CfP response [1], and is currently evaluated as part of the CE2. The method addresses simplification of the GALF in the VTM, which is considered to be complicated with respect to software and hardware implementations, especially due to the high number of multiplications that are necessary per pixel.

# Problem Statement

In current BMS the GALF supports multiple filter kernels with a diamond shaped support region of 7x7. The number of multiplication operations in order to filter one pixel sample is 13 + (7 + 7)/4 = 16.5, which is considered an important hardware design restriction.

# Proposed Method

According to the proposed method, the filter coefficients used in filtering of the luma component are divided into 2 groups. The first group contains that coefficients {C1, C2, C3, C4, C5, C6, C8, C9, C10, C11}, and the second group contains the rest of the coefficients {C7, C12, C13}. The first group coefficients are normatively restricted to use only the following set of values: {-128, –64, –32, –16, –8, –4, 0, 4, 8, 16, 32, 64, 128}, which corresponds to logarithmic (in base 2) quantization of filter coefficients. Hence multiplication with those coefficients can be implemented as a single bit shit operation. The allowed filter coefficient values are mapped to {-6, –5, –4, –3, –2, –1, 0, 1, 2, 3, 4, 5, 6} respectively before binarization in order to reduce the signaling overhead.

The same quantization method is applied to chroma filter. The chroma filter coefficient are grouped into 2 groups, where the first group contains the coefficients {C1, C2, C4, C5} and the second group contains the coefficients {C3, C6, C7}. The first group applies the same normative quantization as in the luma filter case.

Figure 1: Filter coefficients of 9x9 filter kernel according to proposal

It is reported that the proposed logarithmic quantization of the filter coefficients allow for hardware implementations that require much less logic gates, as 10-bit multiplication operation is replaced by simple bit-shifting, and the range of the filter coefficients are restricted to [-128, 128].

According to the proposal, the number of multiplications per pixel is reduced from 16.5 to 4.5, 12 multiplication operations are replaced with bit-shifting operation.

# Experimental Results

The experimental results on top of VTM2.0.1 is presented below.

Table 1: The experimental results of the proposed method on top of VTM2.0.1 anchor

# Conclusion

A logarithmic quantization of ALF filters is proposed to restrict the coefficient values to the set {-128, -64, -32, -16, -8, -4, 0, 4, 8, 16, 32, 64, 128}. Accordingly the proposed method reportedly allows simplified hardware implementations due to the fact that 10 out of 13 luma and 4 out of 7 chroma multiplication operations per pixel sample are replaced by simple bit-shift operations. It is proposed to be included in VVC.

# Conclusion

[1] “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, A. Alshin et. al.,  doc. JVET-J0024, April2018.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).