| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0163 |
| --- | --- |

| Title: | CE9: Report on the results of tests CE9.2.15 and CE9.2.16 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Anand Meher Kotra Biao Wang, Han Gao, Jianle Chen Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz Hongbin Liu, Li Zhang, Kai Zhang Yue Wang, Pengwei Zhao, Dingkun Hong Daniel Luo, Xiaoyu Xiu, Yuwen He, Yan Ye | Tel: Email: | semih.esenlik@huawei.com chunchic@qti.qualcomm.com wchien@qti.qualcomm.com liuhongbin.01@bytedance.com daniel.luo@interdigital.com |
| Source: | Huawei Technologies Co.,Ltd., Qualcomm Inc., InterDigital Communications, Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution document reports the results of the joint CE tests CE9.2.15 and CE9.2.16. The following modifications are proposed in both tests in order to address the complexity issues of DMVR in current BMS2.1:

- Application of bilinear interpolation filtering during motion vector refinement instead of 8-tap DCTIF,

- Reference sample padding in order to eliminate the additional memory access,

- Early-skip condition based on MVD between merge candidates,

- Early-skip condition based on SAD cost function in center search coordinate,

- Disabling of DMVR for small blocks,

- Changing DMVR application condition from (POC – POC1)x(POC-POC2)<0 to (POC-POC1) == (POC2 – POC),

- MRSAD computation using every second row of samples (only applied to CE9.2.16).

On top of VTM2.0.1 anchor the following experimental results are reported:

- CE9.2.15: 1.65% luma, 1.74% and 1.88% chroma coding gain with 3% increase in encoding time and 11% in decoding time.

- CE9.2.16: 1.59% luma, 1.70% and 1.78% chroma coding gain with 2% increase in encoding time and 10% in decoding time.

The proposed modifications are also independently tested in CE9.

# Introduction

Decoder Side Motion Vector Refinement (DMVR) was first introduced in 4th JVET meeting in Chengdu [1]. It was later included in JEM software in the 5th JVET meeting and extensively tested since. Finally it was included in the BMS software in 10th JVET meeting in San Diego. In 11th JVET meeting in Ljubljana usage of refined motion vectors in the spatial MV prediction loop was disabled to address hardware design limitation of DMVR.

Present contribution introduces further complexity reduction measures for DMVR in order to address remaining hardware implementation challenges.

In current DMVR in BMS2.1, 8-tap DCTIF interpolation filtering is used in order to generate predictions from forward and backward direction for refinement search. After the predictions are generated a search range of 2 pixels are used for integer and half-pel refinement search operations.

Although 8-tap interpolation filtering can be implemented very efficiently, it is still desirable to reduce the number of operations. The first aspect of the contribution addresses this issue by replacing the 8-tap interpolation with simple 2 tap bilinear filtering.

The second aspect of the contribution addresses the additional memory access requirement of DMVR by using reference sample padding. Accordingly the additional memory access is eliminated completely without any loss in coding gain.

The third aspect is disabling DMVR for small blocks with sizes 4x4 and 8x4 in order remove the DMVR from reconstruction bottleneck.

Two early skip conditions are introduced in order to reduce the encoding and decoding times. According to first condition, the SAD matching cost function in the center search coordinate is compared with a threshold, if the cost is smaller than the threshold the DMVR process is skipped. According to the second condition motion vector difference between the merge candidate pairs are checked and DMVR is skipped if the difference of the selected merge candidate is less than a pre-defined threshold w.r.t. previous merge candidates in the same merge candidate list.

The POC based DMVR application condition is modified such that DMVR is applied only when reference pictures are equidistant to current picture in respective opposite directions.

Finally in test CE9.2.16 the MRSAD computation is further simplified by using only every second sample row.

# Technical Description

The tests CE9.2.15 and CE9.2.16 are aimed at reducing the complexity of DMVR in BMS2.1. Accordingly the DMVR in BMS2.1 is used as codebase and the following modifications are applied:

## Reference sample padding

Reference sample padding is applied in order to extend the reference sample block that is pointed by the initial motion vector. If the size of the coding block are given by “w” and “h”, then it is assumed that a block of size w+7 and h+7 is retrieved from the reference picture buffer. The retrieved buffer is then extended by 2 samples in each direction by repetitive sample padding using the nearest sample. Afterwards the extended reference sample block is used to generate the final prediction once the refined motion vector is obtained (which can deviate from the initial motion vector 2 samples in each direction).

It is noted that this modification eliminates the external memory access requirement of DMVR completely without any coding loss. The independent experimental result for reference sample padding is reported in CE9.2.2.

## Bilinear interpolation instead of 8-tap DCTIF

According to the proposal bilinear interpolation is applied during the DMVR search process, which means that the predictions used in MRSAD computation are generated using bilinear interpolation. Once the final refined motion vectors are obtained regular 8-tap DCTIF interpolation filter is applied to generate final predictions.

## Disabling of DMVR for small blocks

In test CE9.2.15 DMVR is disabled for blocks with size 4x4. In CE9.2.16 DMVR is disabled for blocks 4x4, 4x8 and 8x4. The independent experimental result for disabling DMVR for small blocks are reported in CE9.2.9.

## Early termination based on MV difference between merge candidates

An additional condition is imposed on DMVR to confine the MV refinement process. With it, DMVR is conditionally disabled when the below condition is satisfied.

- The MV difference between the selected merge candidate and any of the previous ones in the same merge list is less than a pre-defined threshold (that is, ¼-, ½- and 1-pixel-wide intervals for CUs with less than 64 pixels, less than 256 pixels and at least 256 pixels, respectively).

The independent experimental result for this technique is reported in CE9.2.13.

## Early termination based on SAD cost at the center search coordinate

The sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the initial motion vectors of the current CU is calculated. If the SAD is no larger than a predefined threshold, i.e., 2BDepth-9 per sample, the DMVR is skipped; otherwise, the DMVR is still applied to refine the two motion vectors of the current block.

## DMVR application condition

The DMVR application condition is (POC – POC1)x(POC-POC2)<0 as it is implemented in BMS2.1 is replaced by the new condition (POC – POC1)==(POC2 – POC). This means that DMVR is applied only if reference pictures are in opposite time directions and are equidistant to current picture.

## MRSAD computation using every second row

The MRSAD cost is computed only for odd numbered rows of a block, the even numbered samples rows are not considered. Accordingly the number of operations for the MRSAD calculation is halved. The independent experimental result for computation of MRSAD by using only odd numbered sample rows are reported in CE9.2.9.

The results for MRSAD computation using every second row is reported in test CE9.2.16. In CE9.2.15 all samples within the block are used in MRSAD computation.

# Test conditions and anchors

In CE9 the BMS2.1 release candidate 1 is used as anchor software due to the fact that the release of BMS2.1 is delayed, which was announced in the CE9 email reflector. The SIMD setting is SSE42 in both anchor and test.

# Simulation Results

Results for 2 sets of simulations are provided.

Simulation results 1:

- Anchor 1: BMS2.1rc1 with VTM encoder configuration

- Test: Anchor + proposal.

Simulation results 2:

- Anchor 2: BMS2.1rc1 with VTM encoder configuration + DMVR on

- Test: Anchor + proposal.

## Experimental results for CE9.2.15

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.79% | -1.59% | -2.01% | 103% | 108% |
| Class A2 | -2.28% | -2.48% | -2.53% | 102% | 108% |
| Class B | -1.39% | -1.49% | -1.53% | 103% | 112% |
| Class C | -1.42% | -1.61% | -1.73% | 102% | 116% |
| Class E |  |  |  |  |  |
| Overall | -1.65% | -1.74% | -1.88% | 103% | 111% |
| Class D | -1.59% | -1.77% | -1.88% | 103% | 115% |

Table1: Experimental results of CE9.2.15 with respect to Anchor 1(BMS2.1rc1 with VTM encoder configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM config + DMVRon |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.10% | 99% | 93% |
| Class A2 | 0.05% | -0.03% | 0.00% | 99% | 93% |
| Class B | -0.07% | -0.01% | 0.00% | 98% | 91% |
| Class C | -0.03% | 0.00% | -0.05% | 97% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.03% | 98% | 92% |
| Class D | 0.15% | -0.02% | -0.04% | 97% | 92% |

Table2: Experimental results of CE9.2.15 with respect to Anchor 2(BMS2.1rc1 with VTM encoder configuration + DMVR on)

## Experimental results for CE9.2.16

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.78% | -1.65% | -1.89% | 102% | 107% |
| Class A2 | -2.22% | -2.43% | -2.45% | 101% | 107% |
| Class B | -1.34% | -1.45% | -1.48% | 102% | 111% |
| Class C | -1.30% | -1.51% | -1.59% | 102% | 114% |
| Class E |  |  |  |  |  |
| Overall | -1.59% | -1.70% | -1.78% | 102% | 110% |
| Class D | -1.36% | -1.49% | -1.60% | 102% | 114% |

Table3: Experimental results of CE9.2.16 with respect to Anchor 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 with VTM config + DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.03% | 98% | 93% |
| Class A2 | 0.11% | 0.02% | 0.08% | 98% | 92% |
| Class B | -0.02% | 0.03% | 0.06% | 97% | 91% |
| Class C | 0.10% | 0.11% | 0.09% | 97% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | 0.06% | 98% | 92% |
| Class D | 0.39% | 0.26% | 0.25% | 97% | 91% |

Table4: Experimental results of CE9.2.16 with respect to Anchor 2

# Conclusion

This contribution document reports the results of the joint CE tests CE9.2.15 and CE9.2.16. The proposed modifications reportedly reduces the complexity of DMVR considerably with almost no reduction in coding gain compared to DMVR in BMS2.1. Specifically the test CE9.2.16 provides -1.59% luma, -1.70% and -1.78% chroma coding gain compared to VTM2.0.1 with 2% increase in encoding time and 110% in decoding time. It is proposed to include the test CE9.2.16 into VVC.

# Reference

[1] “Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching”, X. Chen, J. An, J. Zheng, doc. JVET-D0029, October 2016

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, August 2018

[3] “ Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, S. Esenlik, Y.-W. Chen, F. Chen, doc. JVET-K1029, August 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).