| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0165-v1 |
| --- | --- |

| Title: | CE3-6.2.1: Extended MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com jin78.heo@lge.com jangwon84.choi@lge.com jungah.choi@lge.com sunmi.yoo@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an extended MPM list which is as simple as existing 3 MPM. In the proposed method, as shown in Figure 1, intra modes of neighbor position A and B are deployed for MPM list generation. Follow the philosophy of 3 MPM in VTM2.0.1, MPM lists are generated using intra modes of L and A only. In the test CE3-6.2.1, abovementioned method is tested on top of BMS 2.0.1 with VTM configuration [1]. The CE3-6.2.1a tests proposed extended MPM method with CTU boundary restriction. In the other hand, CE3-6.2.1b test proposed method with CTU boundary restriction off to have an insight on the tradeoff between performance and line buffer usage. The experiment results reportedly show 0.29%, 0.11% and 0.33% and 0.13% bit rate savings for VTM-AI, VTM-RA for CE3-6.2.1a, and VTM-AI and VTM-RA for CE3-6.2.1b, respectively.

Proposed Method

At the last meeting, HEVC style 3 MPM is adopted in VVC. The rationale behind 3 MPM is simplicity and throughput advantage. 6 MPM existing in JEM is too complex: including exhaust neighbor position search and pruning process, three steps for list generation, line buffer requirement and parsing dependency. Hence following philosophy of 3 MPM, an extended 6 MPM list method is proposed.

In the proposed method, as shown in Figure 1, only intra mode of neighbor position A and B denoted as LEFT and ABOVE are used for MPM list generation. Pseudo code is as below.

Table 1. Pseudo code for proposed method

Determine LEFT and ABOVE intra modes

Set MPM as MPM_ordering_0

If (LEFT==ABOVE)

If (LEFT>=DC_idx), then set MPM as MPM_ordering_1

Else if (LEFT>DC_idx and ABOVE>DC_idx), then set MPM as MPM_ordering_2

Else if (LEFT+ABOVE> DC_idx), then set MPM as MPM_ordering_3

Figure 1. Neighbor blocks of current block

Complexity Analysis

|  | Max number of neighbors to access | Line buffer required? | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increment | Max number of bit operation | Parsing dependency? | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3MPM | 2 | N | 2 | 4 | 15 | 0 | 0 | N | 1 | 1 or 2 | N | - |
| CE3-6.2.1a | 2 | N | 6 | 8 | 26 | 0 | 0 | N | 1 | 1 or 2 | N | - |

Experimental Results

The proposed method is implemented on top of BMS 2.0.1, and evaluated for VTM configuration according to the common test conditions [2] and Core Experiment [3].

Table 1 Experiment result for CE3-6.2.1a all intra (AI) using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.16% | -0.17% | 99% | 100% |
| Class A2 | -0.25% | -0.24% | -0.25% | 100% | 100% |
| Class B | -0.28% | -0.19% | -0.22% | 100% | 100% |
| Class C | -0.30% | -0.26% | -0.08% | 100% | 100% |
| Class E | -0.40% | -0.38% | -0.35% | 99% | 99% |
| Overall | -0.29% | -0.24% | -0.21% | 100% | 100% |
| Class D | -0.19% | 0.10% | -0.37% | 100% | 100% |

Table 2 Experiment result for CE3-6.2.1a random access (RA) using BMS 2.0.1 with VTM config as anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.03% | -0.06% | 100% | 100% |
| Class A2 | -0.12% | 0.00% | -0.07% | 100% | 100% |
| Class B | -0.09% | -0.11% | 0.14% | 100% | 100% |
| Class C | -0.10% | -0.08% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.05% | 0.01% | 100% | 100% |
| Class D | -0.08% | -0.31% | 0.38% | 100% | 100% |

Table 3 Experiment result for CE3-6.2.1b all intra (AI) using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.22% | -0.20% | 100% | 99% |
| Class A2 | -0.29% | -0.30% | -0.33% | 99% | 99% |
| Class B | -0.31% | -0.25% | -0.29% | 99% | 100% |
| Class C | -0.34% | -0.35% | -0.20% | 100% | 101% |
| Class E | -0.49% | -0.49% | -0.55% | 99% | 99% |
| Overall | -0.33% | -0.32% | -0.30% | 100% | 100% |
| Class D | -0.19% | 0.03% | -0.39% | 100% | 101% |

Table 4 Experiment result for CE3-6.2.1b random access (RA) using BMS 2.0.1 with VTM config as anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | -0.05% | 100% | 100% |
| Class A2 | -0.14% | -0.18% | -0.15% | 100% | 100% |
| Class B | -0.12% | -0.15% | 0.10% | 100% | 99% |
| Class C | -0.11% | 0.03% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.09% | 0.00% | 100% | 100% |
| Class D | -0.09% | -0.45% | 0.35% | 100% | 99% |

Summary

The proposed extended MPM list has 0.29% bit rate reduction over VTM-2.0.1, but as simple as existing 3 MPM.

References

- BMS-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, Document of Joint Video Experts Team (JVET), JVET-K1023, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).