| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0179-v2 |
| --- | --- |

| Title: | CE3: 4-tap interpolation filter combined with bilateral reference sample filter (Tests 3.2.1 and 3.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-433 philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

v2: added additional results and description for variation of test 3.2.1

# Abstract

This contribution presents the results on the tool under test in CE3.3 for studying the performance of combining 4-tap interpolation filters with the bilateral reference sample filter proposed in JVET-K0043. Results for AI show -0.59% BD-rate for bilateral reference sample filter plus 4-tap cubic interpolation filter (test 3.2.1) and -0.60% BD-rate for bilateral reference sample filter plus 4-tap cubic and 4-tap Gauss interpolation filter (test 3.2.2).

# Introduction

The intra prediction can be improved by reference sample filtering, such as the bilateral filter proposed in JVET-K0043 [1], and by interpolation filtering, such as the 4-tap cubic and 4-tap Gaussian filters included in JEM7. For CE3.3 tests 3.2.1 and 3.2.2 [2], the combined performance of bilateral reference sample filter and 4-tap interpolation filters is studied.

# Proposed method

Reference sample filtering

In VTM2/HEVC luma reference samples are filtered with a [1 2 1]/4 smoothing filter using the MDIS (mode dependent intra smoothing) condition based on block size and prediction mode.

For tests 3.2.1 and3.2.2 the bilateral reference sample filter proposed in JVET-K0043 [1] is applied to luma blocks with size W×H≥16×16 and all prediction modes m with minm-mHOR, m-mVER>1. For all other blocks the [1 2 1]/4 smoothing filter is applied under the same conditions as in VTM2/HEVC.

Interpolation filtering for angular prediction

In VTM2/HEVC a 2-tap linear interpolation filter is used for prediction of angular modes that are not strictly horizontal, vertical or diagonal.

In JEM7, two different 4-tap interpolation filters (cubic and Gaussian) were included, replacing the 2-tap linear interpolation filter. The cubic interpolation filter is used for all blocks with a prediction size of k≤8, i.e. m×k and k×n for horizontal and vertical prediction modes, respectively. For all other blocks, the Gaussian interpolation filter is used.

Table 1. Test configuration (grey cells indicate VTM2 default tools)

|  | Reference sample filter |  | Angular interpolation filter |  |
| --- | --- | --- | --- | --- |
|  | W×H≥16×16 | otherwise | k≤8 | otherwise |
| Test 3.2.1 | bilateral | [1 2 1]/4 MDIS | 4-tap cubic | 2-tap linear |
| Test 3.2.2 |  |  |  | 4-tap Gaussian |

# Results

According to the CE3 description in JVET-K1023 [2] and the common test conditions in JVET-K1010 [3], the tests are evaluated for intra-only (AI) and random-access (RA) configuration with VTM-2.0.1 software in VTM configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 2. Result for test 3.2.1, VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.51% | -0.42% | -0.42% | 104% | 101% |
| Class A2 | -0.30% | -0.31% | -0.38% | 103% | 101% |
| Class B | -0.43% | -0.56% | -0.58% | 102% | 103% |
| Class C | -0.95% | -1.20% | -1.05% | 100% | 100% |
| Class E | -0.74% | -0.85% | -0.94% | 102% | 101% |
| Overall | -0.59% | -0.68% | -0.69% | 102% | 101% |
| Class D | -0.53% | -0.39% | -0.79% | 100% | 99% |
| Class F (optional) | -0.56% | -0.74% | -0.76% | 101% | 102% |

Table 3. Result for test 3.2.1, VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.30% | -0.34% | -0.25% | 100% | 100% |
| Class A2 | -0.19% | -0.43% | -0.53% | 100% | 99% |
| Class B | -0.22% | -0.52% | -0.43% | 101% | 98% |
| Class C | -0.34% | -0.39% | -0.55% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.43% | -0.45% | 100% | 100% |
| Class D | -0.26% | -0.46% | -0.42% | 101% | 98% |
| Class F (optional) | -0.14% | -0.30% | -0.28% | 100% | 100% |

Table 4. Result for test 3.2.2, VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.70% | -0.50% | -0.53% | 106% | 102% |
| Class A2 | -0.37% | -0.28% | -0.38% | 105% | 102% |
| Class B | -0.45% | -0.50% | -0.51% | 104% | 104% |
| Class C | -0.81% | -1.01% | -0.87% | 102% | 100% |
| Class E | -0.69% | -0.51% | -0.75% | 103% | 102% |
| Overall | -0.60% | -0.58% | -0.61% | 104% | 102% |
| Class D | -0.54% | -0.49% | -0.82% | 102% | 100% |
| Class F (optional) | -0.45% | -0.68% | -0.64% | 102% | 102% |

Table 5. Result for test 3.2.2, VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.43% | -0.42% | -0.39% | 101% | 102% |
| Class A2 | -0.26% | -0.49% | -0.38% | 101% | 98% |
| Class B | -0.25% | -0.57% | -0.50% | 101% | 99% |
| Class C | -0.32% | -0.50% | -0.49% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.51% | -0.45% | 101% | 100% |
| Class D | -0.29% | -0.33% | -0.37% | 101% | 101% |
| Class F (optional) | -0.08% | -0.24% | -0.12% | 101% | 101% |

# Cross-check

The cross-check of tests 3.2.1 and 3.2.2 was done by Vasily Rufitskiy (Huawei).

# Additional results

A variation of test 3.2.1 was additionally tested and cross-checked, which contains the following modifications of the 4-tap cubic interpolation filtering part from test 3.1.2:

- Padding simplification (reference samples are padded prior to 4-tap interpolation instead of conditions in the interpolation process)

- Restriction of 4-tap interpolation to luma channel

- Condition for 4-tap cubic filter is unfiltered reference samples (MDIS)

- No direct copy path for non-fractional positions

- Fix for threshold of MDIS condition

The software changes of test 3.2.1 with modified interpolation filtering have been added to branch CE3-3.2 (https://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS.git).

The cross-check of tests 3.2.1 with modified interpolation filtering was done by Geert van der Auwera (Qualcomm).

Table 6. Result for test 3.2.1 with modified interpolation filtering, VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.50% | -0.35% | -0.37% | 103% | 104% |
| Class A2 | -0.30% | -0.23% | -0.28% | 102% | 101% |
| Class B | -0.49% | -0.52% | -0.57% | 101% | 103% |
| Class C | -1.05% | -1.10% | -0.99% | 99% | 97% |
| Class E | -0.91% | -0.92% | -0.94% | 101% | 99% |
| Overall | -0.66% | -0.64% | -0.64% | 101% | 101% |
| Class D | -0.58% | -0.39% | -0.82% | 99% | 98% |
| Class F (optional) | -0.52% | -0.73% | -0.74% | 100% | 99% |

Table 7. Result for test 3.2.1 with modified interpolation filtering, VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.26% | -0.07% | -0.12% | 100% | 100% |
| Class A2 | -0.21% | -0.10% | -0.17% | 100% | 98% |
| Class B | -0.25% | -0.27% | -0.10% | 100% | 99% |
| Class C | -0.37% | -0.20% | -0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.18% | -0.16% | 100% | 99% |
| Class D | -0.31% | -0.55% | -0.03% | 101% | 97% |
| Class F (optional) | -0.03% | -0.07% | -0.11% | 100% | 97% |

# Conclusion

The results show that the combination of bilateral reference sample filter and 4-tap interpolation filters for angular prediction leads to -0.6% and -0.3% BD-rate for AI and RA, respectively, without increasing the encoding and decoding time. We recommend adopting test 3.2.1 to VVC and VTM, as it requires less stored filter weights and fewer operations than test 3.2.2 with almost the same coding efficiency.

# References

- P. Merkle, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Bilateral reference sample filter (Test 2.7.1),” JVET-K0043, Ljubljana, SI, July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-K1023, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).