| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0180 |
| --- | --- |

| Title: | Crosscheck for CE3-1.1.1 and CE3-1.1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elie Mora Anthony Nasrallah Mickael Raulet 6 Rue Dewoitine 78140 Vélizy-Villacoublay France | Email: Email: Email: | e.mora@ateme.com a.nasrallah@ateme.com m.raulet@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report for CE3 tests 1.1.1 and 1.1.2. These two tests relate to the Multiple Reference Line (MRL) prediction in the intra coding mode. Test 1.1.1 uses two additional reference lines wherein all intra angular modes are tested for each reference line. Test 1.1.2 excludes testing of odd modes (i.e., only even modes are tested) for the additional reference lines. We confirm that our results match those provided by the proponents (ITRI for test 1.1.1, and HHI for test 1.1.2).

- Introduction

The software package was provided at the following git branch : https://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS.git, accessible only by CE3 participants and crosscheckers. The branch “CE3-1.1” contained the code for both tests. The default configuration pertained to test 1.1.1. In order to test 1.1.2, a single macro needed to be enabled : MRL_EXCLUDE_ODD_MODES .

The testing conditions defined in JVET-K1023 [1] were used. The BMS version 2.0.1 was used with ‘vtm’ configuration files to produce the VTM anchor. Two configurations were tested : AI and RA. The source code was compiled using GCC v4.8.5, and the test jobs ran on an Intel(R) Xeon(R) Platinum 8180 server.

The available results are shown in the following tables, and detailed data can be found in the attached spreadsheet. It is confirmed that the obtained results perfectly match those of the proponents.

- Experimental results

Table 1 - Results of CE3-1.1.1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,37% | -0,10% | -0,17% | 140% | 98% |
| Class A2 | -0,46% | -0,23% | -0,29% | 142% | 98% |
| Class B | -0,59% | -0,30% | -0,35% | 136% | 98% |
| Class C | -0,85% | -0,54% | -0,57% | 123% | 97% |
| Class E | -0,70% | -0,44% | -0,37% | 126% | 97% |
| Overall | -0,61% | -0,33% | -0,36% | 133% | 98% |
| Class D | -0,30% | 0,14% | -0,17% | 124% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,33% | -0,18% | -0,30% | 108% | 101% |
| Class A2 | -0,34% | 0,02% | -0,12% | 103% | 98% |
| Class B | -0,32% | -0,27% | 0,00% | 106% | 100% |
| Class C | -0,33% | -0,17% | -0,17% | 109% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0,33% | -0,16% | -0,13% | 107% | 98% |
| Class D | -0,12% | -0,32% | -0,02% | 107% | 102% |

Table 2 - Results of CE3-1.1.2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,40% | -0,06% | -0,28% | 134% | 99% |
| Class A2 | -0,38% | -0,14% | -0,18% | 134% | 99% |
| Class B | -0,45% | -0,22% | -0,17% | 135% | 99% |
| Class C | -0,58% | -0,37% | -0,26% | 125% | 98% |
| Class E | -0,53% | -0,28% | -0,31% | 131% | 98% |
| Overall | -0,47% | -0,22% | -0,23% | 132% | 98% |
| Class D | -0,29% | 0,00% | -0,29% | 124% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,35% | -0,22% | -0,22% | 110% | 99% |
| Class A2 | -0,25% | -0,06% | -0,16% | 107% | 99% |
| Class B | -0,24% | -0,13% | -0,01% | 109% | 98% |
| Class C | -0,22% | -0,08% | -0,03% | 107% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,26% | -0,12% | -0,09% | 108% | 99% |
| Class D | -0,14% | -0,39% | -0,03% | 105% | 99% |

- Software inspection

The code was reviewed, it is confirmed that, in the default configuration, it performs test 1.1.1, and by enabling the MRL_EXCLUDE_ODD_MODES macro, it performs test 1.1.2. Results show that there is no encoder runtime gain coming from excluding testing of odd modes on additional reference lines. However, this exclusion does bring losses as the MRL gains reduce from 0.61 to 0.47 in all intra. It seems straightforward to not exclude odd modes from encoder intra search on additional reference lines.

- Acknowledgement

The work done in this crosscheck was supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

- References

- G. Van Der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-K1023, Ljubljana, July 2018