| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0185 |
| --- | --- |

| Title: | AHG11 & CE1-related: Luma 2xN and Nx2 Block Partitions Support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An Yu-Chen Sun Jian Lou | Tel: Email: | +86-13439745728 jicheng.ajc@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes to support luma 2xN and Nx2 block partitions under the current QTBTT block partitioning structure by decreasing the luma minMtSize down to 2 luma samples for I slice. The experiments results show that 4.7% BD-rate gain for classF, and 12% BD-rate gain for SCC class are achieved, under all intra configuration, with 10% encoding time increase and 24% decoding time increase.

Introduction

In current VVC, the minimum luma CB size is 4 luma samples, and the minimum chroma CB size is 2 chroma samples. The minimum CB size for luma and chroma can be set separately for I slice since the separate block partitioning tree is used for I slice. For examples, the minimum luma CB size can be set to 2 luma samples with the minimum chroma CB size still 2 chroma samples. But for P and B slices, the minimum CB size is set simultaneously for both luma and chroma since the shared block partitioning tree is used for P and B slices, for example, the minimum luma CB size equal to 2 samples implies the minimum chroma CB size equal to 1 sample for 4:2:0 sample format.

Luma 2xN support

In this contribution, the 2xN and Nx2 block partitions are supported under the current QTBTT block partitioning structure by setting the luma minMtSize down to 2 luma samples for I slice. Therefore, in the 2-D level, the minimum luma CB size is 2x2. In order to support the 2xN luma coding block, some changes are made straightforwardly as follows:

- The mode storage unit is modified to 2x2 sample level instead of 4x4 (software change);

- For the transform of 2xN (and Nx2) luma CB, only DCT2 is used, the DST7, and DCT8 are not supported;

- For the deblocking of luma 2-sample edge, the filtering decision and filtering operations are all changed to 2-line processing instead of 4-line processing.

Experimental results

The proposed luma 2xN support has been implemented on top of vtm2.0.1, and the results for all intra configuration are shown in Table 1 below. It can be seen that for the natural video 0.24% BD-rate gain is achieved, but the proposed method is extremely good for screen content video, for class F 4.7% BD-rate gain is achieved, and for class SCC 12% BD-rate gain is achieved with 10% encoding time increase and 25% decoding time increase.

Table 1. experimental results of the proposed luma 2xN

Next step

For the P and B slice, we haven’t implemented the luma 2xN partition support yet, since if the luma CB length is set to 2 samples then the corresponding chroma CB length should be 1 sample for the 4:2:0 sample format. We have another proposal JVET-L0184 which can separate the luma and chroma partitioning tree in all kinds of slices under arbitrary block size.

Therefore, we may have two directions for next step:

- Implement the luma 2xN with chroma 1xN/2 for P and B slice (luma chroma shared block partitioning tree for P and B slices).

- Separate the luma and chroma block partitioning tree for P and B slices first, and then implement the luma 2xN on top of it without chroma changing.

We hope to establish an adhoc group or CE to study this small size (2xN) block coding.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).