| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0195-v2 |
| --- | --- |

| Title: | CE6-related: MTS for non-square CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung, Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak (WILUS) Yonggil Lee (Humax) | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com yglee@humaxdigital.com |
| Source: | WILUS Inc., Humax |  |  |

_____________________________

- Abstract

This contribution proposes to apply MTS (Multiple Transform Selection) for large rectangular CUs. Currently, MTS is applied to a luma block only when both of the width and the height of the block is smaller than or equal to 32. It is proposed to loosen the restriction to apply MTS to the horizontal if the width is smaller than or equal to 32 even though the height is larger than 32, and vice versa. Compared with the current reference software, large rectangular blocks such as 64x32 and 32x64 can exploit selection of the transform kernels among DST-VII and DCT-VIII. Experimental results show that the proposed method achieves 0.00% (AI), -0.04% (RA) and -0.02% (LD) BD-rate with slightly increased encoding time compared to BMS-2.0.1 with VTM configuration. Especially, 0.14% BD-rate gain is observed for 4K resolution test sequences in class A1 with RA configuration. When MTS is enabled for inter coded blocks, experimental results show 0.00% (AI), -0.06% (RA) and -0.05% (LD) BD-rate compared to BMS-2.0.1 with VTM configuration. Experimental results on the proposed method with both the maximum BT size and the maximum TT size equal to 64 show -0.09% (AI), -0.07% (RA) and -0.04% (LD) BD-rate compared to BMS-2.0.1 with VTM configuration.

Introduction

In VTM 2 [1], MTS scheme is used for residual coding for both inter and intra coded blocks in order to achieve better coding performances. It uses multiple transforms from the DCT-VIII and DST-VII. Table 1 shows basis functions of the transform kernels used in VTM 2.

Table 1 – Transform basis functions of DCT-II, DCT-VIII and DST-VII for N-point input

| Transform Type | Basis function Ti(j), i, j = 0, 1, …, N−1 |
| --- | --- |
| DCT-II | Ti(j)=ω0∙2N∙cosπ∙i∙2j+12N where, ω0=2Ni=01i≠0ω0=2Ni=01i≠0ω0=2Ni=01i≠0 |
| DCT-VIII | Ti(j)=42N+1∙cosπ∙(2i+1)∙(2j+1)4N+2 |
| DST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(j+1)2N+1 |

In order to control MTS, separate two flags which enable MTS for inter and intra coded blocks, respectively, are signalled at SPS level. When MTS is enabled at SPS level, MTS is applied to the CUs with both width and height smaller than or equal to 32, and a CU level flag is signalled to indicate whether MTS is applied to the corresponding block or not. When the CU level flag is equal to 0, DCT-II is applied in the horizontal and vertical directions. For luma coding block with the CU level flag equal to 1, additional two flags are signalled to identify transforms to be used in the horizontal and vertical directions, respectively. In case of intra coded block with 1 or 2 non-zero coefficients, DST-VII is applied to both directions without signalling the additional two flags. On the other hand, for inter coded blocks, the additional two flags are signaled regardless of the number of non-zero coefficients. For chroma blocks, only DCT-II is applied as in HEVC. Transform and signaling mapping table is shown in Table 2.

Table – 2 Transform and signaling mapping table

| MTS_CU_flag | MTS_Hor_flag | MTS_Ver_flag | Intra coded block |  | Inter coded block |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |
| 0 | Not signalled | Not signalled | DCT-II | DCT-II | DCT-II | DCT-II |
| 1 | 0 | 0 | DST-VII | DST-VII | DCT-VIII | DCT-VIII |
|  | 1 | 0 | DCT-VIII | DST-VII | DST-VII | DCT-VIII |
|  | 0 | 1 | DST-VII | DCT-VIII | DCT-VIII | DST-VII |
|  | 1 | 1 | DCT-VIII | DCT-VIII | DST-VII | DST-VII |

As in HEVC, the residual of a block can be coded with transform skip mode. To avoid the redundancy of syntax coding, the transform skip flag is not signaled when the CU level MTS flag is not equal to zero.

Since the current MTS scheme can be applied for the blocks with both width and height smaller than or equal to 32, large rectangular blocks with only one of the directions smaller than or equal to 32 such as 32x64, 64x32, 128x32, 32x128, and so on, cannot exploit optimum transform selection among the DCT-VIII and DST-VII. In this contribution, an MTS scheme is proposed for large rectangular blocks without using additional transform kernels.

Proposed method

The proposed MTS is applied to the blocks with width or height smaller than or equal to 32. For the dimension smaller than or equal to 32, DCT-VIII or DST-VII is applied to the corresponding direction. For the dimension larger than 32, DCT-II is applied to the corresponding direction. Based on the following four conditions regarding the size of block, applicable transform kernels and signaling method are defined.

Condition I: ( width <= 32 ) && ( height <= 32 )

Condition II: ( width > 32 ) && ( height <= 32 )

Condition III: ( width <= 32 ) && ( height > 32 )

Condition IV: ( width > 32 ) && ( height > 32 )

For the CUs satisfying one of these three conditions, condition I, II and III, a CU level flag is signalled to indicate whether MTS is applied to the corresponding block. When both width and height are larger than 32, the CU level flag is not signalled and DCT-II is applied to both directions. Figure 1 illustrates applicable transform kernels according to width and height of the block.

Figure 1 – Applicable transform kernels depending on width and height of the block
(a) Condition I, (b) Condition II, (c) Condition III, (d) Condition IV

For the blocks satisfying the Condition I, the same method as in VTM 2 is applied [1].

For the blocks satisfying the Condition II, the selected transform kernels are signalled as shown in Table 3.

Table 3 – Transform kernel and signalling for the blocks satisfying the Condition II

| MTS_CU_flag | MTS_Hor_flag | MTS_Ver_flag | Intra coded block |  | Inter coded block |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |
| 0 | Not signalled | Not signalled | DCT-II | DCT-II | DCT-II | DCT-II |
| 1 | Not signalled | 0 | DCT-II | DST-VII | DCT-II | DCT-VIII |
|  | Not signalled | 1 | DCT-II | DCT-VIII | DCT-II | DST-VII |

For the blocks satisfying the Condition III, the selected transform kernels are signalled as shown in Table 4.

Table 4 – Transform kernel and signalling for the blocks satisfying the Condition III

| MTS_CU_flag | MTS_Hor_flag | MTS_Ver_flag | Intra coded block |  | Inter coded block |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal | Vertical | Horizontal | Vertical |
| 0 | Not signalled | Not Signalled | DCT-II | DCT-II | DCT-II | DCT-II |
| 1 | 0 | Not signalled | DST-VII | DCT-II | DCT-VIII | DCT-II |
|  | 1 | Not signalled | DCT-VIII | DCT-II | DST-VII | DCT-II |

Experimental results

The proposed method was implemented on top of BMS-2.0.1 and tested with VTM configuration under the common test conditions [2].

Test results with inter MTS off

Table 5 shows the experimental results of the proposed MTS compared with VTM-2.0.1 when MTS is disabled for inter coded blocks as in CTC. Since both the maximum BT size and the maximum TT size for I-slices are fixed as 32x32 in VTM 2, the block satisfying the Condition II or III does not occurred in the reference software. Therefore, intra blocks in I-slices are not influenced by the proposed method.

Table 5 – Comparison of the proposed method over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 + VTM cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.38% | -0.43% | 104% | 101% |
| Class A2 | -0.02% | -0.10% | -0.10% | 101% | 100% |
| Class B | -0.03% | -0.09% | -0.12% | 101% | 100% |
| Class C | 0.01% | 0.04% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.11% | -0.14% | 102% | 100% |
| Class D | 0.00% | 0.00% | -0.04% | 100% | 100% |
| Class F (optional) | 0.02% | -0.03% | 0.00% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | -0.40% | 101% | 101% |
| Class C | 0.03% | -0.04% | -0.04% | 100% | 100% |
| Class E | -0.12% | 0.23% | 0.17% | 100% | 100% |
| Overall | -0.02% | 0.03% | -0.14% | 101% | 100% |
| Class D | 0.04% | 0.36% | -0.71% | 100% | 100% |
| Class F (optional) | -0.17% | 0.40% | 0.41% | 101% | 99% |

Test results with inter MTS on

Table 6 shows the experimental results of the proposed method compared with VTM-2.0.1 when MTS is enabled for inter coded blocks.

Table 6 – Comparison of the proposed method over VTM-2.0.1 with inter MTS on

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 + VTM cfg + Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg + Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.28% | -0.32% | 108% | 101% |
| Class A2 | -0.07% | 0.02% | 0.08% | 104% | 101% |
| Class B | -0.05% | -0.10% | -0.11% | 104% | 101% |
| Class C | 0.00% | 0.04% | 0.11% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.07% | -0.05% | 104% | 100% |
| Class D | 0.00% | -0.05% | 0.00% | 102% | 99% |
| Class F (optional) | -0.03% | 0.03% | -0.01% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg + Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.13% | 0.33% | 105% | 100% |
| Class C | -0.05% | 0.20% | 0.26% | 102% | 100% |
| Class E | -0.04% | 0.40% | 0.20% | 105% | 100% |
| Overall | -0.05% | 0.11% | 0.27% | 104% | 100% |
| Class D | -0.06% | 0.38% | -0.19% | 101% | 101% |
| Class F (optional) | -0.16% | -0.24% | -0.07% | 104% | 99% |

Additional test results for I-slice

In order to test the proposed method for I-slices, both the maximum BT size and the maximum TT size for I-slices are set to 64x64. Table 7 shows the summarized results.

Table 7 Comparison of the proposed method over VTM-2.0.1 with both the maximum BT size and the maximum TT size for I-slices equal to 64x64

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 + VTM cfg + MAX_BT_TT_SIZE_64 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.24% | -0.25% | 103% | 101% |
| Class A2 | -0.10% | -0.10% | -0.11% | 102% | 100% |
| Class B | -0.08% | -0.03% | 0.00% | 102% | 99% |
| Class C | -0.01% | -0.07% | -0.01% | 100% | 99% |
| Class E | -0.06% | -0.14% | -0.05% | 102% | 99% |
| Overall | -0.09% | -0.11% | -0.07% | 102% | 100% |
| Class D | -0.01% | -0.05% | 0.01% | 100% | 100% |
| Class F (optional) | -0.01% | 0.01% | -0.04% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg + MAX_BT_TT_SIZE_64 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.57% | -0.40% | 103% | 100% |
| Class A2 | -0.05% | 0.01% | -0.03% | 101% | 99% |
| Class B | -0.06% | 0.00% | -0.13% | 101% | 100% |
| Class C | -0.02% | 0.02% | -0.24% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.11% | -0.19% | 101% | 100% |
| Class D | -0.04% | -0.11% | 0.02% | 100% | 100% |
| Class F (optional) | -0.02% | -0.19% | -0.12% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 + VTM cfg + MAX_BT_TT_SIZE_64 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.01% | 0.21% | 101% | 100% |
| Class C | -0.05% | -0.10% | 0.04% | 101% | 99% |
| Class E | 0.01% | 1.18% | -0.68% | 101% | 100% |
| Overall | -0.04% | 0.27% | -0.07% | 101% | 100% |
| Class D | 0.05% | 0.13% | -0.20% | 100% | 100% |
| Class F (optional) | 0.11% | -0.01% | -0.12% | 101% | 97% |

Conclusion

This contribution proposes an MTS scheme for large rectangular CUs. The proposed MTS can be applied to the blocks with width or height smaller than or equal to 32. For the direction smaller than or equal to 32, DCT-VIII or DST-VII is applied and for the direction larger than 32, DCT-II is applied. As a result, large rectangular blocks with either width or height smaller than or equal to 32 can use transform selection among the DCT-VIII and DST-VII in addition to DCT-II. Experimental results show that the overall BD-rates over BMS-2.0.1 with VTM configuration are 0.00% (AI), -0.04% (RA) and -0.02% (LD). Especially, regarding 4K resolution test sequences in class A1, 0.14% (RA) BD-rate gain is achieved. When MTS is enabled for inter coded blocks, experimental results show 0.00% (AI), -0.06% (RA) and -0.05% (LD) BD-rate compared to BMS-2.0.1 with VTM configuration. Experimental results on the proposed method with both the maximum BT size and the maximum TT size equal to 64 show -0.09% (AI), -0.07% (RA) and -0.04% (LD) BD-rate compared to BMS-2.0.1 with VTM configuration. It is recommended to adopt the proposed method in the next version of reference software.

References

[1] J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2),” JVET-K1002, Ljubljana, SI, July 2018

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, July 2018

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).