| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0199-v2 |
| --- | --- |

| Title: | CE3: Non-linear weighted intra prediction (tests 2.2.1 and 2.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Helle, Jonathan Pfaff, Michael Schäfer, Roman Rischke, Tobias Hinz, Philipp Merkle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-188 +49 30 31002-642 philipp.helle@hhi.fraunhofer.de, jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results of the core experiment tests CE3-2.2.1 and CE3-2.2.2. Test 2.2.1 tests “Non-linear weighted intra prediction” with an adapted MPM-list derivation process. Test 2.2.2 tests “Affine Linear Weighted Intra Prediction” which uses a simplified predictor architecture. Both tests are based on the VTM software version 2.0.1.

For test 2.2.1 BD-Rates of -2.46%, -1.96%, -1.94% (Y-, U- and V-component) are reported for the AI configuration and BD-Rates of -1.25%, -1.16%, -1.21% (Y-, U- and V-component) are reported for the RA configuration. For test 2.2.2 BD-Rates of -1.63%, -0.96%, -0.95% (Y-, U- and V-component) are reported for the AI configuration and BD-Rates of -0.90%, -0.80%, -0.85% (Y-, U- and V-component) are reported for the RA configuration.

# Test CE3-2.2.1

## The technique tested in test CE3-2.2.1 is identical to “Non-linear weighted intra prediction” described in JVET-K0196 [1], but additionally uses a mapping table to adapt the MPM-list (most probable mode list) derivation-process used for coding the conventional luma and chroma intra-prediction modes. The technique described in JVET-K0196 is briefly summarized in sections 1.1 – 1.3 and the adapted MPM-list derivation is described in section 1.4. Section 1.5 reports results of test CE3-2.2.1.

## Non-linear weighted intra prediction (from JVET-K0196)

To predict the samples of a rectangular block, Non-linear weighted intra prediction (NWIP) takes two lines of reconstructed neighbouring samples as depicted in Fig 1 as an input. To these samples, the prediction process applies an affine map using a matrix A1 and an offset-vector b1, then a rectifier function σ and then another affine map using a matrix A2,k and an offset vector b2,k, leading to a prediction pred in the frequency domain. The vector pred is constrained to a fixed sparsity pattern that depends only on the prediction mode k (or predmode)  but not on the input vector r. This means that for some frequency components, the vector pred is constrained to be zero in the corresponding component. For each such frequency component, the row of the matrix A2,k corresponding to that component consists only of zeros and no multiplications need to be carried out in the matrix vector product A2,k∙t1. As a last step, the predicted spatial domain samples are obtained by applying an inverse DCT.

Figure 1: Prediction of MxN samples as described in JVET-K0196

Here, k (or predmode) denotes the prediction mode which is signaled to the decoder in the following way. First, a sorted mode-list is derived by using a mode-predictor function that has a similar structure as the sample-predictor function depicted in Fig 1 and uses distinct matrices and bias vectors. Then, an index into this list is transmitted in the bitstream so the decoder can identify the correct prediction mode. The number of different prediction modes for a block of size MxN is 35 for maxM,N<32 and 11 otherwise.

## Parameters needed to generate the prediction signal

Table 1 shows the minimum, maximum and average number of 16bit values needed for predicting the luma samples in the frequency domain as described in section 1.1. Thus,. the table contains the number of values needed for the matrices A1, A2,k, and the vectors b1, b2,k for predicting a block of shape MxN. The values are calculated among all prediction modes, i.e. over k and are given as a number per luma sample. The values change with k since only the non-zero rows of A2,k and corresponding entries of b2,k are accounted for in Table 1. Note that the table is symmetric around the diagonal because of the transpose-symmetry of the predictors. Redundant values (M>N) are not given and marked with “-”.

In the same fashion as Table 1, Table 2 shows the number of multiplications needed for predicting a block of MxN luma samples in the frequency domain, i.e. the number of multiplications in the products A1r and A2,kt1 . The values are given as number of multiplications per luma sample.

Table 1 Minimum, maximum and average number of 16bit values needed for predicting blocks of MxN luma samples. The values are calculated over all modes k.

|  | Min |  |  |  | Max |  |  |  | Average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M\N | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 |
| 4 | 31.50 | 38.06 | 40.78 | 28.79 | 47.25 | 54.38 | 72.42 | 45.71 | 46.20 | 50.93 | 64.19 | 39.50 |
| 8 | - | 25.44 | 21.53 | 16.07 | - | 48.56 | 55.90 | 27.23 | - | 44.70 | 42.57 | 23.95 |
| 16 | - | - | 21.29 | 11.03 | - | - | 43.12 | 20.41 | - | - | 32.95 | 16.19 |
| 32 | - | - | - | 9.67 | - | - | - | 14.95 | - | - | - | 12.05 |

Table 2 Minimum, maximum and average number of multiplications with 16bit values per luma sample needed for predicting blocks of MxN luma samples. The values are calculated over all modes k.

|  | Min |  |  |  | max |  |  |  | Average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M\N | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 |
| 4 | 30.00 | 36.75 | 39.88 | 28.33 | 45.00 | 52.50 | 70.81 | 44.80 | 44.00 | 49.17 | 62.76 | 38.76 |
| 8 | - | 24.75 | 21.12 | 15.86 | - | 47.25 | 54.84 | 26.75 | - | 43.49 | 41.76 | 23.54 |
| 16 | - | - | 20.98 | 10.91 | - | - | 42.50 | 20.10 | - | - | 32.47 | 15.97 |
| 32 | - | - | - | 9.58 | - | - | - | 14.79 | - | - | - | 11.93 |

## MPM-list derivation for conventional luma and chroma intra-prediction modes

Each intra predicted luma block either uses an NWIP-mode or a conventional prediction mode. As the derivation of the MPM-lists used for coding the conventional luma and chroma prediction modes uses intra modes of neighbouring luma blocks (for luma MPM-list derivation) or the intra mode of the current luma block (for chroma MPM-list derivation), it must be defined how to treat luma blocks that use an NWIP-mode. The technique described in JVET-K0196 does this in the following way. For the luma MPM-list derivation, whenever a neighbouring luma block is encountered which uses an NWIP-mode, this block is treated as if it was not present or not using an intra-prediction mode. For the chroma MPM-list derivation, whenever the current luma block uses an NWIP-mode, the block is treated as if it was using the conventional planar intra-prediction mode.

## Adapted MPM-list derivation for conventional luma and chroma intra-prediction modes

The MPM-list derivation relies on neighbouring intra-prediction modes as clues to the current intra-prediction mode. As more and more blocks use one of the NWIP-modes, treating neighbouring intra-prediction modes as described above in 1.3 leads to a lack of these clues, which could affect entropy coding of the conventional luma or chroma intra prediction mode.

To mitigate this effect, in test CE3-2.2.1 the luma and chroma MPM-list derivation processes for the conventional intra-prediction modes use a fixed table map_nwip_to_angular, mapping an NWIP-mode predmodeNWIP to one of the conventional intra-prediction modes

predmodeAngular=map_nwip_to_angular[predmodeNWIP]
.

For the luma MPM-list derivation, whenever a neighbouring luma block is encountered which uses an NWIP-mode predmodeNWIP, this block is treated as if it was using the conventional intra-prediction mode predmodeAngular. For the chroma MPM-list derivation, whenever the current luma block uses an NWIP-mode, the same mapping is used to translate the NWIP-mode to a conventional intra prediction mode.

## Results of test CE3-2.2.1

Evaluation of the test was performed according to the common test conditions JVET-K1010 [2], for the All Intra (AI) and Random Access (RA) configurations with the VTM software version 2.0.1. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.33% | -2.53% | -2.63% | 273% | 122% |
| Class A2 | -1.99% | -1.61% | -1.53% | 256% | 123% |
| Class B | -2.13% | -1.64% | -1.60% | 267% | 125% |
| Class C | -1.92% | -1.54% | -1.50% | 251% | 132% |
| Class E | -3.37% | -2.83% | -2.83% | 273% | 132% |
| Overall | -2.46% | -1.96% | -1.94% | 264% | 127% |
| Class D | -2.00% | -1.51% | -1.70% | 238% | 136% |
| Class F (optional) | -2.46% | -2.30% | -2.20% | 257% | 126% |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.81% | -1.87% | -2.25% | 128% | 107% |
| Class A2 | -1.01% | -0.64% | -0.49% | 126% | 104% |
| Class B | -1.21% | -1.01% | -1.00% | 131% | 106% |
| Class C | -1.05% | -1.22% | -1.22% | 132% | 107% |
| Class E |  |  |  |  |  |
| Overall | -1.25% | -1.16% | -1.21% | 130% | 106% |
| Class D | -0.94% | -1.13% | -0.76% | 132% | 106% |
| Class F (optional) | -1.50% | -1.47% | -1.65% | 142% | 106% |

# Test CE3-2.2.2

## Affine Linear Intra Predictions

In CE3-2.2.2, new intra prediction modes in the sequel called ALWIP modes (where ALWIP stands for ‘Affine Linear Weighted Intra Prediction’) were tested which perform the following simplifications in comparison to CE3-2.2.1:

- The predictors are affine linear, i.e., the prediction signal is obtained by computing a matrix vector product and adding an offset.

- The input for the predictors is reduced: For an MxN block, only the line of N reference samples above the block and the line of M reference samples to the left of the block are used as an input for the prediction.

- For large block sizes, only half of these reference samples are used as input of the prediction by applying a one-dimensional DCT to the input and keeping only the lower frequencies.

- The MPM-list used in signaling the ALWIP mode of a given block is generated by using solely the intra modes of the above block as well as of the left block, if these modes are available.

## Generation of the prediction signal

For predicting the samples of a rectangular MxN-block, Affine-linear weighted intra prediction (ALWIP) takes one line of reconstructed neighbouring samples as depicted in Figure 2 as an input. If M>8, a one dimensional DCT is applied to the M samples left to the block and only the M/2 lowest frequencies are kept. Similarly, if N>8, a one dimensional DCT is applied to the N samples above the block and only the N/2 lowest frequencies are kept. This results in a vector of new input samples that is of size size(M)+size(N), where size(S):=S if S≤8 and size(S):=S/2, else.
The prediction process applies an affine map using a matrix Ak to these new input samples, and an offset-vector bk, leading to a prediction p in the frequency domain. The vector p is constrained to a fixed sparsity pattern that depends only on the prediction mode  k but not on the input vector r. This means that for some frequency components, the vector p is constrained to be zero in the corresponding component. For each such frequency component, the row of the matrix Ak corresponding to that component consists only of zeros and no multiplications need to be carried out in the matrix vector product Ak∙t. As a last step, the predicted spatial domain samples are obtained by applying an inverse DCT.

Figure 2: Description of MxN samples by ALWIP.

## Signalization of the prediction mode

For blocks of size MxN, 35 ALWIP modes are proposed if maxM,N<32 and 11 ALWIP modes are proposed if maxM,N=32. For each Coding Unit (CU) in intra mode, a flag indicating if an ALWIP mode is to be applied on the corresponding Prediction Unit (PU) or not is sent in the bitstream. If an ALWIP mode is to be applied, the index predmode of the ALWIP mode is signaled using an MPM-list with 3 MPMS. 	
Here, the derivation of the MPMs is performed using the intra-modes of the above and the left PU as follows. For each block-shape, there is a mapping map_angular_to_alwip that assigns to each conventional intra prediction mode predmodeAngular an ALWIP mode

predmodeALWIP=map_angular_to_alwip[predmodeAngular].

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if the above PU has the same shape as the current PU and if ALWIP is applied on the above PU with ALWIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwip[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode

modeALWIPleft .

Finally, a fixed default list is provided that contains three distinct ALWIP modes. Out of this default list and the modes  modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.	
For the conventional intra prediction modes, the generation of the MPM list is adapted in exactly the same way as described in section 1.4. above. Also, the chroma-MPM list derivation is handled in exactly the same way as described in section 1.4 above.

## Parameters needed to generate the prediction signal

The total number of parameters needed for all prediction modes for all block-sizes is given by the total amount of all entries of the matrices Ak and the vectors  bk . In summary, these are 136 492 parameters. Since each entry of these matrices is a 16 bit value, it requires 2184 kbit (0.273 Mbyte) to store all these parameters.
It is pointed out that in a hardware design of a decoder, these parameters could be stored in an external memory exactly as one stores the reference pictures needed for motion compensated prediction (where for example one picture with 3840x2160 luma samples, 4:2:0 chroma sub-sampling and 10bit per sample requires at least 124416 kbit, i.e. 15.6 Mbyte). 	
In the same way as for motion compensated prediction on inter blocks, it is proposed that whenever a proposed intra mode is applied, the coefficients needed for the given mode (i.e. the entries of exactly one matrix Ak and one vector bk) are loaded from the external memory.

Table 1 shows the minimum, maximum and average numbers of 16bit values per pixel needed for predicting the picture samples with the modified architecture as tested in CE3-2.2.2. The table contains the number of values needed for the matrix Ak and the vector bk used for predicting blocks of different shapes. 	
pred

Table 1. Minimum, maximum and average number of 16bit values per pixel needed for predicting a block of MxN luma samples. The values are calculated over all modes k.

|  | Min |  |  |  | Max |  |  |  | Average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M\N | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 |
| 4 | 2.25 | 8.13 | 1.83 | 4.76 | 9 | 13 | 9.34 | 8.53 | 8.56 | 11.51 | 6.99 | 7.34 |
| 8 | - | 2.13 | 1.73 | 1.56 | - | 13.28 | 8.90 | 7.13 | - | 10.73 | 6.32 | 5.29 |
| 16 | - | - | 1.13 | 0.63 | - | - | 4.65 | 4.25 | - | - | 3.00 | 2.93 |
| 32 | - | - | - | 0.84 | - | - | - | 2.77 | - | - | - | 1.51 |

Table 2 shows the minimum, maximum and average numbers of multiplications per pixel needed to generate the prediction signal pred in the frequency domain. Here, each multiplication multiplies a reference sample value with a 16 bit entry of one of the matrices Ak. It can be seen that never more than 12,5 multiplications are needed per sample (case of 8x8 blocks). On the other hand, it is remarked that in the Adaptive loop filter process that was adopted at the last meeting (see [4],  8.5.3), 13 multiplications per sample, each multiplication multiplying a filter coefficient with a sample value are carried out (see [4], 8.5.3.4). Here, the filter coefficients vary on a 4x4 basis within a set of filter coefficients that in turn is not fixed but may change from frame to frame where the filter coefficients are signaled in the bitstream. It is pointed out that in Table 2, the (partial) inverse DCT that needs to be applied to the prediction signal in the frequency domain is not counted for the number of multiplications.  Also, the one-dimensional (partial) forward DCT that is applied to a reference sample line left or above the block whenever the latter consists of more than 8 samples are not counted.

Table 2. Minimum, maximum and average number of multiplications with 16bit values per luma sample needed for predicting a block of MxN luma samples. The values are calculated over all modes k.

|  | Min |  |  |  | Max |  |  |  | Average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M\N | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 | 4 | 8 | 16 | 32 |
| 4 | 2 | 7.5 | 1.69 | 4.53 | 8 | 12 | 8.62 | 8.12 | 7.61 | 10.63 | 6.45 | 6.99 |
| 8 | - | 2 | 1.63 | 1.5 | - | 12.5 | 8.38 | 6.84 | - | 10.10 | 5.95 | 5.07 |
| 16 | - | - | 1.06 | 0.61 | - | - | 4.38 | 4.08 | - | - | 2.83 | 2.81 |
| 32 | - | - | - | 0.81 | - | - | - | 2.69 | - | - | - | 1.47 |

Table 3 shows the worst case memory bandwidth consumption of ALWIP in comparison to bi-predictive motion compensation. Presentation of the values here follows the style of JVET-K0480 [5]. To calculate the number of values loaded per luma sample for bi-predictive motion compensation (bi-pred. MC), the following expression is used:

N+ftMC-1*M+ftMC-1*2NM

Where N, M are the dimensions of the predicted block in luma samples and ftMC=8 is the number of filter taps of the MC-filter. These values are shown in the third row of Table 3 (‘bi-pred. MC number of samples’).

The fourth row of Table 3 (‘Worst case ALWIP mode number of coefficients’) shows the number of coefficients loaded per luma sample when the block is predicted with the worst case ALWIP-mode, i.e. the one with the most number of coefficients that must be loaded. Thus, these values are the same as the ‘Max’ values in Table 1.

The last two rows of Table 3 show the same values as the third and fourth row, but relative to the 8x8 bi-pred. MC case. It is pointed out that the comparison made here makes the assumption that the memory bandwidth cost for a luma sample and the memory bandwidth cost for an ALWIP-coefficient are equal.

Table 3. Worst case memory bandwidth consumption per luma sample and with respect to 8x8 bi-predictive motion compensation.

## Results of test CE3-2.2.2

Evaluation of the test was performed according to the common test conditions JVET-K1010 [2], for the intra-only (AI) and random-access (RA) configurations with the VTM software version 2.0.1. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -2.56% | -1.66% | -1.75% | 283% | 114% |
| Class A2 | -1.37% | -0.79% | -0.75% | 267% | 111% |
| Class B | -1.40% | -0.72% | -0.74% | 278% | 111% |
| Class C | -1.12% | -0.64% | -0.54% | 261% | 115% |
| Class E | -2.02% | -1.27% | -1.22% | 287% | 114% |
| Overall | -1.63% | -0.96% | -0.95% | 275% | 113% |
| Class D | -1.25% | -0.59% | -0.91% | 247% | 117% |
| Class F (optional) | -2.06% | -1.77% | -1.76% | 266% | 113% |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.49% | -1.52% | -1.77% | 135% | 107% |
| Class A2 | -0.72% | -0.41% | -0.27% | 132% | 102% |
| Class B | -0.85% | -0.72% | -0.76% | 138% | 106% |
| Class C | -0.65% | -0.66% | -0.71% | 139% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.90% | -0.80% | -0.85% | 137% | 105% |
| Class D | -0.65% | -0.89% | -0.36% | 138% | 109% |
| Class F (optional) | -1.39% | -1.46% | -1.35% | 148% | 106% |

# Cross check

Cross-checks of both tests CE3-2.2.1 and CE3-2.2.2 were performed by Huawei and Technicolor and the cross-check reports are included in JVET-L0023 [3].

# References

- P. Helle et al., “CE3-related: Non-linear weighted intra prediction (cross-check report in JVET-K0262)”, JVET-K0196, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Ljubljana, SI, July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary Report on Intra Prediction and Mode Coding”, JVET-L0023, Macao, CN, September 2018.

- B. Bross, “Versatile Video Coding, Draft 2”, JVET-K0521-v2, Ljubljana, SI, July 2018

- M. Zhou, B. Heng “Non-CE9: A computational complexity analysis for DMVR”, JVET-K0480-v1, Ljubljana, SI, July 2018

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).