| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0208-v1 |
| --- | --- |

| Title: | CE10-related: Multiple prediction unit shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier Fabrice Le Léannec Philippe Bordes | Tel: Email: | +33(0)299273198 tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution provides test results of multiple prediction unit shapes, this test is built on top of CE10.3.1.b. A CU can be split using 2 prediction units, in either diagonal or inverse diagonal direction or horizontal or vertical direction with multiple positions. Each prediction unit in the CU has its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list. Multiple shapes partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining the two prediction units together. In this contribution, the triangular prediction unit mode is only applied to a CU which block size is larger than or equal to 8×8, and its coding prediction mode is either skip or merge mode. It is reported that as compared to VTM-2.0.1, 0.78% B-D rate saving with 158% encoding time and 105% decoding time is achieved in random access configuration. 1.57% B-D rate saving with 164% encoding time and 94% decoding time is achieved in low delay B configuration.

# Introduction

The concept of triangular prediction unit mode has been introduced [1]. A CU is split into two triangular prediction units, in either diagonal or inverse diagonal direction. Each triangular prediction unit in the CU is inter-predicted using its own uni-prediction motion vector and reference frame index. An adaptive weighting process is performed to the diagonal edge after predicting the triangular prediction units. Then, the transform and quantization process are applied to the whole CU. It is noted that this mode is only applied to skip and merge modes.

# Proposed techniques

Weighting process with increased precision

In [1], each triangular prediction unit is obtained after motion compensation, shifting samples to the internal bitdepth precision and clipping, as shown in Figure 1. The adapted weighting process is then applied to the diagonal edge between the two triangular prediction units.

Figure 1: Weighting process in CE10.3.1.b

In this contribution, it is proposed to use the same process as bi-prediction, the weighting process is done with higher internal precision, then shifting to the internal bitdepth and clipping are done only on the final predictor, as shown in Figure 2. It is to be noted that the weights used are the same as in [1].

Figure 2: Proposed weighting process

Separation of shape and predictor index coding

In [1], an index between 0 and 39 is coded, from this index, A look-up table is used for deriving the splitting direction and motion vectors from the index.

In this contribution, one or more flags are coded to indicate the shape of the prediction units, then an index between 0 and 19 is coded to indicate the motion predictor used using a look-up table.

Multiple shapes

On top of triangular prediction units, 10 more shapes are used as shown in Figure 3.

Figure 3: Proposed multiple shapes

# Simulation results

The following experiments were conducted using the BMS-2.0.1 software with the VTM configuration and the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations.

Table 1: Simulation results of the proposed technique

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1(vtm.cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.56% | -0.96% | 141% | 103% |
| Class A2 | -0.72% | -0.98% | -1.09% | 164% | 101% |
| Class B | -0.60% | -0.65% | -0.78% | 160% | 108% |
| Class C | -1.26% | -1.46% | -1.60% | 165% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -0.91% | -1.10% | 158% | 105% |
| Class D | -0.73% | -0.93% | -0.94% | 175% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1(vtm.cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.14% | -0.88% | -0.98% | 153% | 88% |
| Class C | -1.66% | -2.12% | -2.08% | 171% | 106% |
| Class E | -2.16% | -0.40% | 1.07% | 172% | 92% |
| Overall | -1.57% | -1.18% | -0.83% | 164% | 94% |
| Class D | -1.48% | -1.25% | -1.86% | 175% | 110% |

# References

- R.-L. Liao and C. S. Lim, “CE10: Triangular prediction unit mode”, JVET-K0144, Ljubljana, July 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).