| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0214 v3 |
| --- | --- |

| Title: | CE4 related: Motion predictor pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, F. Galpin, T. Poirier | Email: | antoine.robert@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes a motion predictor pruning process to be applied in all coding modes to avoid redundancy in the lists of predictors. The proposed process has been implemented in the JVET BMS-2.0.1. The reported simulation results in VTM configuration show an average luma BD-rate gain of -0.08% in RA configuration with 4% encoder and 2% decoder complexity decrease, and an average luma BD-rate gain of -0.21% in LDB configuration with 1% encoder and 1% decoder complexity decrease.

# Introduction

In all the current motion predictor lists’ constructions (AMVP, Merge, Affine AMVP and Affine Merge), it is possible to encounter the same predictor several times. This reduces the achievable performances.

This contribution proposes a motion predictor pruning process that allows avoiding such redundancy in the lists of predictors. It is also possible to clean and speed-up these predictor lists’ constructions.

# Description

In all coding modes (AMVP, Merge, Affine AMVP and Affine Merge), motion predictor lists are constructed. But in the current reference software, some predictor redundancy can be observed.

To avoid such redundancies, this contribution proposes a motion predictor pruning process which consists, for each new motion predictor candidate reached to be added into the current list, in:

- Performing all necessary operations to the predictor candidate as clipping, rounding, … operations,

- Comparing the obtained predictor candidate to the already selected ones in the current list,

- Adding it into the current list only if not already present.

As an example, in AMVP, a motion predictor is a motion vector dedicated to a particular reference picture. The construction of a list of 2 predictors is as follows where bold parts correspond to the proposed pruning process along with some speed-ups (in italic):

- Get a spatial predictor from left position

- Get a different spatial predictor from above position

- Set ¼-pel precision for spatial predictor(s)

- AMVR rounding of spatial predictor(s)

- Compare both spatial predictors, if equal then removes one

- If temporal is allowed at slice level and not enough predictors

  - Get a temporal predictor

  - Set ¼-pel precision for temporal predictor

  - AMVR rounding of temporal predictor

  - Compare with already found predictor(s), if temporal equals one then removes it

- Fill list with null motion vector(s) at ¼-pel precision

- Set ¼-pel precision for spatial predictor(s)

- AMVR rounding of spatial predictor(s)

In Merge mode, a motion predictor is one or two motion vector(s) with its associated reference picture. The construction of a list of 6 predictors is as follows where bold parts correspond to the proposed pruning process along with some speed-ups (in italic):

- Get a spatial predictor from left position

  - Early terminations

- Get a different spatial predictor from above position

  - Early terminations

- Get a different spatial predictor from above-right position

  - Early terminations

- Get a different spatial predictor from bottom-left position

  - Early terminations

- If ATMVP is allowed at SPS level and temporal is allowed at slice level

  - Get ATMVP predictor

    - Early terminations

- Get a different spatial predictor from above-left position

  - Early terminations

- If temporal is allowed at slice level

  - Get a temporal predictor

  - Compare with already found predictors except ATMVP one,

  - Add temporal predictor into the current list if not already present

    - Early terminations

- Create combined bidirectional predictors

  - For each tested pairs of already selected predictors, pick L0 predictor from first candidate and L1 predictor from second one

  - If L0 and L1 parts are different

    - Compare with already found predictors except ATMVP one

    - Add it into the current list if not already present

      - Early terminations

- Fill list with L0 and L1 null motion vectors for each reference picture

  - Early terminations

- Fill list with L0 and L1 null motion vectors for first reference picture

  - Early terminations

The same kind of motion predictor pruning process is also performed in the Affine modes (Affine AMVP and Affine Merge) where a motion predictor is then:

- In Affine AMVP, an affine motion model dedicated to a particular reference picture,

- In Affine Merge, one or two affine motion model(s) with its associated reference picture,

Where an affine model is described by two or three motion vectors for 4- or 6-parameters affine models respectively.

Furthermore, the current implementation of this contribution is also designed to consider some JEM tools, if activated, such as LIC, FRUC, …

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-K1010 [1]. BMS-2.0.1 reference software is used with VTM configuration files.

Table 1 and Table 2 show the simulation results of the proposed motion predictor pruning in random access and low delay B configurations. In both configurations, the proposed pruning process allows improving the BD-rate performances for all sequence classes with a reduced complexity.

Table 1: Simulation results of the proposed motion predictor pruning in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.03% | -0.11% | 97% | 99% |
| Class A2 | -0.05% | -0.06% | -0.16% | 92% | 97% |
| Class B | -0.10% | -0.08% | -0.07% | 98% | 101% |
| Class C | -0.07% | 0.04% | -0.10% | 96% | 93% |
| Overall | -0.08% | -0.02% | -0.11% | 96% | 98% |
| Class D | -0.05% | -0.25% | -0.04% | 96% | 89% |

Table 2: Simulation results of the proposed motion predictor pruning in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.15% | -0.12% | -0.16% | 99% | 100% |
| Class C | -0.19% | -0.25% | -0.26% | 99% | 99% |
| Class E | -0.35% | 0.01% | -0.44% | 98% | 99% |
| Overall | -0.21% | -0.13% | -0.26% | 99% | 99% |
| Class D | -0.11% | 0.01% | -0.75% | 100% | 101% |

We want to kindly thank Huawei for their cross-check.

# Conclusion

This contribution proposes a motion predictor pruning process with an associated cleaning. In the current motion predictor lists’ constructions, it is possible to encounter some predictors several times. The proposed motion predictor pruning allows avoiding such redundancy. Simulation results reportedly show that the proposed method can bring an average -0.08% luma BD-rate gain for RA and -0.21% luma BD-rate gain for LDB using the VTM configuration. Simulations also show an associated reduction of the complexity of 4% encoder and 2% decoder for RA, and 1% encoder and 1% decoder for LDB. It is suggested that the motion predictor pruning proposed of this contribution should be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1010, 11th Meeting, Ljubljana, SI, July 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).