| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0216 |
| --- | --- |

| Title: | Non-CE4: Parallel Merge Estimation for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik Han Gao Biao Wang Anand Meher Kotra Jianle Chen | Tel: Email: | semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

A method for parallel estimation of merge mode of neighboring blocks is proposed, which takes into account the binary tree and ternary tree partitions of VVC. The proposed method allows estimation of the merge mode and affine merge mode of two coding blocks if any part of the two blocks are in the same NxN grid, N being in the range of minimum coding block size and maximum coding block size. Simulation results for N equal to 8, 16, 32 and 64 are presented in the contribution document.

# Introduction

Parallel merge estimation tool was part of HEVC and in the current VTM software an implementation similar to that of HEVC is present. On the other hand it is currently not in the specification text. This input contribution aims at implementation of parallel merge estimation into VVC, taking into account the new binary tree and ternary tree partitions.

# Problem Statement

In HEVC, if 2 coding blocks are in the same merge estimation region (MER), the first one is set unavailable for prediction during the merge list construction process of the second block. In VTM, due to binary and ternary splits 2 problems are identified that prevent application of the HEVC parallel merge estimation rule directly to VVC. The problems are:

## A block can extend over more than one MER and cover each MER only partially

This problem is exemplified in the following figure.

Figure 1

Block A extends over 2 MERs and it does not cover both MERs completely, which was not the cases in HEVC. The left and bottom-left merge candidate of block B are both inside the block A. In this kind of partition scenarios block A should be marked either available or unavailable for both candidate positions, there shouldn’t be different decisions for the left and the bottom-left merge candidates of block B.

## The block coding order does not necessarily follow z-scan order

This problem is exemplified in the following figure.

Figure 2

In the figure above the coding order of blocks are ABCDE. Blocks A and C are inside the same MER, similarly the blocks B and D are in the same MER. However disabling MV prediction between (marking unavailable for merge) A and C is not enough to enable parallel merge estimation. Block B, which is coded between A and C in coding order, can predict from A and the block C can predict from B. As a result the encoding of block C still needs to wait block A, which means A and C cannot be processed in parallel.

# Proposed Method

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if it’s bottom right coordinate falls in the extended merge estimation region of the current block”. The extended MER is depicted in the following figure:

Figure 3

Assume that the bottom right coordinate a neighbor block is given by (XNb, YNb) and the top-left coordinate of the current block is given by (XCt, YCt). Moreover assume that the binary logarithm of MER size is given by N. The neighbor block is marked as unavailable for prediction if;

(XNb >> N is equal to XCt >> N) AND (YNb >> N smaller than or equal to YCt >> N)

OR

(YNb >> N is equal to YCt >> N) AND (XNb >> N smaller than or equal to XCt >> N)

Please note that the proposal achieves capability to parallelize 2 blocks if any part of them are inside the same MER. The blocks that can be estimated in parallel in Figure 1 and 2 are as follows:

- Figure 1: Blocks A&B, A&D, C&E and D&F can be processed in parallel (Merge lists can be constructed without dependency to each other).

- Figure 2: Blocks A&C and B&D can be processed in parallel.

## Application of the proposal to affine merge mode

The proposal is applied also to affine merge mode, which means that within a MER, coding blocks that are predicted by merge mode or affine merge mode can be estimated in parallel.

## Application of the proposal to neighbor blocks with sub-pu based inter prediction

According to the proposal, if a neighbor block is predicted with affine or ATMVP, the bottom right coordinate of the coding block is used (instead of the sub-PUs that result from further partitioning of the CB) to check neighbor block availability. In other words the whole CU is considered as available or unavailable by the current block, not the individual sub-PUs.

## Application of the proposal to history based and long distance merge candidates

It is noted that if the proposed algorithm applies to history based and long distance merge candidates without any change. The bottom right coordinate of candidate in question is used in the proposed algorithm just like spatially adjacent candidates.

# Code Changes

The function that is used to set a neighboring block unavailable for merge is changed as follows:

bool PU::isDiffMER(const PredictionUnit &pu1, const PredictionUnit &pu2)

{

const unsigned xN = pu1.lumaPos().x;

const unsigned yN = pu1.lumaPos().y;

const unsigned xP = pu2.cu->lumaPos().x + pu2.cu->lwidth() - 1;

const unsigned yP = pu2.cu->lumaPos().y + pu2.cu->lheight() - 1;

unsigned plevel = pu1.cs->pps->getLog2ParallelMergeLevelMinus2() + 2;

if ((xN >> plevel) == (xP >> plevel) && (yN >> plevel) <= (yP >> plevel))

{

return false;

}

if ((yN >> plevel) == (yP >> plevel) && (xN >> plevel) <= (xP >> plevel))

{

return false;

}

return true;

}

The complete code is included in the contribution for reference.

# Spec Changes

The proposed changes in the specification is provided as attachment.

# Simulation Results

Below are the experimental results of the proposal with MER sizes 8x8, 16x16, 32x32 and 64x64. In the simulations the anchor is VTM2.0.1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | 0.03% | 101% | 103% |
| Class A2 | 0.19% | 0.23% | 0.18% | 101% | 102% |
| Class B | 0.16% | 0.28% | 0.19% | 101% | 104% |
| Class C | 0.31% | 0.46% | 0.37% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.29% | 0.21% | 101% | 104% |
| Class D | 0.34% | 0.42% | 0.37% | 101% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.23% | 0.25% | 100% | 105% |
| Class C | 0.33% | -0.02% | 0.10% | 101% | 104% |
| Class E | -0.13% | 0.44% | -0.35% | 101% | 109% |
| Overall | 0.13% | 0.20% | 0.05% | 101% | 106% |
| Class D | 0.31% | 0.66% | 0.22% | 101% | 106% |
| Class F (optional) | 0.61% | 0.95% | 0.94% | 101% | 108% |

Table 1: Proposal with 8x8 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.30% | 0.32% | 102% | 102% |
| Class A2 | 0.55% | 0.88% | 0.93% | 103% | 102% |
| Class B | 0.56% | 0.63% | 0.55% | 103% | 104% |
| Class C | 0.91% | 1.06% | 1.08% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.57% | 0.73% | 0.72% | 103% | 103% |
| Class D | 0.91% | 1.03% | 0.97% | 103% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.47% | 0.46% | 0.18% | 101% | 104% |
| Class C | 0.81% | 0.63% | 0.78% | 103% | 104% |
| Class E | -0.10% | 0.49% | 0.15% | 101% | 109% |
| Overall | 0.44% | 0.52% | 0.37% | 102% | 105% |
| Class D | 0.88% | 1.37% | 0.82% | 103% | 105% |
| Class F (optional) | 1.41% | 1.86% | 1.80% | 103% | 109% |

Table 2: Proposal with 16x16 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | 0.75% | 0.75% | 103% | 102% |
| Class A2 | 1.23% | 1.84% | 1.92% | 106% | 101% |
| Class B | 1.28% | 1.35% | 1.35% | 105% | 103% |
| Class C | 1.80% | 2.16% | 2.22% | 107% | 104% |
| Class E |  |  |  |  |  |
| Overall | 1.25% | 1.54% | 1.57% | 105% | 103% |
| Class D | 1.70% | 1.87% | 2.15% | 107% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.17% | 0.94% | 0.78% | 104% | 104% |
| Class C | 1.73% | 1.64% | 1.92% | 105% | 102% |
| Class E | 0.18% | 0.15% | 0.20% | 101% | 108% |
| Overall | 1.11% | 0.98% | 1.02% | 103% | 104% |
| Class D | 1.77% | 1.86% | 1.71% | 106% | 105% |
| Class F (optional) | 1.90% | 2.33% | 2.25% | 104% | 108% |

Table 3: Proposal with 32x32 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.11% | 1.38% | 1.44% | 104% | 102% |
| Class A2 | 2.11% | 2.96% | 3.18% | 108% | 101% |
| Class B | 2.31% | 2.35% | 2.33% | 108% | 103% |
| Class C | 2.78% | 3.27% | 3.48% | 109% | 103% |
| Class E |  |  |  |  |  |
| Overall | 2.15% | 2.52% | 2.63% | 108% | 103% |
| Class D | 2.55% | 2.92% | 3.12% | 109% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.16% | 1.82% | 1.78% | 106% | 104% |
| Class C | 2.66% | 2.46% | 2.99% | 107% | 103% |
| Class E | 0.76% | 0.52% | 0.56% | 103% | 109% |
| Overall | 1.98% | 1.71% | 1.88% | 106% | 105% |
| Class D | 2.76% | 3.41% | 2.78% | 108% | 104% |
| Class F (optional) | 2.63% | 3.32% | 3.38% | 106% | 109% |

Table 4: Proposal with 64x64 MER

# Conclusion

A method for estimation of merge mode of neighboring blocks in parallel is proposed, which takes into account the binary tree and ternary tree partitions of VVC. It is asserted that the increased picture resolution necessitates the usage of block based parallelization tools such as parallel merge estimation. The contribution is proposed to be included in the VVC.

# Reference

[1] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, August 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).