| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0218 |
| --- | --- |

| Title: | Syntax and Semantics Changes of overriding QT/BT/TT Split Constraint Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Jianle Chen Biao Wang Anand Meher Kotra | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Syntax and semantic changes regarding to JVET-K1001-v6

…

#### 7.3.2.1 Sequence parameter set RBSP syntax

[Ed. (BB): Preliminary basic SPS, subject to further study and pending further specification development.]

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| partition_constrains_override_enabled_flag | ue(v) |
| sps_log2_min_qt_size_intra_slices_minus2 | ue(v) |
| sps_log2_min_qt_size_inter_slices_minus2 | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slices | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_log2_diff_ctu_max_bt_size_intra_slices | ue(v) |
| sps_log2_diff_ctu_max_bt_size_intra_slices | ue(v) |
| sps_log2_diff_ctu_max_tt_size_intra_slices | ue(v) |
| sps_log2_diff_ctu_max_tt_size_inter_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

…

#### 7.4.3.1 Sequence parameter set RBSP semantics

…

partition_constrains_override_enabled_flag equal to 1 specifies the presence of partition_constrains_override_flag in the slice headers for slices referring to the SPS. partition_constrains_override_enabled_flag equal to 0 specifies the absence of partition_constrains_override_flag in the slice headers for slices referring to the SPS.

sps_log2_min_qt_size_intra_slices_minus2 plus 2 specifies the default minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I) referring to the SPS, unless the default minimum luma sizes of a leaf block resulting from quadtree splitting of a CTU are overridden by the  minimum luma sizes of a leaf block resulting from quadtree splitting of a CTU present in the slice header of the slices referring to the SPS . The value of sps_log2_min_qt_size_intra_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeIntraY = log2_min_qt_size_intra_slices_minus2 + 2	(7-22)

sps_log2_min_qt_size_inter_slices_minus2 plus 2 specifies the default minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS, unless the default minimum luma sizes of a leaf block resulting from quadtree splitting of a CTU are overridden by the  minimum luma sizes of a leaf block resulting from quadtree splitting of a CTU present in the slice header of the slices referring to the SPS. The value of sps_log2_min_qt_size_inter_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeInterY = log2_min_qt_size_inter_slices_minus2 + 2	(7-23)

sps_max_mtt_hierarchy_depth_inter_slices specifies the default maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS, unless the default maximum hierarchy depthes for coding units resulting from multi-type tree splitting of a quadtree leaf are overridden by the maximum hierarchy depthes for coding units resulting from multi-type tree splitting of a quadtree leaf present in the slice header of the slices referring to the SPS. The value of sps_max_mtt_hierarchy_depth_inter_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

sps_max_mtt_hierarchy_depth_intra_slices specifies the default maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 2 (I) referring to the SPS, unless the default maximum hierarchy depthes for coding units resulting from multi-type tree splitting of a quadtree leaf are overridden by the maximum hierarchy depthes for coding units resulting from multi-type tree splitting of a quadtree leaf present in the slice header of the slices referring to the SPS. The value of sps_max_mtt_hierarchy_depth_intra_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

sps_log2_diff_ctu_max_bt_size_intra_slices specifies the default difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split in slices with slice_type equal to 2 (I) referring to the SPS, unless the default differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split are overridden by the differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

sps_log2_diff_ctu_max_bt_size_inter_slices specifies the default difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS, unless the default differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split are overridden by the differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

sps_log2_diff_ctu_max_tt_size_intra_slices specifies the default difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split in slices with slice_type equal to 2 (I) referring to the SPS, unless the default differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split are overridden by the differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_tt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

sps_log2_diff_ctu_max_tt_size_inter_slices specifies the default difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS, unless the default differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split are overridden by the differences between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split present in the slice header of the slices referring to the SPS. The value of sps_log2_diff_ctu_max_tt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

…

### 7.3.3 Slice header syntax

[Ed. (BB): Preliminary basic slice header, subject to further study and pending further specification development.]

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( partition_constrains_override_enabled_flag ) { |  |
| partition_constrains_override_flag | ue(v) |
| if ( partition_constrains_override_flag ) { |  |
| log2_min_qt_size_minus2 | ue(v) |
| max_mtt_hierarchy_depth | ue(v) |
| log2_diff_ctu_max_bt_size | ue(v) |
| log2_diff_ctu_max_tt_size | ue(v) |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

…

### 7.4.4 Slice header semantics

…

partition_constrains_override_flag equal to 1 specifies that partition constrains parameters are present in the slice header. partition_constrains_override_flag equal to 0 specifies that partition constrains parameters are not present in the slice header. When not present, the value of partition_constrains_override_flag is inferred to be equal to 0.

log2_min_qt_size_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU fur the current slice. The value of log2_min_qt_size_inter_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive. When not present, the values of log2_min_qt_size_minus2 is inferred to be equal to sps_log2_min_qt_size_intra_slices_minus2 if slice_type equal to 2 (I) and inferred to be equal to sps_log2_min_qt_size_inter_slices_minus2 if slice_type equal to 0 (B) or 1 (P).

max_mtt_hierarchy_depth specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf for the current slice. The value of max_mtt_hierarchy_depth_intra_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive. When not present, the values of max_mtt_hierarchy_depth is inferred to be equal to sps_max_mtt_hierarchy_depth_intra_slices with slice_type equal to 2 (I) and inferred to be equal to sps_max_mtt_hierarchy_depth_inter_slices with slice_type equal to 0 (B) or 1 (P).

log2_diff_ctu_max_bt_size specifies the difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split for the current slice. The value of log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the values of log2_diff_ctu_max_bt_size is inferred to be equal to sps_log2_diff_ctu_max_bt_size_intra_slices with slice_type equal to 2 (I) and inferred to be equal to sps_log2_diff_ctu_max_bt_size_inter_slices with slice_type equal to 0 (B) or 1 (P).

log2_diff_ctu_max_tt_size specifies the difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a ternary split for the current slice. The value of log2_diff_ctu_max_tt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the values of log2_diff_ctu_max_tt_size is inferred to be equal to sps_log2_diff_ctu_max_tt_size_intra_slices with slice_type equal to 2 (I) and inferred to be equal to sps_log2_diff_ctu_max_tt_size_inter_slices with slice_type equal to 0 (B) or 1 (P).

…

The variables MinQtLog2SizeY, MaxBtLog2SizeY, MinBtLog2SizeY, MaxTtLog2SizeY, MinTtLog2SizeY, MaxBtSizeY, MinBtSizeY, MaxTtSizeY, MinTtSizeY and MaxMttDepth are derived as follows:

MinQtLog2SizeY  =  ( slice_type  = =  I )  ?  MinQtLog2SizeIntraY  :  MinQtLog2SizeInterY	(7-26)

MinQtLog2SizeY  =  log2_min_qt_size_minus2 + 2	(7-26)

MaxBtLog2SizeY  =  CtbLog2SizeY − log2_diff_ctu_max_bt_size	(7-27)

MinBtLog2SizeY  =  MinCbLog2SizeY		(7-28)

MaxTtLog2SizeY  =  ( slice_type  = =  I )  ?  5  :  6	(7-29)

MinTtLog2SizeY  =  MinCbLog2SizeY		(7-30)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-31)

MaxBtSizeY  =  1  <<  MaxBtLog2SizeY		(7-32)

MinBtSizeY  =  1  <<  MinBtLog2SizeY		(7-33)

MaxTtSizeY  =  1  <<  MaxTtLog2SizeY		(7-34)

MinTtSizeY  =  1  <<  MinTtLog2SizeY		(7-35)

MaxMttDepth  =  ( slice_type  = =  I )  ?  max_mtt_hierarchy_depth_intra_slices  :  
				max_mtt_hierarchy_depth_inter_slices	(7-36)

MaxMttDepth  =  1  <<  max_mtt_hierarchy_depth	(7-36)

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).