| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0225-r1 |
| --- | --- |

| Title: | CE11.3.3: Deblocking for 4 x N and N x 4 block boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Zhijie Zhao Jianle Chen | Tel: Email: | anand.meher.kotra @huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results for CE 11 test 11.3.3 which proposes two different ways of performing deblocking for 4 x N and N x 4 block boundaries. CE 11.3.3 subtest 1 uses an asymmetric filter which modifies different number of samples on either side of the edge based on their respective block sizes. For vertical edges, for the blocks whose width is 4 samples, the proposed filter uses three samples in filter decision process and modifies one sample in filter modification. CE 11.3.3 subtest 2 modifies only one sample on either side of the edge whenever at least one of the block sharing the edge has a size of 4 samples. CE 11.3.3 subtest 2 uses the existing Luma weak filer of VTM 2.0.1 with maximum one sample modification.1. Both subtest 1 and subtest 2 ensures that parallel deblocking can be performed without introducing any overlaps in the filtering process.

Objective results for both the techniques are as follows:

For CE 11.3.3 subtest1:

Over VTM 2.0.1 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of 0.01%, -0.25%, -0.23%, -0.42% is achieved without any increase in EncT and DecT.

Over VTM 2.0 Anchor with ALF ON (AI, RA, LDB, LDP):  Luma BD-Rate of 0.08%, -0.13%, -0.20%, -0.19% is achieved without any increase in EncT and DecT.

For CE 11.3.3 subtest2:

Over VTM 2.0.1 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.04%, -0.28%, -0.26%, -0.42% is achieved without any increase in EncT and DecT.

Over VTM 2.0 Anchor with ALF ON (AI, RA, LDB, LDP):  Luma BD-Rate of 0.02%, -0.15%, -0.23%, -0.23% is achieved without any increase in EncT and DecT.

# Introduction

VTM 2.0.1 performs deblocking on an 8 x 8 grid. Therefore deblocking is only performed for all the CU edges which overlap with the 8 x 8 grid. Due to the VTM 2.0 partioning scheme (BT and TT), there may be some long CU edges which may not overlap with the 8 x 8 grid but still have sufficient blocking artifacts. The scenario is depicted in the Fig. 1 below. The edge shown in red color belongs to a 4 x 16 block and current VTM 2.0.1 deblocking does not perform deblocking on this edge, as it does not overlap with an   8 x 8 grid.

Figure 1. VTM 2.0.1 performs deblocking on 8 x 8 grid. Few 4 x N edges are not filtered

# Proposed Method

Two different approaches Subtest1 and Subtest2 are proposed to perform deblocking for 4 x N and N x 4 blocks.

Subtest1 uses an Asymmetric filter where for the blocks whose size is less than 8 samples, only a maximum of three samples are used in the filter decision and only one sample is modified during the filtering step. For the blocks whose size is >=8 samples and share the edge with blocks whose size is less than 4 samples, the deblocking filter still uses a maximum of 4 samples in filter decision and can modify up to 3 samples during filtering step (same as in VTM 2.0.1). This situation is depicted in Fig. 2

Figure 2.  Subtest1 performs an asymmetric filter which modifies different number of samples on either side of the edge based on their respective block sizes.

For the blocks whose size is 4 i.e. block width = 4 for vertical edge and block height = 4 for horizontal edges, the strong filter conditions is modified as follows:

|p3,i – p0,i| + |q3,i – q0,i| < β/8 is changed to |p2,i – p0,i| + |q2,i – q0,i| < β/8

And also both strong filter and weak filter are only allowed to change one sample on either side of the edge i.e. p0 and q0. The filter equations used to calculate p0 and q0 are kept the same as in VTM 2.0.1 deblocking.

Subtest2 enforces the Luma weak filter with one or no sample modification, when any of the blocks sharing the edge has a size of 4 samples. (i.e. width = 4 samples for vertical edge, height = 4 samples for horizontal edge).

The different steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition is evaluated to true i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge. If true then

- Check if the one of the blocks has a block width of 4 samples, if true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

# Complexity Evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE11 document, JVET-K1031 [1].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

- For subtest1:

  - For blocks size >=8 samples: it is the same as in VTM 2.0.1

  - For block size < 8 samples: one sample is modified

- For subtest2:

  - For blocks size >=8 samples: it is the same as in HEVC. Except when the edge is shared with a block whose size is 4 samples. The only one sample is modified

  - For block size < 8 samples: one sample is modified

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size >=8 samples: 4 samples are used (same as in VTM 2.0.1)

  - for block size < 8 samples

    - For subtest1: three samples are used.

    - For subtest2: two samples are only used.

- Number of filtering operations:

  - The maximum number of filter operations per edge, is still the same as in VTM 2.0.1

- Number of line buffers: 4 lines (same as VTM 2.0.1)

- Parallelization capabilities: All edges overlapping with 4 x 4 grid can be deblocked in parallel.

- Luma only (Chroma filter is not changed)

# Experimental results

The proposed method is implemented on top of VTM-2.0.1 [2] and the simulation is performed under common test conditions as described in JVET-K1010 [3].

The results are presented for both ALF ON and ALF OFF conditions as suggested to be reported by the CE11 description document [1].

The results for Subtest1 are as follows:

Table 1: CE 11.3.3 Subtest1 results (Left hand side with ALF OFF, Right hand side with ALF ON)

The results for Subtest2 are as follows:

Table 2: CE 11.3.3 Subtest2 results (Left hand side with ALF OFF, Right hand side with ALF ON)

# Conclusion

This contribution presents the results for CE test 11.3.3. Two different approaches are proposed to deblock the edges belonging to 4 x N and N x 4 blocks. Both approaches ensure parallel deblocking can be performed for all the edges. Maximum Luma BD-Rate gain of 0.42% is achieved for LDP configuration when ALF is switched off. It is suggested to compare the proposed technique with the other CE 11.3 tests in terms of complexity and subjective and objective quality.

# References

- “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, July 2018.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.

- “JVET common test conditions and software reference configurations”, JVET-K1010

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).