| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0226-r1 |
| --- | --- |

| Title: | CE11-related: Position dependent adaptive Tc clipping range for deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Zhijie Zhao Jianle Chen | Tel: Email: | anand.meher.kotra @huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution propose an adaptive Tc clipping range for the deblocking filtering. The current deblocking filter in VTM 2.0.1 uses a fixed clipping range, where all the samples after filtering are clipped to the following range of values [P – 2tc, P + 2tc]. This clipping range does not take into account the position of the sample from the edge and therefore in few cases, the filtering may result in blurring or over smoothing. To address the above issue, the proposed technique defines an adaptive Tc clipping range as a “monotonically decreasing function” where-in the clipping range is defined based on the distance of the sample from the edge i.e.: closer the sample is to the edge, larger is the clipping range allowed and therefore larger is the deviation allowed after filtering. Two different “monotonically decreasing functions” are implemented on top of “VTM 2.0.1” and “VTM 2.0.1 plus the longer tap filter” proposed in JVET-K0369 [4].

The first monotonic function is defined as follows: [Pi – (tc + (tc >> i)), Pi + (tc + (tc >> i))] where i is the sample distance from the edge.

The second monotonic function is defined as follows: [Pi – (2tc >> i), Pi + (2tc >> i)] where i is the sample distance from the edge

Objective results for both the techniques are as follows:

For the “first” monotonically decreasing function:

Over VTM 2.0.1 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of 0.00%, 0.00%, 0.03%, -0.03% is achieved with 98% EncT and 98% DecT.

Over VTM 2.0.1 Anchor with ALF ON (AI, RA, LDB, LDP):  Luma BD-Rate of 0.00%, 0.00%, 0.00%, -0.04% is achieved with 98% EncT and 98% DecT.

Over “VTM 2.0.1 plus the longer tap filter proposed in JVET-K0369” Anchor with ALF OFF (AI, RA, LDB, LDP): -0.02%, -0.06%, 0.08%, -0.06% is achieved with 98% EncT and 98% DecT.

Over “VTM 2.0.1 plus the longer tap filter proposed in JVET-K0369” Anchor with ALF ON (AI, RA, LDB, LDP): 0.00%, 0.00%, 0.00%, 0.02% is achieved with 98% EncT and 98% DecT.

For the “second” monotonically decreasing function:

Over VTM 2.0.1 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of 0.00%, 0.00%, 0.03%,   -0.01% is achieved with 98% EncT and 98% DecT.

Over VTM 2.0.1 Anchor with ALF ON (AI, RA, LDB, LDP):  Luma BD-Rate of 0.00%, 0.00%, -0.02%, -0.01% is achieved with 98% EncT and 98% DecT.

Over “VTM 2.0.1 plus the longer tap filter proposed in JVET-K0369” Anchor with ALF OFF (AI, RA, LDB, LDP): -0.04%, -0.04%, -0.07%, -0.05% is achieved with 98% EncT and 98% DecT.

Over “VTM 2.0.1 plus the longer tap filter proposed in JVET-K0369” Anchor with ALF ON (AI, RA, LDB, LDP): -0.03%, -0.05%, -0.08%, -0.09% is achieved with 98% EncT and 98% DecT.

# Introduction

A typical boundary artifact is shown in Figure 1. The discontinuity observed between the samples p0 and Q0 is generally larger when compared to the other samples.

Figure 1. Typical boundary artifact

The current deblocking filter in VTM 2.0.1 introduces a clipping range after the filtering is performed to ensure that no over-smoothing is performed, especially on the lines/samples which are not used in the filter decision. As shown in Fig. 2 only samples belonging to lines 0 and 3 are used in the deblocking decision process.

Figure 2, Four-sample segment of vertical block boundary between adjacent blocks P and Q. Deblocking decision are made based on lines 0 and 3 (dashed)

The clipping range which is used in the strong filter is defined as [P – 2tc, P + 2tc], where tc is a offset which is dependent on the QP. Higher the QP value, larger is the tc value and hence larger is the clipping range. The sample value after filtering is allowed to be deviated by a greater extent when the clipping range is larger. This clipping range is constant for all the samples p0, p1, p2, and so on. Therefore all the samples are allowed to be deviated using the same range. This therefore may introduce blurredness or over smoothing in some cases. To address this problem, the current proposal proposes a position dependent adaptive tc clipping range which is defined as a “monotonically decreasing function” where-in the clipping range is defined based on the distance of the sample from the edge i.e.: closer the sample is to the edge, larger is the clipping range allowed and therefore larger is the deviation allowed after deblocking.

# Proposed Method

Two different “monotonically decreasing functions” are proposed.

The first monotonically decreasing function is defined as follows: [Pi – (tc + (tc >> i)), Pi + (tc + (tc >> i))] where i is the sample distance from the edge.

Therefore P0 sample after filtering is clipped to the range ( P0 – (tc + (tc >> 0)), P0 + (tc + (tc >> 0)))

P1 sample after filtering is clipped to the range ( P1 – (tc + (tc >> 1)), P1 + (tc + (tc >> 1)))

P2 sample after filtering is clipped to the range ( P2 – (tc + (tc >> 2)), P2 + (tc + (tc >> 2))) and so on.

The second monotonically decreasing function is defined as follows: [Pi – (2tc >> i), Pi + (2tc >> i)] where i is the sample distance from the edge

Therefore P0 sample after filtering is clipped to the range (P0 – (2 * tc >> 0) , P0 + (2* tc >> 0))

P1 sample after filtering is clipped to the range (P1 – (2 * tc >> 1) , P1 + (2* tc >> 1))

P2 sample after filtering is clipped to the range (P2 – (2 * tc >> 2) , P2 + (2* tc >> 2)) and so on.

# Experimental results

## Results for “first monotonically decreasing function”

The proposed method is implemented on top of VTM-2.0.1 [2] and the simulation is performed under common test conditions as described in JVET-K1010 [3].

The results are presented for both ALF ON and ALF OFF conditions

Table 1: First monotonically decreasing function on top of VTM 2.0.1 (Left hand side with ALF OFF, Right hand side with ALF ON)

The proposed method is implemented on top of VTM-2.0.1 [2] plus the longer tap filter proposed in JVET-K0369 [4].

The results are presented for ALF OFF are presented in table 2:

On the left hand side:

Anchor : VTM 2.0.1

Test: Anchor + JVET-K0369 + “first monotonically decreasing function”

On the right hand side:

Anchor: VTM 2.0.1 + JVET-K0369

Test: Anchor + “first monotonically decreasing function”

Table 2: First monotonically decreasing function, ALF OFF, on top of VTM 2.0.1 (left hand side) and VTM 2.0.1+ JVET-K0369 (Right hand side)

The results are presented for ALF ON are presented in table 3:

On the left hand side:

Anchor: VTM 2.0.1

Test: Anchor + JVET-K0369 + “first monotonically decreasing function”

On the right hand side:

Anchor: VTM 2.0.1 + JVET-K0369

Test: Anchor + “first monotonically decreasing function”

Table 3: First monotonically decreasing function, ALF ON, on top of VTM 2.0.1 (left hand side) and VTM 2.0.1+ JVET-K0369 (Right hand side)

## Results for “second monotonically decreasing function”

The proposed method is implemented on top of VTM-2.0.1 [2] and the simulation is performed under common test conditions as described in JVET-K1010 [3].

The results are presented for both ALF ON and ALF OFF conditions

Table 4: Second monotonically decreasing function on top of VTM 2.0.1 (Left hand side with ALF OFF, Right hand side with ALF ON)

The proposed method is implemented on top of VTM-2.0.1 [2] plus the longer tap filter proposed in JVET-K0369 [4].

The results are presented for ALF OFF are presented in table 5:

On the left hand side:

Anchor: VTM 2.0.1

Test: Anchor + JVET-K0369 + “second monotonically decreasing function”

On the right hand side:

Anchor: VTM 2.0.1 + JVET-K0369

Test: Anchor + “second monotonically decreasing function”

Table 5: Second monotonically decreasing function, ALF OFF, on top of VTM 2.0.1 (left hand side) and VTM 2.0.1+ JVET-K0369 (Right hand side)

The results are presented for ALF ON are presented in table 6:

On the left hand side:

Anchor: VTM 2.0.1

Test: Anchor + JVET-K0369 + “second monotonically decreasing function”

On the right hand side:

Anchor: VTM 2.0.1 + JVET-K0369

Test: Anchor + “second monotonically decreasing function”

Table 6: Second monotonically decreasing function, ALF ON, on top of VTM 2.0.1 (left hand side) and VTM 2.0.1+ JVET-K0369 (Right hand side)

# Conclusion

This contribution presents an adaptive Tc clipping range as a “monotonically decreasing function” where-in the clipping range is defined based on the distance of the sample from the edge i.e.: closer the sample is to the edge, larger is the clipping range allowed and therefore larger is the deviation allowed after filtering. Two different “monotonically decreasing functions” are implemented on top of “VTM 2.0.1” and “VTM 2.0.1 plus the longer tap filter” proposed in JVET-K0369 [4]. Both proposals bring slight BD-Rate gain with reduced EncT and DecT and also improve the subjective quality in the cases where blurring or over-smoothing is observed during filtering. We propose to further study this contribution in a deblocking core experiment

.

# References

- “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, July 2018.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.

- “JVET common test conditions and software reference configurations”, JVET-K1010

- “CE2-related: Longer Tap Deblocking Filter”, JVET-K0369

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).