| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0227-v1 |
| --- | --- |

| Title: | AHG12: Sub-bitstream extraction/merging friendly slice address signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin, Yago Sanchez, Karsten Sühring, Thomas Schierl, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The extraction or merging of MCTS sub-bitstreams in HEVC is cumbersome, as potentially all slice headers need to be rewritten. This document proposes changes to the slice_address signalling in VVC in order to allow extraction or merging of MCTS based sub-bitstreams without the necessity of slice header changes.

The proposal is to signal slice addresses in slice headers

- relative to the first CTU of a tile

- in tile scan order

- code slice address depending on tile size

wherein tile refers to a rectangular group of CTUs.

“Final” slice address in raster scan order within the picture are derived through adding a per-tile slice base address given from the tiling structure. As there is currently no tiling syntax in VVC, the presented syntax changes rely on an HEVC-style tiling syntax.

As the current VVC design requires parameter set parsing for access unit boundary detection, this document in addition proposes to integrate an access unit delimiter NAL unit and to mandate its use when the proposed slice address signalling scheme is enabled.

# Introduction

The extraction of MCTS sub-bitstreams in HEVC, e.g. based on the HEVC EIS SEI message, is an important functionality in a number of HEVC based applications. For instance, content preparation for 360-degree video delivery according to the HEVC based viewport-dependent OMAF video profile in ISO/IEC 23090-2 may rely on sub-bitstream extraction of individual tiles. Further applications as in [1] may also rely on carriage of MCTS sub-bitstreams over MPEG2-TS according to ISO/IEC 13818-1 (Ed 7) which also includes sub-bitstream extraction. It is therefore seen as important to add such functionality to VVC.

However, when using HEVC, MCTS sub-bitstream extraction can be cumbersome as each VCL NAL unit of the extracted sub-bitstream may require adjustments. This is because, similar to the current VVC draft specification [3], slice headers contain a raster scan order slice address relative to a first CTU of a coded picture wherein the coded length of the slice address depends on the coded picture size. During extraction or merging, the value and coded length of slice addresses in the slice header of extracted VCL NAL units may require adjustment which in turn may require changes to the byte alignment at the end of each slice header. Therefore, [2] proposed a VVC design goal of allowing extraction (and merging) of VCL sub-bitstreams originating from different bitstreams without VCL NAL unit changes.

This document targets to enable such sub-bitstream extraction or merging by introducing a new scheme for the slice address signalling in VVC.

# Proposal

The proposal is to signal slice addresses in slice headers

- relative to the first CTU of a tile

- in tile scan order

- code slice address depending on tile size

wherein tile refers to a rectangular group of CTUs. In the proposal, the signalled slice addresses are shifted through a per-tile offset, referred to as a slice base address, in order to derive the “final” slice address within the picture in raster scan.

Tile-wise slice base addresses (TileSliceBaseAddr[ k ]) are derived from the tiling structure in the parameter sets.

A fixed length coded tile identifier in slice headers (tile_id_in_sh[ i ]) and parameter sets (tile_id_in_pps[ i ]) associates slices with the slice base addresses.

It is assumed that the proposal allows MCTS sub-bitstream extraction without the necessity to adjust slice headers in all VCL access units. Merging of MCTS sub-bitstreams would require adjustments to the fixed length coded tile identifier in slice headers only in case of conflicts which could be avoided altogether in controlled environments such as closed encoding systems and would also only affect part of the access units.

Design decisions

The design decision to use explicit tile identifiers for association of slices and slice base addresses falicitates the merging use case as it allows to easily change the association if necessary by rewriting the parameter sets only, e.g. in case of merging varying tile rearrangements. In case of tile identifier conflicts, required adjustments to the fixed length coded tile identifiers in VCL NAL units do not affect the remaining syntax in the slice header.

The current VVC draft specification is yet silent about access unit boundary detection. The current VVC design allows access unit boundary detection only through parsing the slice_address in the slice headers which requires parameter set activation. We propose to integrate the access unit delimiter NAL unit into VVC for straight-forward access unit boundary detection without the necessity of parsing slice headers and parameter sets. Its use is mandated when the proposed slice address signalling scheme is enabled (slice_base_addr_per_tile_enabled_flag equal to 1).

Furthermore, tile syntax is yet absent from the latest VVC draft specification. Therefore, the proposal assumes presence of HEVC-style tile syntax and may need slight adjustments to the final VVC tiling structure.

# Specification text

The following is specification text for the proposal relative to K1001-v6 with changes highlighted in yellow. The HEVC-style tiling syntax is highlighted in turquoise and is envisioned to be adjusted to the final VVC tiling syntax.

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, is derived as follows:

[…]
for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ )
		tileIdxTileIdMap[ tile_id_in_pps[ tileIdx ] ] = tileIdx
		for( y = rowBd[ j ]; y < rowBd[ j + 1 ]; y++ )	(X-X)
			for( x = colBd[ i ]; x < colBd[ i + 1 ]; x++ )
				TileId[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = tileIdx

The list TileSizeInCtbsY[k] for k ranging from 0 to ((num_tile_rows_minus1+1) * ( num_tile_columns_minus1 +1) - 1), inclusive, specifying the size of the k-th tile within the picture in units of CTBs, is derived as follows:

for( j = 0, k = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, k++ ) 		TileSizeInCtbsY [ k ] = colWidth[ i ] * rowHeight[ j ]

The list TileSliceBaseAddr[k] for k ranging from 0 to ((num_tile_rows_minus1+1) * ( num_tile_columns_minus1 +1) - 1), inclusive, specifying the slice base address in tile scan of the first slice of the k-th tile within the picture in bitstream order in units of CTBs, is derived as follows:

TileSliceBaseAddr[ 0 ] = 0
	for( k = 1; k  <=  ( ( num_tile_rows_minus1 + 1 ) * (num_tile_columns_minus1 + 1) – 1) ; k++ )
		TileSliceBaseAddr[ k ] = TileSliceBaseAddr[ k – 1 ] + TileSizeInCtbsY [ k – 1 ]

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| slice_base_addr_per_tile_enabled_flag | u(1) |
| if ( slice_base_addr_per_tile_enabled_flag ) { |  |
| for( i = 0; i < (( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) – 1 ); i++ ) |  |
| tile_id_in_pps[ i ] | u(8) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

slice_base_addr_per_tile_enabled_flag equal to 0 specifies that the variable CtbAddrInRs of a slice is derived solely from slice_ address. slice_base_addr_per_tile_enabled_flag equal to 1 specifies that deriviation of CtbAddrInRs is based on slice_address and a tile dependent base address TileSliceSegBaseAddr[ k ]. When slice_base_addr_per_tile_enabled_flag is not present, it is infered to be equal to 0.

tile_id_in_pps[ i ] specifies the id of tiles in bitstream order. The value of tile_id_in_pps[ i ] shall be in the range of 0 to 255 and tile_id_in_pps[ i ] shall not have the same value as tile_id_in_pps[ j ] for i unequal to j. When tile_id_in_pps is not present, it is infered to be equal to 0

It is a constraint of bitstream conformance that when slice_base_addr_per_tile_enabled_flag is equal to 1, access unit delimiter NAL units are present in the bitstream.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if ( slice_base_addr_per_tile_enabled_flag ) |  |
| tile_id_in_sh | u(8) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

tile_id_in_sh specifies the index of the tile that the slice belongs to. The value of tile_id_in_sh shall be in the range of 0 to 255. When tile_id_in_sh is not present, it is inferred to be equal to 0. No more than one tile within a picture shall have the same value of tile_id_in_sh.

slice_address specifies the address of the first CTB in the slice, in CTB raster scan of a picture when slice_ base_addr_per_tile_enabled_flag  equal to 0 and tile scan of a picture otherwise.

The variable maxNumCtbY is defined as ( slice_base_addr_per_tile_enabled_flag ? TileSizeInCtbsY[ tileIdxTileIdMap[ tile_id_in_sh ]  ] : PicSizeInCtbsY )

The length of the slice_address syntax element is Ceil( Log2( PicSizeInCtbsY maxNumCtbY ) ) bits. The value of slice_address shall be in the range of 0 to PicSizeInCtbsY maxNumCtbY  − 1, inclusive.

When slice_base_addr_per_tile_enabled_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture and the variable CtbAddrInRs, specifying a CTB address in CTB raster scan of a picture, is set equal to slice_address. Otherwise, when slice_base_addr_per_tile_enabled_flag is equal to 1, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit belonging to the same tile within the same coded picture and the variable CtbAddrInRs is set equal to CtbAddrTsToRs[ slice_address + TileSliceBaseAddr[ tileidxTileidMap[ tile_id_in_sh ] ] ] .

# References

- R. Skupin, Y. Sanchez, C. Hellge, T. Schierl (HHI), T. Wrede, T. Christophory (SES), “AHG12: MCTS extraction usage scenario and showcase”, JCTVC-Z0032, Jan 2017

- M. M. Hannuksela, A. Zare, M. Homayouni, R. Ghaznavi-Youvalari, A. Aminlou, “Design goals for tiles”, JVET-K0300, July 2018

- B. Bross, “Versatile Video Coding (Draft 2)”, JVET-K1001-v6, September 2018

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).