| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0241-v4 |
| --- | --- |

| Title: | AHG10: Adaptive lambda ratio estimation for rate control in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zizheng Liu, Yiming Li, Zhenzhong Chen Wuhan University Xiang Li, Shan Liu Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents some improvements based on the current rate control scheme proposed in JVET-K0390. With the proposed adaptive lambda ratio estimation algorithm, when using the anchor bit rate of BMS2.1 with VTM configuration as the target, there are 0.34%/4.57%/4.09% for Y/U/V coding efficiency improvements in random access (RA) configuration reported when compared with the rate control algorithm in JVET-K0390.

Introduction

The R-lambda rate control algorithm for HEVC, JCTVC-K0103 [1] and M0036 [2], were introduced and adopted in HM at JCT-VC. Recently JVET-K0390 [3] further developed the R-lambda rate control algorithm by updating the rate-distortion (R-D) parameters of skip and non-skip CTUs separately with different strategies. In this proposal, we present some improvements based on JVET-K0390. The improvements come from an adaptive lambda ratio estimation in random access configuration.

Solution

In this proposal, we employ an adaptive lambda ratio estimation method for RA coding structure. A GOP in RA configuration contains 16 pictures belonging to 5 coding levels. Supposing the target number of bits for this GOP is , the picture level bit allocation problem is to solve:

(1)

(2)

where  represents the influence of picture .

We executed some experiments of VVC without rate control to obtain the influence of different coding levels and trained the lambda ratio for different coding levels. The modified lambda ratios of different levels are set as:

| level | Lambda ratio |
| --- | --- |
| 0 | 1.0 |
| 1 | 0.725 * log(encRCSeq->getLastLambda() / pow(2.0, bitdepth_luma_scale)) + 0.7963; |
| 2 | 1.3 * lambdaRatio[1]; |
| 3 | 2.15 * lambdaRatio[1] |
| 4 | 2.8 * lambdaRatio[1] |

Results

The experiments were based on BMS 2.1 with VTM configuration and were carried out on HPC cluster in Linux platform. Compared with the method in JVET-K0390, our method can achieve 0.34%/4.57%/4.09% for Y/U/V coding efficiency improvements in random access configuration as shown in Table 1. The performance of the proposed method comparing to fix QP method is also shown in Table 2.

Table 1 Proposed method vs. JVET-K0390 (current rate control method in BMS2.1)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -3.25% | -2.45% | 99% | 100% |
| Class A2 | 0.15% | -5.67% | -5.49% | 99% | 100% |
| Class B | -0.33% | -5.14% | -4.65% | 99% | 102% |
| Class C | -0.98% | -4.03% | -3.57% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -4.57% | -4.09% | 99% | 100% |
| Class D | -0.72% | -4.88% | -4.59% | 98% | 98% |

Table 2 Proposed method vs. FixQP

|  | Low delay B Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | 11.29% | 3.71% | 3.28% | 101% | 99% |
| Class A2 |  |  |  |  |  | 3.89% | 0.13% | 0.20% | 101% | 95% |
| Class B | -2.36% | -11.58% | -13.13% | 101% | 102% | 2.87% | 0.57% | 2.22% | 104% | 101% |
| Class C | -2.64% | -8.88% | -7.88% | 102% | 101% | 1.62% | -1.72% | -0.42% | 101% | 98% |
| Class E | 0.10% | -6.36% | -5.66% | 99% | 100% |  |  |  |  |  |
| Overall | -1.84% | -9.37% | -9.51% | 101% | 101% | 4.42% | 0.50% | 1.32% | 102% | 99% |
| Class D | -0.75% | -4.49% | -3.67% | 102% | 101% | 3.15% | 8.82% | 11.09% | 100% | 100% |

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- B. Li, H. Li, L. Li, and J. Zhang, “Rate control by R-lambda model for HEVC,” JCT-VC K0103, Oct. 2012.

- B. Li, H. Li, and L. Li, “Adaptive bit allocation for R-lambda model rate control in HM,” JCT-VC M0036, Apr. 2013.

- Y. Li, Z. Chen, X. Li and S. Liu, “Rate Control for VVC,” JVET-K0390, Jul. 2018.