| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0242-v2 |
| --- | --- |

| Title: | AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Zhenzhong Chen, Yiming Li Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution presents the simulation results of dense residual convolutional network based in-loop filter (DRNLF), which was proposed in JVET-K0391. Multiple modifications have been carried out to simplify the network in terms of network structure, training materials, and training process. With the proposed modifications, the number of the network parameters have been reduced from 810K to 22K. Simulation results report 2.34%, 1.61%, 2.14% BD-rate savings for Y, Cb, and Cr components compared with VTM2.0.1 under AI configuration.

Introduction

In [1], a dense residual convolutional network based in-loop filter (DRNLF) was proposed to be used as an additional filter in VTM software, which provides impressive coding performance improvement. However, the employ of DRNLF introduces great computational costs. In this contribution, a simplified network structure is employed for the trade off between computational efficiency and performance. Besides, the network is trained on dataset in YUV color spaces with a loss function designed to balance the coding gain between different components.

Technical description

Proposed Decoding Scheme

In current VTM software, multiple in-loop filters including deblocking filter (DF), sample adaptive offset (SAO) and adaptive loop filter (ALF) are employed. Similar to [1], the proposed DRNLF is introduced into VTM as an additional filter placed after DF, and before SAO and ALF, as shown in Fig.1. The decision of whether to apply proposed filter or not is based on the rate-distortion optimization (RDO).

Fig. 1 Proposed decoding scheme in VTM

DRNLF

In this section, the network structure of DRN is described firstly and then training details are provided.

Network Structure

The network structure of DRN is shown in Fig.2, where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Compared with [1], simplified network is used in this contribution.

Firstly, the 3x3 convolutional layers outside the global identity skip connection are removed, which contributes to slightly better performance and faster coverage during training the network..

Secondly, normalized QP map is concatenated with the reconstructed frame as the inputs to DRN so that only one model is needed for different QPs.

Thirdly, the network is trained on dataset in YUV color spaces with a loss function designed to balance the coding gain among different components (see Sec. 2.2.2).

Fourthly, in order to reduce the computational costs of the network, the number of DRU is reduced from 8 to 4 and convolution kernel is reduced from 64 to 32, respectively.

Lastly, 3x3 conventional convolutional layer is replaced by 3x3 depth-wise separable convolutional (DSC) layer in each DRU.

After the above five modifications, the total number of the model parameters in this contribution is 22K compared to 810K in [1].

Fig. 2 Network structure of DRN.

Training details

DIV2K dataset [2] is utilized to generate the training sets and validating set for the proposed network. There are 800 images in its training set and 100 images in its validation set. The network is trained on YUV color space directly. The images in DIV2K dataset are converted from RGB color space to YUV color space before compression. The images are compressed by VTM-2.0.1 [3] using different QPs under All Intra (AI) configuration. The compressed images and its corresponding QP are used as the input to the network. And the images before compression are used as the ground truth. Given a set of images with compression artifacts , and the corresponding ground truth images . The goal of the training is to minimizes the following loss:

|  |  | (1) |
| --- | --- | --- |
|  |  | (2) |

Where  denotes the 	parameters of the proposed DRN,  denotes mean square error (MSE),  is the weight of component indexed by . In this contribution, the weights of Y, Cb, Cr are set to 10, 1, 1, respectively.

Simulation Results

The proposed DRNLF is integrated into VTM-2.0.1 as an additional filter placed after DF, and before SAO and ALF. All the sequences recommended in common test condition (CTC) are evaluated with All-Intra configuration. The QP values are 22, 27, 32, and 37, respectively. The results of the experiments running without GPU are provided in Table 1.

Table 1 Experimental Results of the proposed scheme

|  | All Intra Main10 (with CPU only) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.56% | -2.21% | -4.20% | 124% | 6611% |
| Class A2 | -1.89% | -1.24% | -0.83% | 110% | 3818% |
| Class B | -1.45% | -1.71% | -2.47% | 108% | 5120% |
| Class C | -3.08% | -1.92% | -1.75% | 107% | 6914% |
| Class E | -4.06% | -0.81% | -1.39% | 117% | 11210% |
| Overall | -2.34% | -1.61% | -2.14% | 112% | 6198% |
| Class D | -3.74% | -2.13% | -2.41% | 105% | 6635% |

Several tests are conducted to investigate the influences in terms of network structure, training materials, and training process. Table 2 gives a summary of the tests, where N and M denotes the number of dense residual unit (DRU) and convolution kernel respectively, T denotes the color space of the training materials, W denotes the weights of components in Eq. 2, S denotes whether the new structure are used or not and DSC denotes whether 3x3 DSC are applied to replace 3x3 convolutional layer in DRU or not. The details of the experiments are describes in following section.

Table 2 Summary of the Tests

|  | N | M | C | W | S | DSC |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 8 | 64 | RGB | 1:1:1 | F | F |
| Test 2 | 8 | 64 | YUV | 10:1:1 | T | F |
| Test 3 | 4 | 32 | RGB | 1:1:1 | T | F |
| Test 4 | 4 | 32 | YUV | 1:1:1 | T | F |
| Test 5 | 4 | 32 | YUV | 4:1:1 | T | F |
| Test 6 | 4 | 32 | YUV | 10:1:1 | T | F |
| Test 7 | 4 | 64 | YUV | 10:1:1 | T | F |
| Test 8 | 8 | 32 | YUV | 10:1:1 | T | F |
| Test 9 | 4 | 32 | YUV | 10:1:1 | T | T |

*Test1: The network in [1]

*Test9: Proposal

Compared with the network of last proposal [1]

Table 3 shows the overall BD-rate results of Test 1 and Test 2. In Test 1, the network of JVET-K0391 is employed. The network in Test 2 employs the first 3 simplification as described in Sec. 2 . Comparison between Test1 and Test2 shows that the simplified network is better than [1] with the equivalent amount of parameters.

Table 3 Compared with the network of JVET-K0391

|  | Y | U | V |
| --- | --- | --- | --- |
| Test 1 | -3.06% | -12.09% | -13.24% |
| Test 2 | -4.42% | -9.90% | -10.98% |

Comparisons of the training materials

In this section, Test 3-6 are conducted to investigate the influences of the training materials and loss function. Network structure is the same among these 4 tests (i.e. N=4, M=32, S=True, DSC=False), and the network is trained with different training sets and different weights in loss functions. To be specific, Test 3 are trained on RGB images with weights of 1:1:1, Test 4-6 are trained on YUV images with weights of 1:1:1, 4:1:1, 10:1:1, respectively. The results are shown in Table 4.

Table 4 Comparisons of the training materials

|  | T | W | Y | U | V |
| --- | --- | --- | --- | --- | --- |
| Test3 | RGB | 1:1:1 | -1.84% | -8.17% | -9.18% |
| Test4 | YUV | 1:1:1 | -1.70% | -8.01% | -9.33% |
| Test5 | YUV | 4:1:1 | -2.49% | -7.00% | -7.79% |
| Test6 | YUV | 10:1:1 | -2.84% | -3.97% | -4.13% |

Comparisons of the networks

Table 5 shows the overall BD-rate results and the number of parameters of networks with different structure. Test 9 employs the network in [1] that is trained on RGB images, while the others are trained on YUV images with weights of 10:1:1. The ratio are based the model parameters of Test 2. To further reduce the computational costs, depth-wise separable convolutional layer is employed to replace the standard convolutional layer in DRU. The relation between the ratio of parameters and BD-rate savings is shown in Fig. 3.

Table 5 Comparisons of the networks

|  | N | M | Param. | Ratio | Y | U | V |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test2 | 8 | 64 | 738755 | 100% | -4.42% | -9.90% | -10.98% |
| Test8 | 8 | 32 | 186083 | 25% | -3.68% | -6.94% | -8.41% |
| Test7 | 4 | 64 | 336579 | 46% | -3.30% | -8.06% | -9.09% |
| Test6 | 4 | 32 | 85347 | 12% | -2.84% | -3.97% | -4.13% |
| Test9 | 4 | 32 | 22371 | 3% | -2.34% | -1.61% | -2.14% |
| Test1 | 8 | 64 | 812035 | 110% | -3.06% | -12.09% | -13.24% |

*Test1: The network in [1]

*Test9: Proposal

Fig. 3 Relation between the ratio of parameters and BD-rate savings

Conclusions

This contribution presents modifications of DRNLF in terms of network structure, training materials, and training process for the trade off between computational efficiency and performance. Simulation results report 2.34%, 1.61%, 2.14% BD-rate savings for luma, and both chroma components compared with VTM2.0.1 under AI configuration.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “Dense Residual Convolutional Neural Network based In-Loop Filter,” JVET-K0391, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

- VTM 2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1

Appendix

Table 6 to Table 13 show the per-class summary table of each Test.

Table 6 Experimental Results of Test 1

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.68% | -9.90% | -9.89% |  |  |
| Class A2 | -2.18% | -12.90% | -9.91% |  |  |
| Class B | -2.07% | -11.85% | -13.54% |  |  |
| Class C | -4.23% | -13.59% | -16.59% |  |  |
| Class E | -5.40% | -11.84% | -14.94% |  |  |
| Overall | -3.06% | -12.09% | -13.24% |  |  |
| Class D | -4.82% | -13.71% | -17.87% |  |  |

Table 7 Experimental Results of Test 2

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.10% | -6.99% | -8.76% | 104% | 1163% |
| Class A2 | -3.56% | -11.77% | -8.96% | 102% | 1212% |
| Class B | -3.14% | -9.63% | -10.81% | 103% | 1524% |
| Class C | -5.58% | -11.58% | -13.70% | 105% | 1040% |
| Class E | -5.97% | -11.00% | -14.66% | 109% | 1520% |
| Overall | -4.42% | -9.90% | -10.98% | 104% | 1287% |
| Class D | -5.97% | -11.00% | -14.66% | 105% | 1332% |

Table 8 Experimental Results of Test 3

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -6.67% | -6.48% |  |  |
| Class A2 | -1.39% | -9.55% | -6.17% |  |  |
| Class B | -1.17% | -7.71% | -9.36% |  |  |
| Class C | -2.64% | -10.22% | -12.91% |  |  |
| Class E | -3.40% | -6.31% | -9.62% |  |  |
| Overall | -1.84% | -8.17% | -9.18% |  |  |
| Class D | -3.18% | -9.53% | -13.98% |  |  |

Table 9 Experimental Results of Test 4

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.26% | -5.81% | -7.42% |  |  |
| Class A2 | -1.22% | -9.20% | -6.13% |  |  |
| Class B | -0.98% | -7.57% | -9.70% |  |  |
| Class C | -2.35% | -10.08% | -12.38% |  |  |
| Class E | -2.79% | -9.72% | -13.82% |  |  |
| Overall | -1.70% | -8.01% | -9.33% |  |  |
| Class D | -2.79% | -9.72% | -13.82% |  |  |

Table 10 Experimental Results of Test 5

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -4.66% | -5.57% |  |  |
| Class A2 | -1.90% | -7.49% | -5.12% |  |  |
| Class B | -1.54% | -6.91% | -7.85% |  |  |
| Class C | -3.38% | -9.12% | -10.96% |  |  |
| Class E | -4.28% | -6.16% | -8.34% |  |  |
| Overall | -2.49% | -7.00% | -7.79% |  |  |
| Class D | -3.97% | -8.74% | -12.01% |  |  |

Table 11 Experimental Results of Test 6

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.97% | -3.10% | -3.39% |  |  |
| Class A2 | -2.18% | -4.55% | -2.52% |  |  |
| Class B | -1.83% | -4.08% | -4.36% |  |  |
| Class C | -3.76% | -4.70% | -5.65% |  |  |
| Class E | -4.81% | -3.07% | -4.09% |  |  |
| Overall | -2.84% | -3.97% | -4.13% |  |  |
| Class D | -4.34% | -5.00% | -6.57% |  |  |

Table 12 Experimental Results of Test 7

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.97% | -5.53% | -6.82% | 123% | 758% |
| Class A2 | -2.64% | -8.72% | -6.34% | 116% | 773% |
| Class B | -2.23% | -7.87% | -9.07% | 106% | 958% |
| Class C | -4.45% | -10.26% | -12.22% | 107% | 868% |
| Class E | -5.01% | -9.91% | -13.36% | 105% | 1080% |
| Overall | -3.30% | -8.06% | -9.09% | 111% | 881% |
| Class D | -5.01% | -9.91% | -13.36% | 105% | 1080% |

Table 13 Experimental Results of Test 8

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.55% | -4.61% | -6.40% | 119% | 850% |
| Class A2 | -2.89% | -7.60% | -5.97% | 106% | 794% |
| Class B | -2.53% | -6.74% | -8.36% | 105% | 1086% |
| Class C | -4.76% | -9.01% | -11.52% | 105% | 810% |
| Class E | -6.08% | -6.19% | -8.79% | 107% | 1064% |
| Overall | -3.68% | -6.94% | -8.41% | 108% | 924% |
| Class D | -5.30% | -8.75% | -12.36% | 105% | 1101% |

Table 14 Experimental Results of Test 9

|  | All Intra Main10 (with GPU) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.56% | -2.21% | -4.20% | 102% | 755% |
| Class A2 | -1.89% | -1.24% | -0.83% | 102% | 594% |
| Class B | -1.45% | -1.71% | -2.47% | 102% | 811% |
| Class C | -3.08% | -1.92% | -1.75% | 102% | 755% |
| Class E | -4.06% | -0.81% | -1.39% | 102% | 922% |
| Overall | -2.34% | -1.61% | -2.14% | 102% | 765% |
| Class D | -3.74% | -2.13% | -2.41% | 102% | 1100% |