| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0246 |
| --- | --- |

| Title: | CE12-4: SDR In-loop Reshaping |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu, fpu}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This proposal reports CE12-4 subtests on SDR in-loop reshaping using low complexity in-loop reshaping architecture. The proposed reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) complementary chroma residue scaling applied to chroma components. The loop filter is applied in the original (non-mapped) domain for both intra and inter slices.

Summary performance gains of CE12-4 subtests for in-loop reshaping of SDR Content under CTC compared to VTM2.0.1 SDR Anchor are:

- RA BDrate Y/U/V: {-1.32%, 2.07%, 1.62%}

- AI BDrate Y/U/V: { -0.96%, 2.56%, 2.13%}

Performance gains are larger for higher resolution video (UHD and HD). The decoding running time is 105% of SDR anchor and encoding running time is within 107% of SDR anchor.

# Introduction

In-loop reshaping is proposed in JVET-J0015 [1] and was tested on SDR video in JVET-K0309 [3] with VTM1.0 code base. CE12-4 is designed to evaluate the in-loop reshaping architecture with VTM2.0.1 code base. CE12-4 and CE12-2 [5] use the same software build to support both SDR and HDR in-loop reshaping.

# CE12-4 Algorithm Description

## In-loop reshaping architecture and components

The complete in-loop reshaping architecture comprises the in-loop luma mapping, the in-loop chroma residue scaling, and configurable domain for loop filter application. For CE12-4, low complexity in-loop luma reshaping configuration is used where the loop filter application is always in the original domain.

### In-loop luma mapping

The in-loop luma reshaper is implemented as a pair of look-up tables (LUTs); but only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT.

Each LUT is a one-dimensional, 10-bit, 1024-entry mapping table (1D-LUT). One LUT is a forward LUT, FwdLUT, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. The other LUT is an inverse LUT, InvLUT, that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

For intra slices, only the InvLUT is applied.  For inter slices, both FwdLUT and InvLUT are applied.

LUTs are applied before loop filtering for both intra and inter slices.

The processing operations for intra slices are illustrated in Figure 1.  InvLUT maps intra reconstructed values in the reshaped domain to intra reconstructed values in the original domain. (Yi=InvLUT[Yr])

The processing operations for inter slices are illustrated in Figure 2.  FwdLUT maps motion-compensation values in the original domain to the reshaped domain. (FwdLUT[Ypred])  InvLUT then maps inter reconstructed values in the reshaped domain to inter reconstructed values in the original domain. (Yi=InvLUT[Yres+FwdLUT[Ypred]])

Figure 1. Intra slice reconstruction with in-loop luma reshaper

Figure 2. Inter slice reconstruction with in-loop luma reshaper

### In-loop chroma residue scaling

To compensate for the luma signal interaction with the chroma signal when luma reshaping is performed, the chroma components are adjusted with chroma residue scaling. This is an alternative implementation of the luma-based chroma QP Offset (CQPO) implementation in JVET-J0015[1] and JVET-K0308[2] to avoid the decoder-pipeline dependency issue in hardware implementation. At the encoder side, for the residue (CxRes = CxOrg - CxPred) of chroma component Cx of each CU,

CxResScaled = CxRes * cScaleYCU

CxResScaled is the scaled Cb/Cr residue signal of the CU to be transformed and quantized. At the decoder side, CxResScaled is the scaled chroma residue signal after inverse Q/T, and

CxRes = CxResScale / cScaleYCU

The final reconstruction of chroma component is CxRec = CxPred + CxRes. In this way, the decoder can start inverse quantization and transform immediately for chroma decoding after syntax parsing. The cScale being used for a CU is shared by the Cb and Cr components, and is derived as:

Equivalent cQPO[YCU]=-6*log2(FwdLUT'YCU)                (1)

cScaleYCU=FwdLUT'YCU=2-cQPO[YCU]6                       (2)

where YCU is the average predicted luma value of current CU in inter slices (where dual tree coding is not used and therefore reconstructed luma is not available), and YCU is the average reconstructed luma value of current CU in intra slices (where dual tree coding is used). In CE12-4 software, the scales are calculated and stored with 16bit fixed point integers and the scaling operations at both encoder and decoder side are implemented with fixed point integer arithmetic.

### Loop filter application

The in-loop reshaping architecture allows the loop filters (deblocking, SAO, and ALF) to be performed either in reshaped domain or in original domain. In CE12-4, the lowest complexity option is enabled so that inverse reshaping always happens before loop filtering.

### Adaptive SDR in-loop reshaping model

The adaptive reshaping curve is constructed by assigning a number of codewords to each bin(/piece) directly. The total codewords range are divided into 32 bins, which means each bin contains 32 codewords for 10 bit full range video. The goal of the reshaping is to find an optimized codeword allocation for each bin such that best RDO can be achieved. In the current implementation, each bin is assigned from one of four codeword numbers {M0, M1, M2, M3}, M0=0, M1=32, M1<M2<=64 and 0<M3<M1. For limited video range, no codewords (i.e., M0) will be assigned to bin {0, 1, 30, 31}. M2 and M3 are automatically set based on content characteristics. The FwdLUT is an integral of codewords assigned to each bin.

The encoder applies the algorithm in Table 1 for every intra slice. In the syntax, the number of codewords assigned to each bin is signaled. The decoder applies the algorithm in Table 2.

Table 1. Derivation process of adaptive SDR reshaping curve (encoder)

- Analyze the image to gather the histogram (bin_hist) and variance (bin_var) for each bin. The bin variance is computed by averaging the pixel variance of all pixels belonging to that bin. The pixel variance is computed by 5x5 windows. - Set parameters Thr1, Thr2, M2, M3 based on bin_hist and bin_var;

- Compare bin_var with two thresholds: Thr1 and Thr2 if bin_var = 0, the bin is allocated M0 (zero) codeword. else if bin_var < Thr1, the bin is allocated M2 codewords; else if bin_var > Thr2, the bin is allocated M3 codewords; else the bin is allocated M1+1 codewords. - Prepare syntax elements (Table A-4) for signaling (number of codewords in each bin).

- If sum of the number of codewords in each bin exceed 1024, normalize it to [0 1023].

- For each bin, divide bin assignment by 32 to find codeword mapping for each codeword CW[Y].

- Construct FwdLUT by integral all codeword assignment. FwdLUT(0) = 0; maxY=1023; for Y = 0: maxY-1 FwdLUT[Y+1] = Clip3(0, maxY, (round) (FwdLUT[Y] + CW[Y]));

- InvLUT is derived by mapping FwdLUT.

Table 2. Derivation process of adaptive SDR reshaping curve (decoder)

- Parse syntax elements (Table A-4) and calculate number of codewords in each bin.

- If sum of the number of codewords in each bin exceed 1024, normalize it to [0 1023].

- For each bin, divide bin assignment by 32 to find codeword mapping for each codeword CW[Y].

- Construct FwdLUT by integral all codeword assignment. FwdLUT(0) = 0; maxY=1023; for Y = 0: maxY-1 FwdLUT[Y+1] = Clip3(0, maxY, (round) (FwdLUT[Y] + CW[Y]));

- InvLUT is derived by mapping FwdLUT.

For chroma coding, luma-based chroma QP Offset is derived using codewords assignment: FwdLUT’(Y) = (CW[k]/32) when Y belongs to the k-th bin, assuming piecewise linear model. Because CW[k] is limited to [1 64], we can precompute cQPO LUT using equation 1. Then the chroma residue scaling factor is calculated accordingly from cQPO.

## Encoder optimization

### Mode decision

If luma reshaper is applied to the current CTU for an inter slice, weighted SSE is used for luma in mode decision. For all luma values in each bin, the same weight is used. The weight is derived from codeword assignment in that bin as follows:

w_rsp(k) = (cw_bin(k)/32)^2, w_rsp(k) is weight for k-th bin.

For intra slice, SSE is used for luma in mode decision.

### Slice Adaptation

In-loop reshaping allows slice level and CTU level adaptation. It therefore allows flexible rate-distortion optimization between slices and CTUs. In CE12-4, slice adaption is automatically enabled. The encoder automatically decides from picture analysis statistics if reshaping should be enabled or disabled for current slice.

# Simulation Results

The CE tests are implemented in BMS2.0.1 software with VTM tool settings (JEM_TOOLS = 0) and with JEM-comparable HDR implementation (WCG_EXT=1) to support both HDR and SDR in-loop reshaping. The decoding process is implemented using fixed point integer arithmetic. The test is done by disabling BMS tools and used VTM CTC with AI and RA. SIMD optimization (AVX2) is automatically enabled in run time. The encoding time is measured using cluster. The decoding time is measured using a PC station specified by Table 3.

Table 3. Technical specification of the PC workstation for decoding

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

Table 4 show BDRate for fixed QP test condition for CE12-4.

Table 4. BDRate for in-loop SDR reshaping: AI and RA

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | 2.57% | 0.68% | 108% | 106% |
| Class A2 | -1.53% | 2.39% | 2.48% | 107% | 105% |
| Class B | -0.84% | 2.55% | 2.16% | 105% | 103% |
| Class C | -0.40% | 2.75% | 2.88% | 107% | 105% |
| Class E | -0.24% | 2.50% | 2.17% | 113% | 107% |
| Overall | -0.96% | 2.56% | 2.13% | 107% | 105% |
| Class D | -0.54% | 2.99% | 2.61% | 107% | 105% |
| Class F (optional) | -0.71% | 1.81% | 1.82% | 106% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.69% | 3.31% | 2.22% | 106% | 107% |
| Class A2 | -1.01% | 2.13% | 2.09% | 101% | 105% |
| Class B | -1.53% | 1.52% | 0.64% | 106% | 105% |
| Class C | -1.00% | 1.76% | 2.05% | 111% | 103% |
| Overall | -1.32% | 2.07% | 1.62% | 106% | 105% |
| Class D | -0.48% | 2.14% | 2.58% | 109% | 101% |
| Class F (optional) | -0.92% | 1.19% | 1.33% | 106% | 105% |

## Decoder Complexity Analysis

A strict decoding environment was created by using a dedicated PC (Table 3). Decoding for each rate point in each test case was performed sequentially without other multi-tasking on the PC. Decoding time reported for CE12-4 are drawn from decoding logs from this strict decoding environment. In addition, complexity evaluation are made with conceptual algorithm analysis.

[Note: It was observed that decoding runtime collected using cluster is highly unreliable and thus cannot be solely used to judge the complexity of the proposal.]

All in-loop related operations, such as LUT construction, luma mapping and chroma residue scaling, are all implemented with fixed point arithmetic at the decoder side. The major computational complexity of in-loop reshaping is the mapping for luma samples using the pair of FwdLUT and InvLUT. The pair of reshaping LUTs are implemented using 2-byte (10-bit), 1024 entry integer arrays. The in-loop reshaper performs luma sample mapping in-place, meaning no extra storage buffer is needed for luma samples. For luma mapping in inter slices and chroma residue scaling, the operations are performed on residue signal, which means at decoder side, if cbf=0 (no coefficient), there is no need to perform inverse reshaping and chroma scaling. This can potentially save a lot of operations for low bit rate video decoding.

- Inverse luma reshaping in inter slices: Yr=InvLUTYres+FwdLUTYpred

When cbf_luma = 0, Yres=0, it is simplified as Yr=Ypred, and no mapping is required.

- Inverse chroma residue scaling: CxRes = CxResScale / cScaleYCU

When cbf_cx = 0, CxResScale=CxRes=0. There is no need to calculate and perform scaling.

The in-loop reshaper does not employ any additional SIMD optimization expect for those already in the VTM2.0.1 codebase. The decoding running time increase over VTM2.0.1 is limited to be under 5%, measured on the dedicated PC. In addition, the complexity of in-loop reshaping and its components is mainly determined by picture resolution and will not change if decoder complexity increases due to the enabling of more complex coding tools (e.g. BMS coding tools under test).

# Conclusion

Summary performance gains of CE12-4 for in-loop reshaping of SDR Content under fixed QP CTC compared to VTM2.0.1 SDR Anchor have been reported in this contribution.

- RA BDrate Y/U/V {-1.32%, 2.07%, 1.62%}

- AI BDrate Y/U/V { -0.96%, 2.56%, 2.13%}

Higher gain is achieved for higher resolution video (UHD and HD).

The decoding running time increase is within 5%. The encoding running time increase is within 7%.

It is recommended that the proposed reshaper be adopted as a core coding tool in VVC.

# Reference

- X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April, 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8)”, JVET-K0308, Ljubljana, SI, July, 2018.

- F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen, “SDR In-loop Reshaping”, JVET-K0309, Ljubljana, SI, July, 2018.

- E. Francois, D. Rusanovskyy, P. Yin, “Description of Core Experiment 12 (CE12): mapping functions”, JVET-K1032, Ljubljana, SI, July 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12-2: HDR In-loop Reshaping”, JVET-L0245, Macao, CN, October 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgement

We would like to thank ByteDance and Qualcomm for crosschecking our CE results.

# Annex A: Syntax and Semantics

The reshaping related syntax is added in SPS() (Table A-1), slice header(Table A-2) and CTU(Table A-3) to allow sequence/slice/CTU level adaptation, respectively. The adaptive reshaper model slice_reshaper_model () (Table A-4) is signalled in slice header of each intra slice.

Table A-1 Reshaping syntax in SPS

| SPS() | Descriptor |
| --- | --- |
| …… |  |
| sps_reshaper_enable_flag /*1: reshaping on, else off */ | u(1) |
| if (sps_reshaper_enable_flag) { |  |
| sps_reshaper_signal_type /* 0:SDR, 1:PQ, 2:HLG*/ | u(2) |
| sps_reshaper_chromaAdj_flag | u(1) |
| sps_reshaper_model_present_flag | u(1) |
| if (sps_reshaper_model_present_flag) |  |
| sps_reshaper_model () |  |
| } |  |

sps_reshaper_enable_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type equal to 0 specifies SDR (gamma) transfer characteristics; sps_reshaper_signal_type equal to 1 specifies PQ transfer characteristics; sps_reshaper_signal_type equal to 2 specifies HLG transfer characteristics; and sps_reshaper_signal_type equal to 3 is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved value of sps_reshaper_signal_type.  When sps_reshaper_signal_type is not present, it is inferred to be equal to 0.

sps_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is used in CVS. sps_reshaper_chromaAdj equal to 0 specifies that chroma residue scaling is not used in CVS. When sps_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

sps_reshaper_model_present_flag equal to 1 specifies sps_reshaper_model() is present in SPS. sps_reshaper_model_present_flag equal to 0 specifies sps_reshaper_model() is not present in SPS. When sps_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

Table A-2 Reshaping syntax in slice header

| slice_header() | Descriptor |
| --- | --- |
| …… |  |
| if ( sps_reshaper_enable_flag ) { |  |
| slice_reshaper_model_present_flag | u(1) |
| if ( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if ( slice_reshaper_enable_flag) { |  |
| if ( sps_reshaper_chromaAdj_flag ) |  |
| slice_reshaper_chromaAdj_flag | u(1) |
| slice_reshaper_CTU_control_flag | u(1) |
| } |  |
| } |  |
| …… |  |

slice_reshaper_model_present_flag equal to 1 specifies slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 specifies slice_reshaper_model() is not present in slice header. When slice_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

slice_reshaper_enable_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enable_flag equal to 0 specifies that reshaper is not enabled for the current slice. When slice_reshaper_enable_flag is not present, it is inferred to be equal to 0.

slce_reshaper_CTU_control_flag equal to 1 specifies that reshaper adaption is allowed for each CTU in current slice. slice_reshaper_CTU_control_flag equal to 0 specifies that reshaper is not allowed to be adapted for each CTU in current slice. When slice_reshaper_CUT_control_flag is not present, it is inferred to be equal to 0.

Table A-3. Reshaping syntax in CTU

| coding_tree_unit() | Descriptor |
| --- | --- |
| …… |  |
| if ( slice_reshape_CTU_control_flag ) { |  |
| reshaper_CTU_flag | ae(v) |
| } |  |
| …… |  |

reshaper_CTU_flag equal to 1 specifies that reshaper is used for the current CTU. reshaper_CUT_flag equal to 0 specifies that reshaper is not used for the current CTU. When reshaper_CTU_flag is not present, it is inferred to equal to slice_reshaper_enabled_flag.

slice_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is enabled for the current slice. slice_reshaper_chromaAdj_flag equal to 0 specifies that chroma residue scaling is not enabled for the current slice. When slice_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

Table A-4. Reshaper model syntax in slice header

| slice_reshaping_model() { | Descriptor |
| --- | --- |
| reshape_model_profile_type | u(4) |
| reshape_model_min_bin_idx | ue(v) |
| reshape_model_delta_max_bin_idx | ue(v) |
| reshape_model_num_cw_minus1 | ue(v) |
| for ( i = 0; i < reshape_model_num_cw_minus1 + 1; i++) { |  |
| reshape_model_delta_abs_CW [ i ] | u(5) |
| if ( reshape_model_delta_abs_CW > 0 ) |  |
| reshape_model_delta_sign_CW_flag [ i ] | u(1) |
| } |  |
| for ( i = reshape_model_min_bin_idx; i <= reshape_model_max_bin_idx; i++ ) { |  |  |
| reshape_model_bin_profile_delta [ i ] | ue(v) |
| } |  |
| } |  |

reshape_model_profile_type specifies the profile type to be used in the reshaper construction process.

reshape_model_min_bin_idx specifies the minimum bin index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to 31, inclusive.

reshape_model_delta_max_bin_idx is set equal to the maximum allowed bin index (31) minus the maximum bin index to be used in the reshaper construction process.

reshape_model_num_cw_minus1 plus 1 specifies the number of codewords to be signalled.

reshape_model_delta_abs_CW [ i ] specifies the i-th absolute delta codeword value.

reshape_model_delta_sign_CW_flag [ i ] specifies the sign of the i-th delta codeword as follows:

- If reshape_model_delta_sign_CW_flag [ i ] is equal to 0, the corresponding variable RspModelDeltaCW [ i ] has a positive value.

- Otherwise (reshape_model_delta_sign_CW_flag [ i ]  is equal to 0), the corresponding variable RspModelDeltaCW [ i ] has a negative value.

When reshape_model_delta_sign_CW_flag [ i ] is not present, it is inferred to be equal to 0.

The variable RspModelDeltaCW[i] = (1 - 2* reshape_model_delta_sign_CW [i]) * reshape_model_delta_abs_CW [ i ];

The variable OrgCW is set equal to 1 << ( BitDepthY – 5 );

The variable RspModelCW [ i ] = OrgCW + RspModelDeltaCW [ i ].

reshape_model_bin_profile_delta [ i ]  specifies the delta value to be used to adjust the profile of the i-th bin in the reshaper construction process.

The variable RspCW[i] for ith bin is derived as following:

- if reshape_model_bin_profile_delta [ i ]  is equal to 0, RspCW [i] = OrgCW + 1.

- Otherwise, CW [i] = RspModelCW[ j-1 ] when  reshape_model_bin_profile_delta [ i ] is equal to j.