| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0249-v1 |
| --- | --- |

| Title: | Picture order count for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes that picture order count (POC) as it is in HEVC is adopted into the VVC draft. It is claimed that deriving relative output distances is vital for deriving motion vector predictors by scaling and the proponents claimed that POC has proven to be a well tested provider of that functionality.

It is proposed that syntax and semantics for two code words are added:

- The signaling of POC LSB in the slice header (slice_pic_order_cnt_lsb)

- The signaling of the number of bits to use for the POC LSB syntax element (log2_max_pic_order_cnt_lsb_minus4) in the SPS.

It is also proposed that a process for deriving PicOrderCntVal is added. The proposed text is based on the text for picture order count in HEVC v5 with a minimal (according to the proponents) set of adaptations for VVC:

- POC LSB is proposed to not be signaled for IRAP_NUT slice headers but inferred to be equal to 0. In HEVC, POC LSB is not signaled but inferred to be equal to 0 for IRAP NUTs (IDR_W_RADL and IDR_N_LP)

- POC MSB is proposed to be calculated relative to the previous reference picture in decoding order with TemporalId equal to 0. In HEVC, it is calculated relative to "the previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL, RADL or SLNR picture."

- There are also minor editorial changes relative to the HEVC text due to elements in HEVC not present in the VVC draft. The POC text in HEVC uses various NAL unit types, NoRaslOutputFlag and refers to Annex C which is not used in the the VVC draft.

# Introduction

The picture order count (POC) concept is a well-known concept used in HEVC to identify pictures, derive motion parameters and check decoder conformance. This contribution proposes that POC is adopted into the VVC draft. The proposed text is based on the text for picture order count in HEVC v5. The POC mechanisms in HEVC are essentially identical to the picture order count text for POC type 0 in the AVC specification.

The relative output distance between pictures is heavily used in VVC for derivation of AMVP motion vector predictors. For example, when a neighboring block uses a different reference picture than the current block, the motion vector of the neighboring block is scaled according to the POC distances as shown in the figure below. A similar motion vector scaling mechanism is also used in ATMVP.

Figure 1 – A current block C using an MVB referencing picture POC-1, MVB being predicted by a scaled motion vector MVA used by block A referencing picture POC-2.

Relative output distances were used already in the H.263 video coding specification in e.g. Annex G “PB-frames mode”. There, a temporal reference (TR) is signaled for each picture and scaling of motion vectors according to TR values is done for calculating vectors for the B-picture in a PB-frame. TR is also included into the H.261 video coding specification.

We propose that syntax and semantics for log2_max_pic_order_cnt_lsb_minus4 and slice_pic_order_cnt_lsb are added to the SPS and slice header respectively. We also propose that a process for deriving a PicOrderCntVal for each picture is added.

# Proposal

There are two syntax elements for POC signaling in HEVC. There is the actual signaling of POC LSB in the slice segment header (slice_pic_order_cnt_lsb) and the signaling of the number of bits to use for that syntax element (log2_max_pic_order_cnt_lsb_minus4) in the SPS.

The AVC text for POC type 0 for frame coding and the HEVC text for POC are very similar. The only differences between the proposed text, the AVC text and the HEVC text, according to the proponents, are as follows:

- In this proposal, POC LSB is proposed to not be signaled for IRAP_NUT slice headers but inferred to be equal to 0

  - In HEVC, it is not signaled but inferred to be equal to 0 for IRAP NUTs (IDR_W_RADL and IDR_N_LP)

  - In AVC, it is always signaled when POC type is 0, but 0 must be signaled in slice headers for IDR pictures when e.g. the Baseline profile is used.

- In this proposal, POC MSB is proposed to be calculated relative to the previous reference picture in decoding order with TemporalId equal to 0

  - In HEVC, it is calculated relative to "the previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL, RADL or SLNR picture."

  - In AVC, it is calculated relative the previous reference picture in decoding order (note that not all profiles in HEVC support temporal layers)

- Editorial differences (substantial editorial changes relative to the AVC text, minor editorial changes relative to the HEVC text)

## Proposed syntax and semantics relative to HEVC

The changes shown here is relative to HEVC to make it clear what the differences are between this proposal and the HEVC specification. Proposed removed text is shown by strikethrough red and proposed added text is shown by underline red text. There are two differences: 1) adaptation to the single NAL unit type for IRAP used in VVC and 2) removal of the HEVC reference to the generation process for unavailable reference pictures (which we do not propose to include into the VVC text).

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| ... |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nal_unit_type != IRAP_NUT IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| ... |  |

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7-8)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

slice_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When slice_pic_order_cnt_lsb is not present, slice_pic_order_cnt_lsb is inferred to be equal to 0, except as specified in clause 8.3.3.1.

## Proposed decoding process relative to HEVC

### Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see clause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL, RADL or SLNR picture.

- The variable prevPicOrderCntLsb is set equal to slice_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (slice_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( ( slice_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(8-2)

NOTE 1 – All IDR IRAP pictures will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR IRAP pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive.

In one CVS, the PicOrderCntVal values for any two coded pictures shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same CVS, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).