| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0268-v1 |
| --- | --- |

| Title: | CE1: Zero-Unit with uniform paring process (Test 1.2.1 and Test 1.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane, San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, Zero-Unit (ZU) is proposed to handle the coding structure at picture boundaries. A ZU is defined as a special Coding Unit (CU) with either width or height not in a form of 2N. Same as normal CU, a ZU can be split by BT, TT or QT and the parsing process is also kept unchanged. Two tests are conducted in this contribution. In Test 1.2.1, cbf_flag of a ZU must be signaled to be 0 with an encoder restriction. In test 1.2.2, a ZU must be further split with an encoder restriction. Simulation results reportedly show 0.06%, 0.00% and -0.10% BD-rate changing in test 1.2.1, 0.07%, 0.12% and 0.20% BD-rate changing in test 1.2.2, on the Y component for All Intra (AI), Random Access (RA) and LB (Low-delay B) configurations in average, respectively, compared with VTM-2.0.1 with almost the same encoding and decoding time.

# Introduction

In VTM, a quadtree (QT)/binary tree (BT)/ternary tree (TT) coding structure is applied. The width or height of a Coding Unit (CU) is always in a form of 2N. After a quarter-, ternary-, or binary- split, a CU can be partitioned into four, three, or two sub-CUs, the widths or heights of which are still in a form of 2N. For Coding Tree Units (CTUs) at picture boundaries, the width or height of the CTU inside the picture may not be in the form of 2N. To handle this case, VTM applies a boundary-aware parsing process. A CTU at boundary is parsed in a different way compared with a CTU not at boundary. Different available splitting methods are defined for different boundary cases.

To unify the parsing process, Zero-Unit (ZU) is proposed with a boundary-blind parsing process.

# Zero-Unit

A ZU is defined as a special Coding Unit (CU) with either width or height not in a form of 2N. A ZU can also be BT, QTor TT into sub-blocks, which can be a ZU or a normal CU.

Suppose the size of a ZU is W×H, then {w0, w1, h0, h1} are calculated as:

w0=2N, w1=W- w0; where 2N <W<=2N+1;

h0=2M, h1=W- h0; where 2M <H<=2M+1.

If a ZU is split by QT, the four sub-blocks are with sizes w0× h0, w1× h0, w0× h1 and w1× h1, respectively. If a ZU is split by vertical BT, the two sub-blocks are with sizes w0× H and w1×H, respectively. In addition, a ZU can be split by horizontal BT, and the two sub-blocks are with sizes W× h0 and W× h1, respectively. It should be noted that a sub-block may be a ZU or normal CU depending on whether both its width and height are in the form of 2N.

Fig. 1 shows an example of how a ZU with size 64×48 can be split. By QT, it is split into two normal CUs with the size 32×32 and two normal CUs with the size 32×16, as shown in Fig. 1 (a). By horizontal BT, it is split into one normal CU with the size 64×32 and the other with the size 64×16, as shown in Fig. 1 (b).  By vertical BT, it is split into two ZUs with the size 32×48 as shown in Fig. 1 (c).

A ZU can also be split by TT. {w0, w1, w2, h0, h1, h2} are calculated as:

w0=2N, w1=2N+1, w2=W- w0- w1; where 3*2N <W<=3*2N+1;

h0=2N, h1=2N+1, h2=H- h0- h1; where 3*2N <H<=3*2N+1;

If a ZU is split by vertical TT, the three sub-blocks are with sizes w0× H, w1× H and w1×H, respectively. If a ZU is split by horizontal TT, and the three sub-blocks are with sizes W× h0, W× h1 and W× h2, respectively.

Fig. 2(a) shows an example of how a ZU with size 64×48 is split by horizontal TT. It is split into two normal CUs with the sizes 64×8 and 64×16, and one ZU with the size 64×24. By vertical TT, it is split into three ZUs with the sizes 16×48, 32×48 and 16×48, as shown in Fig. 2 (b).

(a)

(b)

(c)

Fig. 1 Examples of ZU QT/BT split.

(a)

(b)

Fig. 2 Examples of TT split.

# Experimental Results

The proposed method is implemented on VTM-2.0.1. The simulations were performed following the JVET CTC [1]. Two tests are conducted. In Test 1.2.1, cbf_flag of a ZU must be signaled to be 0 as an encoder restriction. In test 1.2.2, a ZU must be split as an encoder restriction. Results of test 1.2.1 and test 1.2.2 are summarized in Table 1 and Table 2 respectively. More detail results can be found in the attached spreadsheets.

Table 1: Compared with VTM-2.0.1 (Test 1)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.06% | 100% | 104% | -0.05% | -0.03% | -0.09% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.06% | 0.01% | 100% | 102% | -0.01% | 0.01% | -0.15% | 100% | 99% |  |  |  |  |  |
| Class B | 0.03% | 0.08% | 0.07% | 99% | 100% | -0.05% | -0.06% | 0.11% | 101% | 100% | -0.01% | -0.17% | -0.41% | 101% | 102% |
| Class C | 0.12% | 0.23% | 0.33% | 95% | 97% | 0.10% | 0.16% | 0.31% | 98% | 97% | 0.09% | 0.31% | 0.11% | 97% | 95% |
| Class E | 0.14% | 0.02% | 0.07% | 99% | 101% |  |  |  |  |  | -0.50% | -0.19% | -0.51% | 99% | 91% |
| Overall | 0.06% | 0.10% | 0.12% | 98% | 101% | 0.00% | 0.02% | 0.07% | 100% | 99% | -0.10% | -0.02% | -0.27% | 99% | 97% |

Table 1: Compared with VTM-2.0.1 (Test 2)

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.08% | 0.00% | 102% | 105% | 0.11% | 0.14% | 0.09% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.02% | 0.06% | 0.03% | 100% | 100% | 0.07% | 0.14% | 0.00% | 101% | 99% |  |  |  |  |  |
| Class B | 0.06% | 0.05% | 0.06% | 100% | 100% | 0.13% | 0.15% | 0.22% | 100% | 99% | 0.16% | 0.08% | 0.10% | 100% | 100% |
| Class C | 0.12% | 0.23% | 0.33% | 96% | 98% | 0.14% | 0.23% | 0.34% | 98% | 100% | 0.15% | 0.02% | 0.01% | 97% | 94% |
| Class E | 0.14% | 0.02% | 0.07% | 100% | 98% |  |  |  |  |  | 0.33% | 0.27% | 0.13% | 99% | 94% |
| Overall | 0.07% | 0.09% | 0.11% | 100% | 100% | 0.12% | 0.17% | 0.18% | 100% | 100% | 0.20% | 0.10% | 0.07% | 99% | 96% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, Jul. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).