| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0272-v2 |
| --- | --- |

| Title: | CE3-related: Modified chroma derived mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM2.0, separate block partitioning structure for luma and chroma components is enabled in I slices. Therefore, one chroma coding block may correspond to multiple luma coding blocks. When deriving the chroma derived mode (DM), the intra prediction mode of the luma coding block containing the top-left luma sample in the corresponding luma block is utilized. It is proposed to use the intra prediction mode of the luma coding block covering the center position of the corresponding luma block as chroma DM. Simulation results reportedly show that the proposed method achieves 0.11% BD rate reduction under All Intra Main10 configuration.

# Introduction

In Versatile Video Coding (VVC), separate block partitioning structure for luma and chroma components is enabled in I slices. That is, for the same coding tree unit, different partitioning structures may be selected for luma and chroma, respectively. Therefore, one chroma coding block may correspond to multiple luma coding blocks. When deriving the chroma derived mode (DM), the intra prediction mode of the luma coding block covering the top-left position of the corresponding luma block is utilized. An example was shown in Figure 1 with the co-located position denoted by TL.

Figure 1 Example of one chroma block and its corresponding luma block [1]

However, the correlation between one chroma block and the luma block covering the center position of the corresponding luma block may be even higher compared to that covering the top-left position.

# Proposed method

When deriving the chroma derived mode (DM), the intra prediction mode of the luma coding block covering the center position of the corresponding luma block is utilized.

Figure 2. Proposed position for DM derivation from the corresponding luma block.

Modified decoding process on top of JVET-K1001 version 6 are described as follow. The deleted parts are marked with red and strikethrough while the newly added parts are highlighted in yellow.

### 8.2.1	General decoding process for coding units coded in intra prediction mode

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for chroma samples is specified as follows:

- The derivation process for the chroma intra prediction mode as specified in clause 8.2.3 is invoked with the luma location ( xCb, yCb ), the variable nTbW set equal to ( cbWidth / 2 ), the variable nTbH set equal to ( cbHeight / 2 ) as inputs.

…

### 8.2.3	Derivation process for chroma intra prediction mode

Input to this process areis:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture.

- a variable nTbW specifying the width of the current chroma transform block,

- a variable nTbH specifying the height of the current chroma transform block,

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + nTbW ][ yCb + nTbH ] as specified in Table 8-2 – and Table 8-3.

…

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + nTbW ][ yCb + nTbW ] when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + nTbW ][ yCb + nTbW ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + nTbW  ][ yCb + nTbW  ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + nTbW ][ yCb + nTbW ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 77 | 77 | 77 | 77 | 77 |
| 5 | 0 | 50 | 18 | 1 | X |

# Simulation results

The proposed method has been implemented on top of reference software VTM-2.0.1 [2]. To evaluate its coding efficiency, simulations are conducted following the common test conditions [3]. Table 1 shows the BD-rate gain and processing time impact of the proposed approach.

Table 1. Performance of proposed DM under All Intra Main 10

|  |  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U | V | EncT | DecT |
| Class A1 |  | -0.06% |  | -0.11% | -0.07% | 100% | 100% |
| Class A2 |  | -0.10% |  | -0.06% | -0.13% | 100% | 101% |
| Class B |  | -0.07% |  | -0.09% | -0.08% | 100% | 100% |
| Class C |  | -0.06% |  | -0.12% | 0.09% | 99% | 98% |
| Class E |  | -0.28% |  | -0.02% | -0.49% | 98% | 96% |
| Overall |  | -0.11% |  | -0.08% | -0.12% | 99% | 99% |
| Class D |  | -0.07% |  | 0.17% | -0.22% | 100% | 100% |
| Class F |  | -0.04% |  | -0.16% | 0.00% | 98% | 94% |

# Conclusions

In this contribution, location of the luma block used by DM generation is modified from that covering the top-left position to the center position of corresponding luma block. Simulation results demonstrate there is 0.11% luma coding gain under all intra configuration. It is suggested to adopt this to VVC.

# Reference

- “JVET common test conditions and software reference configurations”, L. Zhang, W.-J. Chien, J. Chen, X. Zhao, M. Karczewicz, JVET-D0111, Oct. 2016.

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).