| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0278 |
| --- | --- |

| Title: | CE4.2.5: Simplification of affine merge list construction and move ATMVP to affine merge list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Yu Han and Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution report the results of CE4.2.5: simplification of affine merge list construction and move ATMVP to affine merge list. Compares to the CE4.2.1 affine merge common base, the contributions are summarized as follows:

- Pruning of inherited affine merge candidate by checking PU index instead of CPMVs comparison.

- Remove pruning for constructed affine merge candidate.

- No MV scaling operations for affine merge candidate.

- Move the ATMVP candidate from normal merge list to the affine merge list

In test a, 1) and 2) are applied, the average bd-rates when compares to VTM2.0.1 are -0.72%, -0.55%, -0.73%  in RA and -0.33%, -0.20%, -0.37% in LB. In test b, all the 3 items are applied, the average bd-rates when compares to VTM2.0.1 are -0.76%, -0.64%, -0.76% in RA, and -0.53%, -0.53%, -0.69% in LB.

# Introduction

In VTM2.0.1, only one affine merge candidate is used. The affine merge candidate is the first available inherited affine merge candidate from the blocks A1, B1, B0, A0 and B2 , as shown in Fig. 1.

Fig.  1

In the common base for CE4.2 [1], the affine merge is improved by adding more inherited affine merge candidates and constructed affine merge candidates. The number of potential inherited affine merge candidate is 5, and they are check from the blocks A1, B1, B0, A0 and B2 in order. The number of potential constructed affine merge canidates is 10. The constructed affine candiadtes are derived by different patterns of control-point MVs selection.

In order to remove the duplicate affine merge candidates, full pruning is applied when inserting a candidate. Besides, when deriving the constructed affine merge candidate, MV scaling is performed if the refenrece pictures of 3 MVs are different.

# Proposal

In this contribution, it’s proposed to prune the inherited affine candidate by checking PU index, and remove the MV scaling operations and pruning process for the constructed candidate to simplify the affine merge list construction. And it’s also proposed to move ATMVP candidate from normal merge list to affine merge list, such that the sub-PU base merge candidates are in one separate merge list.

## Test a: simplification of affine common base

In test a, when checking a neighboring affine coded block for inherited affine merge candidate, the PU index of the neighboring block is compared with previous checked neighboring block. If it’s not the same PU as in any of the previously checked neighboring block, the derived inherited affine merge candidate is directly added to the candidate list without any comparison with the existing candidates. When deriving the constructed affine merge candidate, the reference picture of all the control-point MVs should be the same, otherwise the constructed affine merge candidate is considered as unavailable. The constructed affine merge candidate is also directly inserted into the list without pruning.

## Test b: move ATMVP candidate to affine merge list

Test b is based on test a. The ATMVP merge candidate in the normal merge list is moved to the first position of the affine merge list.

# Test results

The simulations were performed under the common test condition, and the random access (RA) and low delay B (LB) results are reported. Test a result is shown in Table 1 and test b result is shown in Table 2. The overall average bd-rate for test a are -0.72%, -0.55%, -0.73%  in RA and -0.33%, -0.20%, -0.37% in LB.  The overall average bd-rate for test b are -0.76%, -0.64%, -0.76% in RA, and -0.53%, -0.53%, -0.69% in LB..

Table 1: Test a result

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.59% | -0.90% | 109% | 123% |
| Class A2 | -0.96% | -1.00% | -1.08% | 106% | 105% |
| Class B | -0.65% | -0.46% | -0.56% | 105% | 105% |
| Class C | -0.46% | -0.31% | -0.55% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.55% | -0.73% | 106% | 108% |
| Class D | -0.58% | -0.55% | -0.62% | 104% | 105% |
| Class F (optional) | -0.12% | -0.13% | -0.09% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.28% | -0.49% | 114% | 126% |
| Class C | -0.23% | -0.16% | -0.27% | 105% | 100% |
| Class E | -0.46% | -0.10% | -0.30% | 105% | 102% |
| Overall | -0.33% | -0.20% | -0.37% | 109% | 111% |
| Class D | -0.31% | 0.33% | -0.48% | 104% | 101% |
| Class F (optional) | -0.14% | 0.33% | 0.07% | 103% | 98% |

Table 2: Test b result

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.63% | -0.92% | 109% | 124% |
| Class A2 | -0.98% | -0.90% | -1.08% | 105% | 110% |
| Class B | -0.69% | -0.61% | -0.63% | 102% | 105% |
| Class C | -0.54% | -0.49% | -0.56% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.76% | -0.64% | -0.76% | 104% | 109% |
| Class D | -0.72% | -0.64% | -0.77% | 102% | 102% |
| Class F (optional) | -0.18% | -0.18% | -0.17% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.51% | -0.53% | 113% | 127% |
| Class C | -0.29% | -0.34% | -0.29% | 105% | 105% |
| Class E | -1.17% | -0.83% | -1.49% | 105% | 113% |
| Overall | -0.53% | -0.53% | -0.69% | 108% | 116% |
| Class D | -0.33% | 0.14% | 0.07% | 102% | 101% |
| Class F (optional) | -0.10% | -0.20% | 0.45% | 104% | 105% |

# Conclusion

In this contribution, the CE4.2.5 results are reported. It’s suggested to adopt test b, wherein the ATMVP merge candidate is moved to the affine merge list.

# References

[1]. H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Documents. Joint Video Experts Team (JVET)  JVET-K1024, Ljubljana, SI, 10–18 July 2018

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).