Specification changes related to current CTU CPR mode and integer chroma vector

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_mts_inter_enabled_flag | u(1) |
| sps_cpr_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.3.1 Sequence parameter set RBSP semantics

……

sps_cpr_enabled_flag equals to 1 specifies that current picture referencing may be used in decoding of pictures in the CVS. sps_cpr_enabled_flag equal to 0 specifies that current picture referencing is not used in the CVS. When sps_cpr_enabled_flag is not present, it is inferred to be equal to 0. When sps_cpr_enabled_flag is equal to 1, the current picture is used as a reference picture and is put at the last position in the reference picture list 0.

……

qtbtt_dual_tree_intra_flag equal to 1 specifies that for I slices, or for P slices where the current picture is the only reference picture for the slice, each CTU is split into coding units with 64x64 luma samples using an implicit quadtree split and that these coding units are the root of two separate coding_quadtree syntax structure for luma and chroma.

#### 7.4.4.1 General slice header semantics

……

The variable CurrPicIsOnlyRef, specifying if the current decoded picture is the only reference picture for the current slice. It is set to be sps_cpr_enabled_flag && ( slice_type = = P ) && ( num_ref_idx_l0_active_minus1 = = 0 ).

#### 7.3.4.2 Coding tree unit syntax

| if( ( slice_type = = I ¦ ¦ CurrPicIsOnlyRef ) && qtbtt_dual_tree_intra_flag ) { |  |
| --- | --- |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| Else |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0, SINGLE_TREE ) |  |

#### 7.3.4.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| if( treeType ! = DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| …… |  |
| } else if( treeType ! = DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| …… |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| if ( !sps_cpr_enabled_flag ¦ ¦ !( inter_pred_idc[ x0 ][ y0 ] = = PRED_L0 && ref_idx_l0[ x0 ][ y0 ] = = num_ref_idx_l0_active_minus1 )) |  |
| amvr_flag[ x0 ][ y0 ] | ae(v) |
| if ( amvr_flag[ x0 ][ y0 ] ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

#### 7.4.5.5 Coding unit semantics

……

MotionModelIdc[ x ][ y ] represents motion model of a coding unit as illustrated in Table 7-5. The array indices x, y specify the luma sample location ( x, y ) relative to the top-left luma sample of the picture.

……

It is a requirement of bitstream conformance that when the current picture is a the reference picture for the current coding unit, the MotionModelIdc[ x ][ y ]shall be equal to 0.

……

amvr_flag[ x0 ][ y0 ] specifies the resolution of motion vector difference. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When amvr_flag[ x0 ][ y0 ] is equal to 0, the resolution of motion vector difference is 1/4 fractional-pel; otherwise, when amvr_flag[ x0 ][ y0 ] is equal to 1, the resolution of motion vector difference is not fractional-pel.

When amvr_flag[ x0 ][ y0 ] is not present, if sps_cpr_enabled_flag  is equal to 1, it is inferred to be equal to 1; otherwise, if sps_cpr_enabled_flag is equal to 0, it is inferred to be equal to 0.

amvr_mode[ x0 ][ y0 ] specifies the resolution of motion vector difference. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When amvr_flag[ x0 ][ y0 ] is equal to 1,

- If amvr_mode[ x0 ][ y0 ] is equal to 0, the resolution of motion vector difference is one integer-pel;

- Otherwise, if amvr_mode[ x0 ][ y0 ] is equal to 1, the resolution of motion vector difference is four integer-pel.

When amvr_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The variable MvShift is set equal to ( amvr_flag[ x0 ][ y0 ] + amvr_mode[ x0 ][ y0 ] ) << 1 and the variables MvdL0[ x0 ][ y0 ][ 0 ], MvdL0[ x0 ][ y0 ][ 1 ], MvdL1[ x0 ][ y0 ][ 0 ], MvdL1[ x0 ][ y0 ][ 1 ] are modified as follows:

……

## 8.3 Decoding process for coding units coded in inter prediction mode

### 8.3.1 General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- ……

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If MotionModelIdc[ xCb ][ yCb ] is equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked:

      - If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, with the treeType, luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ], the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 as outputs.

      - If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, with the treeType, luma coding block location ( xCb, yCb ), the chroma coding block width cbWidth in luma samples, and the chroma coding block height cbHeight in luma samples as inputs, and the number of chroma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 as outputs.

- Otherwise ( MotionModelIdc[ xCb ][ yCb ] is equal to 1 or 2), the following applies:

- The derivation process for affine motion vector components and reference indices as specified in clause 8.3.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight as inputs, and the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ], and the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..(cbWidth >> 2) − 1, and ySbIdx = 0..( cbHeight >> 2 ) − 1, with X being 0 or 1 as outputs.

- The prediction samples of the current coding unit are derived as follows:

- The variables numSbX and numSbY are modified as follows

numSbX  =  ( MotionModelIdc[ xCb ][ yCb ]  =  =  0 )  ?  numSbX  :  ( cbWidth >> 2 )	(8-99

numSbY  =  ( MotionModelIdc[ xCb ][ yCb ]  =  =  0 )  ?  numSbY  :  ( cbHeight >> 2 )	(8-100

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- The decoding process for inter blocks as specified in clause 8.3.4.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ], the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples and two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- If treeType is equal to DUAL_TREE_CHROMA, the reference indices refIdxL0 is set to 0; and refIdxL1 is set to -1; the prediction list utilization flags predFlagL0 is set to 1 and predFlagL1 is set to 0.

- The decoding process for inter blocks as specified in clause 8.3.4.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- The residual samples of the current coding unit are derived as follows:

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdx as inputs, and the array resSamplesL as output.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdx as inputs, and the array resSamplesCb as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdx as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

### 8.3.2 Derivation process for motion vector components and reference indices

#### 8.3.2.1 General

Inputs to this process are:

- ……

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- the chroma motion vectors in 1/32 fractional-sample accuracy mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

Let the variable LX be RefPicListX, with X being 0 or 1, of the current picture.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.3.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ], mvL1[ xSbIdx ][ ySbIdx ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

- Otherwise, if treeType is equal to DUAL_TREE_CHROMA, The variables numSbX and numSbY are derived as follows:

numSbX = cbWidth >> 2 		(8-101)

numSbY = cbHeight >> 2 		(8-102)

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following ordered steps apply:

……

It is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ]  >>  4 ), yCb + ( mvLX[ 1 ]  >>  4 )  ) as inputs, and the output the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ]  >>  4 ) + cbWidth – 1, yCb + ( mvLX[ 1]  >>  4 ) + cbHeight − 1 ) as inputs, and the output the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  4 ) + cbWidth is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  4 ) + cbHeight is less than or equal to 0.

- The following conditions shall be true:

  - ( yCb + ( mvLX[ 1 ]  >>  4 ) ) >>  CtbLog2SizeY = yCb >>  CtbLog2SizeY

  - ( yCb + ( mvLX[ 1 ]  >>  4 )  + cbHeight − 1) >>  CtbLog2SizeY = yCb >>  CtbLog2SizeY

  - ( xCb + ( mvLX[ 0 ]  >>  4 ) ) >>  CtbLog2SizeY = xCb >>  CtbLog2SizeY

  - ( xCb + ( mvLX[ 0 ]  >>  4 )  + cbWidth − 1) >>  CtbLog2SizeY = xCb >>  CtbLog2SizeY

If CuPredMode[ xCb + xSbIdx*4 ][ yCb + ySbIdx*4 ] for luma coding blocks is equal to MODE_INTRA, mvLX[ xSbIdx ][ ySbIdx ][0] and mvLX[ xSbIdx ][ ySbIdx ][1] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY – 1, are set to be 0.

When predFlagLX, with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] from collocated luma samples and refIdxLX as input, and the output being mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, it is a requirement of bitstream conformance that when the reference picture is the current picture, the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 shall obey the following constraints:

- One or both the following conditions shall be true:

- The value of ( mvCLX[ 0 ]  >>  5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- The value of ( mvCLX[ 1 ]  >>  5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

#### 8.3.2.11 Derivation process for temporal luma motion vector prediction

……

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If slice_temporal_mvp_enabled_flag is equal to 0, or if the reference picture is the current picture, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (slice_temporal_mvp_enabled_flag is equal to 1 and the reference picture is not the current picture), the following ordered steps apply:

……

### 8.3.4 Decoding process for inter blocks

#### 8.3.4.1 General

This process is invoked when decoding coding unit whose CuPredMode[ xCb ][ yCb ] is not equal to MODE_INTRA.

Inputs to this process are:

- ……

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

- ……

Outputs of this process are:

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCb of chroma prediction samples for the component Cb,

- an (cbWidth / 2)x(cbHeight / 2) array predSamplesCr of chroma prediction samples for the component Cr.

……

- For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

–	The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.3.4.2 with refIdxLX as input.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

–	The array predSamplesLXL and the arrays predSamplesLXCb and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in clause 8.3.4.3 with the luma location ( xCb, yCb ), the luma coding subblock width sbWidth, the luma coding subblock height sbHeight, the motion vectors mvLX[ xSb ][ ySb ], mvCLX[ xSb ][ ySb ], and the reference arrays refPicLXL, refPicLXCb, and refPicLXCr as inputs.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

–	The arrays predSamplesLXCb and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in clause 8.3.4.3 with the luma location ( xCb, yCb ), the luma coding subblock width sbWidth, the luma coding subblock height sbHeight, the motion vectors mvCLX[ xSb ][ ySb ], and the reference arrays refPicLXCb, and refPicLXCr as inputs.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- The prediction samples inside the current luma coding subblock, predSamplesL[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived by invoking the weighted sample prediction process specified in clause 8.3.4.4 with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 as inputs.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.3.4.4 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.3.4.4 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 2 as inputs.

……

#### 8.3.2.13 Derivation process for chroma motion vectors

……

Input to this process is are a luma motion vector mvLX and the reference indices refIdxLX.

For the derivation of the chroma motion vector mvCLX, the following applies:

- If the reference picture with index refIdxLX from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-237)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-238)

- Otherwise

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 3 + SubWidthC ) ) * 32	(8-239)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 3 + SubHeightC ) ) * 32	(8-240)

#### 8.3.4.3 Fractional sample interpolation process

##### 8.3.4.3.1 General

Inputs to this process are:

- ……

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

……

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- a luma motion vector mvLX given in 1/16-luma-sample units,

- the selected reference picture sample array refPicLXL

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- a chroma motion vector mvCLX given in 1/32-chroma-sample units,

- the selected reference picture sample array refPicLXL and the arrays refPicLXCb and refPicLXCr.

Outputs of this process are:

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- an (sbWidth)x(sbHeight) array predSamplesLXL of prediction luma sample values,

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

- two (sbWidth / 2)x(sbHeight / 2) arrays predSamplesLXCb and predSamplesLXCr of prediction chroma sample values.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, Llet ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb and refPicLXCr.

……

- The prediction luma sample value predSamplesLXL[ xL ][ yL ] is derived by invoking the process specified in clause 8.3.4.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL as inputs.

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, Llet ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLXCb and refPicLXCr.

……

### Binarization process

#### General

Table 9-4-- Syntax elements and associated binarizations

| coding_unit( ) | …… |  |  |
| --- | --- | --- | --- |
|  | amvr_flag[ ][ ] | FL | cMax = 1 |
|  | amvr_mode[ ][ ] | TRFL | cMax = 1cMax = 2, cRiceParam = 0 |
|  | cu_cbf | FL | cMax = 1 |