| vJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0297-v1 |
| --- | --- |

| Title: | CE8-related: CPR mode with local search range optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu (Tencent) Eric Chai (Ubilinx) | Tel: Email: | xiaozhongxu@tencent.com eric.chai@ublnx.com |
| Source: | Tencent, Ubilinx |  |  |

_____________________________

# Abstract

In this contribution, the effective search range for CPR mode with local reference constraints is increased, while it is asserted that the required memory to store reconstructed samples for CPR referencing is kept unchanged. Compared to the constraint with only 1 CTU size of reference area in CE8 test (CE8.3.1b), the proposed method can achieve higher coding efficiency. The reported BD rate changes from the proposed method (reuse the memory buffer on 64x64 basis) relative to VTM-2.0.1 are as follows:

- In AI, -0.27%/-13.61%/-41.31% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.14%/-11.13%/-25.50% for CTC/Class F/SCC 1080p classes, separately.

- In LB, 0.02%/-6.07%/-15.71% for CTC/Class F/SCC 1080p classes, separately.

# Introduction

In CE8.3.1, CPR mode can use reference samples from within the same CTU as the current block. This approach, when implemented in certain platforms, requires a memory buffer with the size as large as 1 CTU of reference samples for storage. This memory buffer is referred as reference sample memory.

However, when processing data from the first CU in a CTU, depending on the coding order and block partitioning structure, not all the storage spaces in the above-mentioned reference sample memory are used to update with new reconstructed samples of the current CTU from the beginning. In fact, one location in the reference sample memory can be used to store the reference samples of the previous code CTU until the current block in the same location is coded, which has the same offset from the top left position of the CTU as compared to that said location in the previous CTU. When the part of reference sample memory has not yet been updated, its samples can be used for CPR compensation. This effectively extends the CPR search range from within the current CTU to some parts of the previous CTU.

# Algorithm

It is firstly to propose not to allow CPR block size to be larger than 64x64 luma samples. The following discussions are based on this proposal.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Fig. 1: current CTU processing order and its available reference samples in current and left CTU (when quad-tree split is enforced at 128x128 level)

Assuming the first level split of the CTU is quad-tree split. For each of four 64x64 blocks, its available reference samples in current CTU and left CTU are different. In Fig. 1, illustrations of reference area for four 64x64 blocks are shown, where the 64x64 block with vertical strips is where the current coding block is located (the coding block can be smaller than 64x64). The shaded blocks are the reconstructed blocks. The ones with “X” mark are not available for reference since they should be used to update with the reference samples from the current CTU in the corresponding locations.

Based on this, the following will apply:

- If current block falls into the top-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the top-right, bottom-left and bottom-right 64x64 blocks of the left CTU, using CPR mode.

- If current block falls into the top-right 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the bottom-left and bottom-right 64x64 blocks of the left CTU, using CPR mode.

- If current block falls into the bottom-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the bottom-right 64x64 blocks of the left CTU, using CPR mode.

- If current block falls into the top-left 64x64 block of the current CTU, it can only refer to the already reconstructed samples in the current CTU, using CPR mode.

The above assumption works for the case that the CTU will be split in quad-tree at first level. For example, when separate luma/chroma coding tree (dual-tree) is used.

Note that the VVC standard allows flexible block partitioning strategies with quad-tree, binary-tree and ternary-tree. If the first level split is not quad-tree, a CTU may be split at the first level by binary-tree split into 128x64 or 64x128 blocks.

For example, when dual-tree is not used. If vertical binary-tree split is applied at the first level from the CTU, then the coding order of 2nd and 3rd of the four 64x64 blocks in Fig. 1 will be different. In Fig. 2, illustrations of the availability of reference samples for a coding block in top-right 64x64 block and bottom-left 64x64 block are shown, when the vertical binary-tree split is applied at 128x128 level from the CTU. When the horizontal binary-tree split is applied at 128x128 level from the CTU, the coding order and availability of reference samples for a coding block in top-right 64x64 block and bottom-left 64x64 block are the same as in Fig. 1.

X

X

Curr

X

X

X

Curr

Fig. 2: current CTU processing order and its available reference samples in current and left CTU when first level vertical binary-tree split is applied.

Based on the above observation, the following search range constraints are proposed:

Proposed method: reuse the reference sample memory on 64x64 basis.

We consider each of the 64x64 blocks in the reference memory buffer as a whole, when some part of the 64x64 block has been updated with reconstructed samples from the current CTU, the reference samples from the left CTU in this whole 64x64 block cannot be used. More specifically, depending on the location of the current coding block relative to the current CTU, the follows apply:

- If current block falls into the top-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the bottom-right 64x64 blocks of the left CTU, using CPR mode. And, if luma location (0, 64) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the bottom-left 64x64 block and bottom-right 64x64 block of the left CTU, using CPR mode. If luma location (64, 0) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the top-right 64x64 block of the left CTU, using CPR mode.

- If current block falls into the top-right 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (0, 64) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the bottom-left 64x64 block and bottom-right 64x64 block of the left CTU, using CPR mode.

- If current block falls into the bottom-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (64, 0) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the top-right 64x64 block and bottom-right 64x64 block of the left CTU, using CPR mode.

- If current block falls into the top-left 64x64 block of the current CTU, it can only refer to the already reconstructed samples in the current CTU, using CPR mode.

In table I, the availabilities of reference samples from left CTU are summarized for method 1. TL, TR, BL and BR refer to top-left, top-right, bottom-left and bottom-right, separately. Mark “X” means not available, mark “Y” means available.

Table I: reference sample availability from left CTU in method

| Ref sample in left CTU Current sample in current CTU | TL 64x64 | TR 64x64 | BL 64x64 | BR 64x64 |
| --- | --- | --- | --- | --- |
| TL 64x64 | X | Y (when (64, 0) location is not reconstructed) /X | Y (when (0, 64) location is not reconstructed) /X | Y |
| TR 64x64 | X | X | Y (when (0, 64) location is not reconstructed) /X | Y (when (0, 64) location is not reconstructed) /X |
| BL 64x64 | X | Y (when (64, 0) location is not reconstructed) /X | X | Y (when (64, 0) location is not reconstructed) /X |
| BR 64x64 | X | X | X | X |

# Simulation results

The simulation is performed using CE8-3 software (BMS-2.1rc1), under VTM configuration. CPR mode is turned on. Common test conditions in CE8 [1] are followed.

Table II: coding performance comparison the proposed method on top of VTM-2.0.1 and CE8-3-1b (1CTU, no chroma interpolation)

# Conclusion

In this contribution, the search range of CPR mode is extended from one CTU (current CTU) to include some parts of its left CTU. By proper design the search range, the required memory to store reconstruction samples for CPR compensation is kept as 1 CTU size of samples, while the coding performance of the proposed method is improved compared to CE8.3.1b.

It is recommended to consider include the proposed search range optimization in this contribution into VTM.

# References

[1] X. Xu, K. Müller, L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-K1028, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ubilinx may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).