| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L326-v3 |
| --- | --- |

| Title: | CE14: Hadamard transform domain filter (Test 3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Dmitry Kuryshev Jianle Chen | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com kuryshev.dmitry@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

The results of CE14 test 3 on Hadamard transform domain filter applied after reconstruction are reported.

- For test CE14-3a with LUT size 140 bytes the reported luma BD-rates over BMS-2.0.1 with VTM configuration are: -0.48%/-0.70%/-0.68% with 9%/5%/4% encoding time and 10%/4%/4% of decoding time increase for AI/RA/LDB configurations correspondingly;

- For test CE14-3b with LUT size 70 bytes the reported luma BD-rates over BMS-2.0.1 with VTM configuration are: -0.47%/-0.70%/-0.66% with 9%/5%/3% encoding time and 10%/4%/4% of decoding time increase for AI/RA/LDB configurations correspondingly.

The suggested configuration is CE14-3b providing less memory footprint and requiring table with 16 entries of 7-bit elements for filtering process. The filter is not applied to blocks 4x4, has multiplication free realization, single implementation both for intra and inter CU filtering and allows to be implemented within 1 clock cycle in hardware.

# Technology description

Hadamard transform domain filter was proposed in Ljubljana meeting in contribution JVET-K0068. The filter is always applied to luma reconstructed blocks with non-zero transform coefficients, excluding 4x4 blocks and if quantization parameter is larger than 17. The filter parameters are explicitly derived from the coded information. Proposed filter, if applied, is performed on decoded samples right after block reconstruction. The filtered result is used both for output as well as for spatial and temporal prediction. The filter has same implementation both for intra and inter CU filtering.

## Filtering process

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+m*σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, m=4 is normalization constant equal to number of spectrum components, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2.64*2(0.1296*QP-11)

The first spectrum component corresponding to DC value is bypassed without filtering.

- Inverse 4 point Hadamard transform of filtered spectrum

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of pixels the accumulated values are normalized by number of processing groups used for each pixel filtering. Due to use of padding of one sample around the block number of processing groups is equal to 4 for each pixel in the block and normalization is performed by right shifting on 2 bits.

Scan

Hadamard transform

Filtering with an 16 or 32 entries look-up table

Inverse Hadamard transform

To accumulation buffer

A – current pixel

BCD – neighboring pixels

The filtering process is schematically presented on Figure 1.

Figure 1 Filtering process

Equivalent filter shape is 3x3 pixels as depicted on Figure 2.

Figure 2 Equivalent filter shape

It can be seen that all pixels in the block can be processed independently in case of maximum parallelism is required. It should also be noted that results of 2x2 groups filtering are reused for spatial collocated samples. Generally each new pixel in the block requires one 2x2 filtering, rest three are reused as can be seen from source code on Figure 3.

## LUT details

F(i,σ)=Ri   ,  AbsRi≥THRLUTRi,σ, R(i)>0-LUT-Ri,σ, R(i)≤0

It can be noted that filtering implies multiplication of spectrum component R(i) on scaling coefficient which is always less than 1. It can also be observed that at high values of R(i) scaling coefficient is close to 1. Based on these observation spectrum filtering is implemented using lookup table that allows to exclude multiplications and division from filtering operations:

, where: LUTRi,σ=R(i)3R(i)2+m*σ2.

The threshold THR is set to 128, that in case of straightforward implementation reuquires to store 128 (1 << 7) entries of 7-bit values per each QP.

The size of LUT influences on amount of on-chip memory requirements and the cost of a hardware implementation of the filter. To reduce amount of on-chip storage the LUT if calculated only for limited set of QPs starting from QP 20 with constant interval of 8. Totaly five pre-defined LUTs (for five QPs group) are stored. For filtering of current block CU’s QP is rounded to closest one from the table.

For further reduction of LUT size the N lowest bits are dropped (or ignored) during LUT generation. That allows to have sparse table representation.

- For test 14-3a N is equal to 2 that leads to 7 - 2 = 5 bits of table depth (32 entries of 7-bit values);

- For test 14-3b N is equal to 3 that leads to 7 - 3 = 4 bits of table depth (16 entries of 7-bit values).

Thus total memory size required for entire LUT storage:

- For test 14-3a: 5x32x7 bits = 1120 bits = 140 bytes;

- For test 14-3b, 5x16x7 bits = 560 bits = 70 bytes;

Configuration 14-3b is targeting to less memory footprint and requiring table with 16 entries of 7-bit elements for filtering process therefore this configuration is suggested.

# Computational complexity

The Figure 3 demonstates the filering process implemented in C++ code. It can be seen that filter has no multiplication operations involved. Eight additions are required both for forward and backward Hadamard transform, 4 for accumulation and 4 for rounding opearations. It should be noted that rounding operation requires just 1-bit addition being implememented in hardware. Eight additions of Hadamard transform can be split on two sequential operations each of them having 4 additions in parallel. Look-up table requires 2 checks for each access. The filter requires 3 access to the LUT per sample considering filtering of DC is skipped.

// 2x2 scan pattern
  static tHtdfOffset Scan[4] = { { 0,0 },{ 0,1 },{ 1,0 },{ 1,1 } };
  
  // init pointers according to scan pattern
  const int p0 = Scan[0].r*strideBlk + Scan[0].c;

const int p1 = Scan[1].r*strideBlk + Scan[1].c;

const int p2 = Scan[2].r*strideBlk + Scan[2].c;

const int p3 = Scan[3].r*strideBlk + Scan[3].c;

const int p0_out = Scan[0].r*strideAcc + Scan[0].c;

const int p1_out = Scan[1].r*strideAcc + Scan[1].c;

const int p2_out = Scan[2].r*strideAcc + Scan[2].c;
const int p3_out = Scan[3].r*strideAcc + Scan[3].c;

for (int r = 0; r < height - 1; ++r)

{

Pel *pIn = &blockIn[r*strideBlk];
    Pel *pOut = &blockOut[r*strideBlk];

Pel *pAcc = &accBlock[r*strideAcc];

for (int c = 0; c < width - 1; ++c, pIn++, pOut++)

{

const int x0 = pIn[p0];

const int x1 = pIn[p1];

const int x2 = pIn[p2];

const int x3 = pIn[p3];

// forward transform

const int y0 = x0 + x2;

const int y1 = x1 + x3;

const int y2 = x0 - x2;

const int y3 = x1 - x3;

const int t0 = y0 + y1;

const int t1 = y0 - y1;

const int t2 = y2 + y3;

const int t3 = y2 - y3;

// filtering

const int z0 = t0;  // skip DC

const int z1 = RdTbl(t1, tbl, thr);

const int z2 = RdTbl(t2, tbl, thr);

const int z3 = RdTbl(t3, tbl, thr);

// backward transform

const int iy0 = z0 + z2;

const int iy1 = z1 + z3;

const int iy2 = z0 - z2;

const int iy3 = z1 - z3;

const int f0 = (iy0 + iy1) >> HTDF_BIT_RND4;

const int f1 = (iy0 - iy1) >> HTDF_BIT_RND4;

const int f2 = (iy2 + iy3) >> HTDF_BIT_RND4;

const int f3 = (iy2 - iy3) >> HTDF_BIT_RND4;

pAcc[p0_out] += f0;

pAcc[p1_out] += f1;

pAcc[p2_out] += f2;

pAcc[p3_out] += f3;

// normalization

pOut[p0] = ClipPel((pAcc[p0_out] + HTDF_CNT_SCALE_RND) >> HTDF_CNT_SCALE, clpRng);

}

}

inline int RdTbl(const int z, const uint8_t *tbl, const int thr)

{

return  (z > 0 ? ( z < thr ?  tbl[(  z + HTDF_TBL_RND) >> HTDF_TBL_SHIFT] : z) 
: (-z < thr ? -tbl[((-z)+ HTDF_TBL_RND) >> HTDF_TBL_SHIFT] : z));

}

Figure 3 Source code of the filter

Total amount of operations required per each sample:

- Multiplications: 0,

- Additions: 20 + 4 of 1-bit additions for rounding,

- Shifts: 8,

- Checks: 6.

# Hardware implementation

Hadamard transform domain filter is placed just after block reconstruction and processes samples that can be involved into subsequent blocks reconstruction particularly as a reference samples of intra prediction. Thus the latency introduced by the filter needs to be minimized to ensure entire block reconstruction pipeline is not affected much.

Hadamard transform is considered to be relatively simple for hardware implementation. Only additions are required for its implementation but not multiplications. As can be seen from source code on Figure 3 forward and backward transforms contain 8 addition which can be done in parallel by using combinational logic with two sequintial addition operations.The more attention is required to fast and parallel access to the LUT.

## SRAM based LUT

In Figure 4 exemplary implementation with LUT stored in on-chip single port Static RAM is depicted.

Once data prom previous processing step is available in the buffer by rising edge of the clock it is accessed by combinational logic implementing forward Hadamard transform (containing two subsequent additions). After completing the combinational logic address is available for each LUT. Using invertor and falling edge of the clock the data is accessed from the SRAM. Second combinational logic implementing backward Hadamard transform and normalization is started immediately after data from LUT is available. The output filtered samples became available at the end of current clock cycle and are ready for processing by next algorithm at the next rising edge of the clock. Total amount of LUTs depends on desired level of parallelism. E.g. processing right and bottom samples line of 8x4 block in parallel requires (8+3)*2 = 22 LUTs instantiation. Considering that LUT required for filtering can be preloaded from ROM once QP for current CU is available using suggested design with 16 table entries for filtering total amount of memory required for parallel processing of right-bottom samples will be 16*22 = 352 elements of 7 bit values.

Figure 4 Exemplary hardware filter design with SRAM based LUT

## Flip-flop based LUT

Considering that one table for filtering process is limited by 16 entries it looks more efficient to implement LUT based on flip-flops. Such design does not require several instantiations of LUT in memory for parallel processing and clock edge for data access. Parallel access is provided by multiplexers as depicted in Figure 6 which illustrates 2x2 group transform domain filtering. By using suggested filter design only 16 flip-flops of 7 bits are required to provide parallel access to the LUT during the filtering process. The QP specific LUT can be loaded to flip-plops once QP for current CU is available.

Figure 5 2x2 group transform domain filtering with flip-flop based LUT

Figure 6 below demonstrates how filtering can be implemented for 4 horizontal samples in parallel. Input block contains 18 samples (x0 - x17). Pixels to be filtered are marked by grey color (x7 - x10). The 2x2 processing groups are combined together with reusing filtering results for spatial neighboring samples. The final averaging is performed before filtered samples are placed to output buffer.

Figure 6 Filtering of 4x1 block in parallel

Considering entire filter design based on flip-flop LUT it can be seen that filer can be implemented using combinational logic only without necessity of intermediate buffers or clock edge for RAM access. The total filtering latency is defined by timing of combinational logic involved. Whether it fits to one clock latency depends on certain clock rate and manufacturing process as is illustrated in the next section.

## Prototype RTL design

This section reports results of synthesizing prototype RTL design with different clock rate and manufacturing process. The analysis was carried out, gate count and latency estimations were obtained for 1x1, 1x4 and 1x8 pixel blocks. Each sample is processed in parallel as in Figure 6 and suggested design with 16x7-bit entries table is used for filtering.

| Output rate(pixe/clk) | 1 | 4 | 8 |
| --- | --- | --- | --- |
| Latency(clk) | 1 | 1 | 1 |
| Gate count | 14565 | 39020 | 65816 |

Table 1 28nm TSMC, CLK 450 MHz (2.22ns)

| Output rate(pixe/clk) | 1 | 4 | 8 |
| --- | --- | --- | --- |
| Latency(clk) | 2 | 2 | 2 |
| Gate count | 15459 | 38235 | 68806 |

Table 2 28nm TSMC	, CLK 770 MHz (1.3ns)

| Output rate(pixe/clk) | 1 | 4 | 8 |
| --- | --- | --- | --- |
| Latency(clk) | 1 | 1 | 1 |
| Gate count | 13589 | 35000 | 61737 |

Table 3 16nm TSMC, CLK 770 MHz (1.3ns)

Given above analysis allows came to the conclusion that proposed filter can be implemented in hardware within one clock at 770MHz 16nm and 450MHz 28nm design. At 770MHz 28nm design the filter has two clock latency.

It should also be noted that filter is the same both for inter and inter CU filtering. That eliminates necessity to have different realizations of the filter that reduces implementation cost.

# Complexity analysis summary

| Test# | filter shape | Computational complexity (# of multiplications/ additions/shifts/checks*) per sample | Precision of multiply | Parallel friendly (independent processing of samples) | Number of clock cycles | memory requirement | How to derive filter coeffs | Minimal CU Size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14-3a | 3x3 | 0/20+4 l-bit add for round/8/6 | n/a | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 140 bytes (32 7-bit values per qp group) | Precal-culated in LUT | 4x8, 8x4 |
| 14-3b | 3x3 | 0/20+4 l-bit add for round /8/6 | n/a | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 70 bytes (16 7-bit values per qp group) | Precal-culated in LUT | 4x8, 8x4 |

# * max/min/abs are countered as checks

Table 4 CE14-3 complexity analysis summary

# Experimental results

## VTM based results

The simulations were carried out according to JVET common test condition [1]. The BMS-2.0.1 was used as software basis. Decoder running time was verified on local machine using concatenated bitstreams. The result of simulations using VTM configuration file is presented in the following tables:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.29% | 0.24% | 110% | 110% |
| Class A2 | -0.35% | 0.31% | 0.32% | 109% | 109% |
| Class B | -0.42% | 0.29% | 0.32% | 109% | 109% |
| Class C | -0.83% | 0.23% | 0.42% | 110% | 109% |
| Class E | -0.54% | 0.28% | 0.19% | 109% | 112% |
| Overall | -0.48% | 0.28% | 0.31% | 109% | 110% |
| Class D | -0.61% | 0.47% | 0.01% | 109% | 107% |
| Class F (optional) | -0.82% | 0.07% | 0.04% | 108% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.23% | -0.25% | 105% | 103% |
| Class A2 | -0.92% | -0.57% | -0.43% | 105% | 102% |
| Class B | -0.69% | -0.02% | -0.27% | 106% | 104% |
| Class C | -0.75% | 0.10% | 0.14% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.14% | -0.19% | 105% | 104% |
| Class D | -0.67% | -0.14% | 0.53% | 104% | 104% |
| Class F (optional) | -0.77% | 0.10% | 0.13% | 104% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.58% | 0.52% | 0.54% | 104% | 103% |
| Class C | -0.74% | 0.33% | 0.52% | 104% | 105% |
| Class E | -0.75% | 0.31% | 1.09% | 102% | 103% |
| Overall | -0.68% | 0.40% | 0.67% | 104% | 104% |
| Class D | -0.90% | 0.75% | 0.28% | 104% | 104% |
| Class F (optional) | -0.73% | 0.20% | -0.09% | 103% | 104% |

Table 5 Coding performance of test 14-3a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.28% | 0.28% | 110% | 110% |
| Class A2 | -0.33% | 0.27% | 0.29% | 109% | 109% |
| Class B | -0.40% | 0.31% | 0.39% | 109% | 109% |
| Class C | -0.82% | 0.20% | 0.32% | 109% | 109% |
| Class E | -0.54% | 0.34% | 0.25% | 109% | 112% |
| Overall | -0.47% | 0.28% | 0.32% | 109% | 110% |
| Class D | -0.65% | 0.43% | 0.22% | 109% | 107% |
| Class F (optional) | -0.81% | 0.01% | 0.12% | 108% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.36% | -0.24% | 105% | 104% |
| Class A2 | -0.92% | -0.49% | -0.37% | 105% | 102% |
| Class B | -0.69% | -0.12% | 0.12% | 106% | 104% |
| Class C | -0.75% | 0.14% | 0.10% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.17% | -0.06% | 105% | 104% |
| Class D | -0.69% | -0.30% | 0.49% | 104% | 103% |
| Class F (optional) | -0.82% | 0.11% | 0.11% | 104% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.57% | 0.49% | 0.69% | 104% | 103% |
| Class C | -0.76% | 0.31% | 0.42% | 104% | 105% |
| Class E | -0.67% | -0.66% | -0.48% | 102% | 103% |
| Overall | -0.66% | 0.14% | 0.31% | 103% | 104% |
| Class D | -0.88% | 0.92% | 0.31% | 103% | 104% |
| Class F (optional) | -0.74% | 0.38% | -0.15% | 103% | 104% |

Table 6 Coding performance of test 14-3b

## BMS based results

For optional BMS based simulations BMS-2.1 was used as software basis. The result of simulations using BMS configuration file is presented in the following tables:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.35% | 0.22% | 110% | 111% |
| Class A2 | -0.44% | 0.23% | 0.28% | 109% | 109% |
| Class B | -0.46% | 0.30% | 0.28% | 109% | 109% |
| Class C | -0.75% | 0.23% | 0.11% | 109% | 107% |
| Class E | -0.58% | 0.18% | 0.19% | 109% | 110% |
| Overall | -0.51% | 0.26% | 0.22% | 109% | 109% |
| Class D | -0.56% | 0.26% | 0.23% | 109% | 107% |
| Class F (optional) | -0.88% | 0.10% | 0.10% | 107% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.31% | -0.11% | 107% | 103% |
| Class A2 | -1.06% | -0.36% | -0.35% | 106% | 102% |
| Class B | -0.82% | 0.14% | -0.18% | 107% | 102% |
| Class C | -0.87% | -0.10% | 0.13% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.80% | -0.12% | -0.12% | 107% | 102% |
| Class D | -0.72% | 0.10% | 0.05% | 106% | 102% |
| Class F (optional) | -0.87% | 0.04% | 0.33% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | 0.32% | 0.95% | 106% | 104% |
| Class C | -0.83% | 0.24% | 0.51% | 105% | 105% |
| Class E | -0.56% | 1.39% | 0.97% | 102% | 102% |
| Overall | -0.68% | 0.56% | 0.81% | 105% | 104% |
| Class D | -0.99% | 0.17% | 1.06% | 105% | 103% |
| Class F (optional) | -0.74% | 0.61% | -0.15% | 104% | 103% |

Table 7 Coding performance of test 14-3a over BMS2.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.27% | 0.28% | 110% | 111% |
| Class A2 | -0.43% | 0.20% | 0.25% | 109% | 109% |
| Class B | -0.43% | 0.29% | 0.26% | 109% | 109% |
| Class C | -0.75% | 0.18% | 0.06% | 110% | 107% |
| Class E | -0.54% | 0.11% | 0.16% | 109% | 110% |
| Overall | -0.49% | 0.22% | 0.20% | 109% | 109% |
| Class D | -0.57% | 0.33% | 0.38% | 109% | 107% |
| Class F (optional) | -0.88% | 0.04% | 0.05% | 107% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.21% | -0.13% | 107% | 103% |
| Class A2 | -1.04% | -0.47% | -0.32% | 106% | 102% |
| Class B | -0.79% | 0.02% | -0.06% | 107% | 102% |
| Class C | -0.81% | 0.04% | 0.19% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.77% | -0.12% | -0.06% | 107% | 102% |
| Class D | -0.67% | 0.09% | 0.36% | 106% | 102% |
| Class F (optional) | -0.86% | 0.14% | 0.23% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.1 with BMS config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | 0.41% | 0.61% | 106% | 104% |
| Class C | -0.80% | 0.37% | 0.53% | 105% | 104% |
| Class E | -0.53% | 0.92% | -0.37% | 102% | 102% |
| Overall | -0.67% | 0.53% | 0.34% | 105% | 104% |
| Class D | -0.98% | 0.39% | 1.35% | 104% | 103% |
| Class F (optional) | -0.60% | 0.89% | 0.05% | 104% | 101% |

Table 8 Coding performance of test 14-3b over BMS2.1

## Additional test

Additional test for suggested configuration 14.3b with filter implemented in SSE was carried out. The simulation results (for decoder only) are in the following table:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.28% | 0.28% |  | 107% |
| Class A2 | -0.33% | 0.27% | 0.29% |  | 106% |
| Class B | -0.40% | 0.31% | 0.39% |  | 107% |
| Class C | -0.82% | 0.20% | 0.32% |  | 107% |
| Class E | -0.54% | 0.34% | 0.25% |  | 107% |
| Overall | -0.47% | 0.28% | 0.32% |  | 107% |
| Class D | -0.65% | 0.43% | 0.22% |  | 106% |
| Class F (optional) | -0.81% | 0.01% | 0.12% |  | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.36% | -0.24% |  | 102% |
| Class A2 | -0.92% | -0.49% | -0.37% |  | 101% |
| Class B | -0.69% | -0.12% | 0.12% |  | 102% |
| Class C | -0.75% | 0.14% | 0.10% |  | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.17% | -0.06% |  | 102% |
| Class D | -0.69% | -0.30% | 0.49% |  | 102% |
| Class F (optional) | -0.82% | 0.11% | 0.11% |  | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.57% | 0.49% | 0.69% |  | 103% |
| Class C | -0.76% | 0.31% | 0.42% |  | 104% |
| Class E | -0.67% | -0.66% | -0.48% |  | 101% |
| Overall | -0.66% | 0.14% | 0.31% |  | 103% |
| Class D | -0.88% | 0.92% | 0.31% |  | 104% |
| Class F (optional) | -0.74% | 0.38% | -0.15% |  | 101% |

Table 9 Coding performance of test 14-3b with filter implemented in SSE

The source code of this test is available by this link:
https://vcgit.hhi.fraunhofer.de/JVET-K-CE14/VVCSoftware_BMS/tree/CE14.3-SSE.

# Conclusions

Suggested configuration CE14-3b has 16 elements of 7-bit values required for filtering process with total LUT size of 70 bytes for five QP groups providing luma BD-rates of -0.47%/-0.70%/-0.66% for AI/RA/LDB configurations over BMS-2.0.1 with VTM configuration and -0.49%/-0.77%/-0.67% for AI/RA/LDB configurations over BMS-2.1 with BMS configuration. The filter has same design for inter and intra CU filtering, has no multiplication operations involved and can be implemented in hardware within 1 clock cycle delay.

Suggested to adopt Hadamard transform domain filter to next version of VTM.

# References

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).