| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L329-v1 |
| --- | --- |

| Title: | CE3-related: CCLM prediction with single-line neighbouring luma samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM-2.0, CCLM prediction mode employs a six-tap down-sampling filter to down-sample neighbouring luma samples during CCLM parameter derivation process. As a result, two rows of neighbouring luma samples above the current block and three columns of neighbouring luma samples left to the current block are required, which increases the line buffer size and computational complexity. In this contribution, down-sampling filters with less taps are applied on the neighbouring luma samples. Meanwhile, only neighbouring luma samples located at one row above and one column left to the current block are required. Simulation results reportedly show 0.08% and 0.03% BD-rate loss under All Intra (AI) and Random Access (RA) configurations, respectively with almost the same running time.

# Introduction

Fig. 1 Locations of the samples used for the derivation of α and β

Fig. 2 Down-sampling filtering in CCLM

LM prediction in VTM-2.0 builds a linear model between the luma samples and chroma samples.  Neighbouring luma and chroma reconstructed samples are used to derive the parameters α and β in the linear model as shown in Fig.1. With the 4:2:0 format, LM prediction applies a six-tap down-sampling filter to get the down-sampled luma sample corresponding to a chroma sample as shown in Fig. 2. In a formula way, a down-sampled luma sample Rec’L[x, y] is calculated from reconstructed luma samples as:

|  | (1) |
| --- | --- |

As a result, two rows of neighbouring luma samples above the current block and three columns of neighbouring luma samples left to the current block are required by the training process.

# CCLM prediction with single-line neighbouring luma samples

To constrain the neighbouring luma samples required by the training process in a single line, down-sampling filters with less taps are applied as shown in Fig. 3(a):

- For neighbouring luma samples above:

| . | (2) |
| --- | --- |

- For neighbouring luma samples left:

| . | (3) |
| --- | --- |

The luma samples inside the block are still down-sampled with the six-tap filter.

Two solutions are provided in this contribution with usage of different sets of neighbouring luma samples. Suppose the width and height of one block is denoted as W and H, respectively. In solution #1, W above neighbouring samples and H left neighbouring samples are involved in the training process in Fig. 3(a). In solution #2, 2W above neighbouring samples and 2H left neighbouring samples are involved as shown in Fig. 3(b). It should be noted that the extended neighbouring samples in solution #2 have already been used by wide angle intra-prediction.

| (a) | (b) |
| --- | --- |

Fig. 3 LM prediction with single-line neighbouring luma samples.

# Experimental Results

The proposed CCLM prediction methods are implemented on VTM-2.0.1. Test #1 and test #2 are conducted for solution #1 and solution #2, respectively. Simulation results of All Intra (AI) and Random Access (RA) configuration for Test 1 and Test 2 are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Restuls for Test 1

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 2.11% | 1.56% | 100% | 102% | 0.13% | 1.46% | 1.17% | 99% | 99% |
| Class A2 | 0.11% | 0.45% | 0.29% | 100% | 99% | 0.05% | 0.55% | 0.23% | 99% | 98% |
| Class B | 0.05% | 0.49% | 0.91% | 100% | 99% | 0.05% | 0.66% | 1.04% | 99% | 101% |
| Class C | 0.18% | 1.10% | 1.34% | 99% | 98% | 0.09% | 1.00% | 1.09% | 100% | 106% |
| Class E | -0.01% | 0.15% | 0.06% | 101% | 105% |  |  |  |  |  |
| Overall | 0.11% | 0.83% | 0.87% | 100% | 100% | 0.08% | 0.89% | 0.92% | 99% | 101% |

Table 2: Results of Test 2

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 1.37% | 0.96% | 101% | 100% | 0.08% | 0.82% | 0.57% | 101% | 97% |
| Class A2 | 0.07% | 0.01% | 0.04% | 100% | 100% | 0.02% | 0.13% | 0.16% | 100% | 95% |
| Class B | 0.03% | 0.16% | 0.38% | 100% | 100% | 0.01% | 0.08% | 0.43% | 101% | 98% |
| Class C | 0.18% | 0.70% | 1.01% | 99% | 98% | 0.04% | 0.69% | 0.74% | 102% | 107% |
| Class E | -0.01% | -0.26% | -0.35% | 102% | 101% |  |  |  |  |  |
| Overall | 0.08% | 0.39% | 0.44% | 100% | 99% | 0.03% | 0.40% | 0.49% | 101% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, Jul. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).