#### Derivation process for luma affine control point motion vectors from a neighbouring block

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- a luma location ( xNb, yNb ) specifying the top-left sample of the neighbouring luma coding block relative to the top-left luma sample of the current picture,

- two variables nNbW and nNbH specifying the width and the height of the neighbouring luma coding block,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variables log2NbW and log2NbH are derived as follows:

log2NbW = Log2( nNbW )		(8-269)

log2NbH = Log2( nNbH ) 		(8-270)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = MvLX[ xNb ][ yNb ][ 0 ] << 7		(8-271)

mvScaleVer = MvLX[ xNb ][ yNb ][ 1 ] << 7 		(8-272)

dHorX = ( MvLX[ xNb + nNbW − 1 ][ yNb ][ 0 ] − MvLX[ xNb ][ yNb ][ 0 ] ) << ( 7 − log2NbW )	(8-273)

dVerX = ( MvLX[ xNb + nNbW − 1 ][ yNb ][ 1 ] − MvLX[ xNb ][ yNb ][ 1 ] )  << ( 7 − log2NbW )	(8-274)

If yNb + nNbH is equal to yCb,

mvScaleHor = MvLX[ xNb ][ yCb-1][ 0 ] << 7

mvScaleVer = MvLX[ xNb ][ yCb-1][ 1 ] << 7

dHorX = ( MvLX[ xNb + nNbW − 1 ][ yCb-1][ 0 ] − MvLX[ xNb ][ yCb-1] [ 0 ] )  << ( 7 − log2NbW )

dVerX = ( MvLX[ xNb + nNbW − 1 ][ yCb-1][ 1 ] − MvLX[ xNb ][ yCb-1] [ 1 ] )  << ( 7 − log2NbW )

Otherwise,

mvScaleHor = MvLX[ xCb-1 ][ yNb ][ 0 ] << 7

mvScaleVer = MvLX[ xCb-1 ][ yNb  ][ 1 ] << 7

dHorX = ( MvLX[ xCb-1 ][ yNb + nNbH  -1][ 1 ] − MvLX[ xCb-1 ] [ yNb] [ 1 ] )  << ( 7 − log2NbH )

dVerX = -( MvLX[ xCb-1 ][ yNb + nNbH  -1][ 0 ] − MvLX[ xCb-1 ] [ yNb ] [ 0 ] )  << ( 7 − log2NbH )

The variables dHorY and dVerY are derived as follows:

- If MotionModelIdc[ xNb ][ yNb ] is equal to 2, the following applies:

dHorY = ( MvLX[ xNb ][ yNb + nNbH − 1 ][ 0 ] − MvLX[ xNb ][ yNb ][ 0 ] ) << ( 7 − log2NbH ) 	(8-275)

dVerY = ( MvLX[ xNb ][ yNb + nNbH − 1 ][ 1 ] − MvLX[ xNb ][ yNb ][ 1 ] ) << ( 7 − log2NbH ) 	(8-276)

- Ohterwise (MotionModelIdc[ xNb ][ yNb ] is equal to 1), the following apply,

dHorY = − dVerX		(8-277)

dVerY = dHorX			(8-278)

If MotionModelIdc[ xNb ][ yNb ] is equal to 2, the following applies:

If yNb + nNbH is equal to yCb and nNbW is greater than 8, the following applies:

auxMVx = MvLX[ xNb + nNbW − 1 ][ yCb-1][ 0 ] + MvLX[ xNb ][ yCb-1] [ 0 ] ;

auxMVx = auxMVx >=0 ? (auxMVx+1)>>1: -( (-auxMVx+1)>>1);

auxMVy = MvLX[ xNb + nNbW − 1 ][ yCb-1][1 ] + MvLX[ xNb ][ yCb-1] [1 ] ;

auxMVy = auxMVy >=0 ? (auxMVy+1)>>1: -( (-auxMVy+1)>>1);

dHorY = ( MvLX[ xNb + (nNbW >>1) ][ yCb -1][ 0 ] − auxMVx) << 4;

dVerY = ( MvLX[ xNb + (nNbW >>1) ][ yCb -1][ 1] − auxMVy) << 4;

Otherwise, if xNb + nNbW is equal to xCb and nNbH is greater than 8, the following applies:

auxMVx = MvLX[ xCb− 1 ][ yNb + nNbH -1][ 0 ] + MvLX[ xCb-1 ][ yNb] [ 0 ] ;

auxMVx = auxMVx >=0 ? (auxMVx+1)>>1: -( (-auxMVx+1)>>1);

auxMVy = MvLX[ xCb− 1 ][ yNb + nNbH -1] [1 ] + MvLX[ xCb-1 ][ yNb] [1 ] ;

auxMVy = auxMVy >=0 ? (auxMVy+1)>>1: -( (-auxMVy+1)>>1);

dHorX  = ( MvLX[ xCb− 1 ][ yNb + (nNbH>>1)][ 0 ] − auxMVx) << 4;

dVerX = ( MvLX[ xCb− 1  ][ yNb + (nNbH>>1)][ 1 ] − auxMVy) << 4;

If yNb + nNbH is equal to yCb, yNb is set equal to yCb-1. Otherwise, xNb is set equal to xCb-1.

The luma affine control point motion vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1 are derived as follows:

…

#### Derivation process for motion vector arrays from affine control point motion vectors

-The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

for X=0,1, x = xCb,..xCb+3 and y = yCb,…yCb+3:

MvLX[ x ][ cbHeight - 4 + y ][0]  = ( mvScaleHor + dHorY * cbHeight )

MvLX[ x ][ cbHeight - 4 + y ][1]  = ( mvScaleVer +  dVerY * cbHeight )

MvLX[ cbWidth -4 + x ][ cbHeight - 4 + y ][0]  = ( mvScaleHor + dHorX * cbWidth  + dHorY * cbHeight )

MvLX[ cbWidth -4 + x ][ cbHeight - 4 + y ][1]  = ( mvScaleVer + dVerX * cbWidth  + dVerY * cbHeight )

MvLX[ x+( cbWidth >> 1 )  ][ cbHeight - 4 + y ][0]  = ( mvScaleHor + dHorX * (cbWidth>>1)  + dHorY * (cbHeight+8) )

MvLX[ x+( cbWidth >> 1 )  ][ cbHeight - 4 + y ][1]  = ( mvScaleVer + dVerX * (cbWidth>>1)  +  dVerY * (cbHeight+8) )

MvLX[  cbWidth -4 + x ][ ( cbHeight >> 1 )  + y ][0]  = ( mvScaleHor + dHorX * (cbWidth-4)  + dHorY * (cbHeight>>1) )

MvLX[  cbWidth -4 + x ][ ( cbHeight >> 1 )  + y ][1]  = ( mvScaleVer + dVerX * (cbWidth-4)  + dVerY * (cbHeight>>1) )