| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0330-v2 |
| --- | --- |

| Title: | CE4-related: Affine model inheritance from single-line motion vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Affine model inheritance in VTM-2.0 needs to access motion vectors from not-adjacent blocks, which requires additional line-buffers. In this contribution, affine model inheritance only needs to access motion vectors in a single line which is located above and left to the current block. Therefore, line-buffer size could be reduced. Simulation results reportedly show 0.04% and 0.03% BD-rate changing under Random Access (RA) and Low-delay B (LB) configurations, respectively.

# Introduction

Fig. 1 CPMV derivation in affine inheritance.

In VTM-2.0, affine inheritance is employed to derive the Control Point Motion Vectors (CPMVs) of the current affine coded block from a neighbouring affine coded block to construct the affine merge candidate or the affine MVP candidate. As shown in Fig. 1, CPMVs mv0N and mv1N (mv2N is also involved if the neighbouring block is coded with the 6-paramter affine model) are used to derive the CPMVs of the current block.

The affine inheritance procedure requires additional line-buffers to store non-adjacent MVs such as mv0N and mv2N, which are never accessed in HEVC.

# Affine model inheritance from single-line motion vectors

To address the line buffer issue, it is proposed that the affine model is inherited by only accessing the motion vectors in a single line. In the proposed solution #1, the inherited affine model always has 4 parameters. In the proposed solution #2, the inherited affine model can also has 6 parameters.

In solution #1:

- If the affine model is inherited from a neighbouring block above the current block:

  - The top-left and top-right CPMVs of the current block are derived from the bottom-left and bottom right CPMVs of the neighbouring block. As shown in Fig. 2(a), mv2N and mv3N are used to derive mv0C and mv1C.

- Otherwise (the affine model is inherited from a neighbouring block left to the current block)

  - The top-left and top-right CPMVs of the current block are derived from the top-left and bottom-left CPMVs of the neighbouring block. As shown in Fig. 2(b), mv1N and mv3N are used to derive mv0C and mv1C.

In solution #2:

- If the affine model is inherited from a neighbouring block above the current block:

  - If the neighbouring block employs the 4-parameter affine model or the width of the neighbouring block is equal to 8, the affine model is inherited as in solution #1;

  - Otherwise, the top-left, top-right and bottom-left CPMVs of the current block are derived from the bottom-left and bottom right CPMVs of the neighbouring block, and an auxiliary MV stored in the bottom-middle 4×4 block of the above neighbouring block. As shown in Fig. 3(a), mv2N, mv3N and mvAB are used to derive mv0C, mv1C and mv2C as:

- Otherwise (the affine model is inherited from a neighbouring block left to the current block)

  - If the neighbouring block employs the 4-parameter affine model or the height of the neighbouring block is equal to 8, the affine model is inherited as in solution #1;

  - Otherwise, the top-left, top-right and bottom-left CPMVs of the current block are derived from the top-left and bottom-left CPMVs of the neighbouring block and an auxiliary MV stored in the right-middle 4×4 block of the left neighbouring block. As shown in Fig. 3(b), mv1N, mv3N and mvAR are used to derive mv0C and mv1C and mv2C.

With the proposed design, only one row of MVs above the current block and one column of MVs left to the current block need to be accessed by the affine model inheritance.

| (a) | (b) |
| --- | --- |

Fig. 2 Affine model inheritance only accessing MVs in a single line with solution #1.

| (a) | (b) |
| --- | --- |

Fig. 3 Affine model inheritance only accessing MVs in a single line with solution #2.

Fig. 4 shows the MV storage after decoding an affine coded CU. The four corner 4×4 blocks store the four CPMVs at positions (x0, y0), (x0+w, y0), (x0, y0+h), (x0+w, y0+h), respectively. If the CU is coded with the 6-parameter affine model, bottom-middle and right-middle 4×4 blocks store auxiliary MVs at positions (x0+w/2, y0+h+8) and (x0+w+8, y0+h/2), respectively. All other 4×4 blocks store the MVs at their central positions.

Fig. 4 MV storage after decoding an affine CU.

# Experimental Results

The proposed method has been implemented on top of VTM-2.0.1, and results of solution #1 and solution #2 with RA and LB configuration are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for Test 1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.24% | 0.11% | 100% | 98% |  |  |  |  |  |
| Class A2 | 0.71% | 0.58% | 0.59% | 100% | 99% |  |  |  |  |  |
| Class B | 0.21% | 0.13% | 0.10% | 101% | 101% | 0.09% | -0.08% | 0.10% | 101% | 100% |
| Class C | 0.04% | 0.12% | 0.05% | 102% | 103% | 0.03% | -0.21% | -0.15% | 100% | 97% |
| Class E |  |  |  |  |  | -0.14% | 0.25% | 0.13% | 102% | 104% |
| Overall | 0.26% | 0.24% | 0.19% | 101% | 101% | 0.01% | -0.04% | 0.02% | 101% | 100% |

Table 2: Results for Test 2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.03% | 99% | 98% |  |  |  |  |  |
| Class A2 | 0.12% | -0.02% | 0.04% | 99% | 99% |  |  |  |  |  |
| Class B | 0.04% | 0.04% | -0.01% | 99% | 98% | 0.05% | -0.15% | -0.17% | 100% | 99% |
| Class C | 0.02% | 0.11% | 0.01% | 99% | 94% | 0.07% | 0.02% | -0.18% | 98% | 94% |
| Class E |  |  |  |  |  | -0.06% | -0.03% | -0.86% | 99% | 99% |
| Overall | 0.04% | 0.03% | 0.01% | 99% | 97% | 0.03% | -0.06% | -0.34% | 99% | 97% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, Jul. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).