| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0331-v1 |
| --- | --- |

| Title: | CE6 related: On Index Signalling of Multiple Transform Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.v@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM2.0, Multiple Transform Selection (MTS) is employed wherein a block could adaptively select transform matrices from a set of transform matrices. The index of selected transform matrix is signalled to the decoder side. To save the bits due to index signalling, for an intra-coded block, if there are less than 3 non-zero coefficients, the index signalling is skipped and default transform matrices are used implicitly. A counter is required to record how many non-zero coefficients are available within one coding unit which increases the burden of parsing process. In this contribution, the counter is replaced by the position of the last non-zero coefficient. Simulation results reportedly show that proposed method has no BD rate changes for All Intra configurations but both encoding and decoding time are reduced.

# Introduction

In Versatile Video Coding (VVC), Multiple Transform Selection (MTS) is employed wherein a block could adaptively select transform matrices from a set of transform matrices. The index of selected transform matrix is signalled to the decoder side.

To save the bits due to index signalling, for an intra-coded block, if there are less than 3 non-zero coefficients, the index signalling is skipped and default transform matrices are used implicitly. A counter is required to record how many non-zero coefficients are available within one coding unit (CU). which increases the burden of parsing process. For the worst case, it requires 64×64 times additions or 64×64 times comparisons (whether the counter is larger than 2 for each non-zero coefficient) and 3 additions.

# Proposed method

It is proposed to replace checking the non-zero coefficients by checking the position of the last non-zero coefficient.

The proposed syntax changes on top of JVET-K1001 version 6 are as follows: (red text shows what is newly added, highlighted yellow part with strikethrough shows what is removed).

# 7.3.4.9	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| do { |  |
| … |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numSigCoeff = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| numSigCoeff++ |  |
| par_level_flag[ n ] | ae(v) |
| rem_abs_gt1_flag[ n ] | ae(v) |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + 2 * rem_abs_gt1_flag[ n ] |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| if( rem_abs_gt1_flag[ n ] ) |  |
| rem_abs_gt2_flag[ n ] | ae(v) |
| } |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( rem_abs_gt2_flag[ n ] ) |  |
| abs_remainder[ n ] |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * ( rem_abs_gt2_flag[ n ] + abs_remainder[ n ] ) |  |
| } |  |
| … |  |
| } |  |
| if( cu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && numSigCoeff (LastSignificantCoeffX+ LastSignificantCoeffY) > N ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) ) ) { |  |
| mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

In this contribution, the threshold N in above table is set to 2, 3, and 4. Different simulation results are reported in section 4.

# Simulation results

The tables below show the BD-rate gain and processing time impact of the proposed approach. Three sets of simulations are conducted, following the common test conditions [1] on top of reference software VTM-2.0.1 [2] .

Table 1. Performance of proposed method under All Intra Main 10

|  |  | N = 2 |  |  |  |  |  |  |  |  | N = 3 |  |  |  |  | N = 4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | EncT |  | DecT | Y |  | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% |  |  | -0.03% |  | -0.03% |  | 99% | 99% | 0.01% |  | 0.00% | -0.04% | 97% | 99% | 0.02% | -0.06% | -0.01% | 97% | 98% |
| Class A2 | -0.01% |  |  | -0.04% |  | -0.05% |  | 102% | 99% | 0.00% |  | 0.01% | -0.09% | 99% | 99% | 0.02% | -0.04% | -0.07% | 98% | 98% |
| Class B | 0.00% |  |  | 0.00% |  | -0.03% |  | 99% | 99% | 0.01% |  | -0.05% | 0.00% | 98% | 100% | 0.02% | -0.09% | -0.08% | 97% | 99% |
| Class C | -0.01% |  |  | -0.10% |  | 0.08% |  | 99% | 98% | -0.03% |  | -0.17% | 0.04% | 98% | 101% | -0.01% | -0.08% | 0.00% | 97% | 98% |
| Class E | -0.01% |  |  | -0.08% |  | -0.12% |  | 98% | 96% | 0.02% |  | -0.07% | -0.13% | 97% | 101% | -0.01% | -0.06% | -0.28% | 97% | 100% |
| Overall | 0.00% |  |  | -0.05% |  | -0.02% |  | 99% | 98% | 0.00% |  | -0.06% | -0.04% | 98% | 100% | 0.02% | -0.08% | -0.07% | 97% | 98% |
| Class D | -0.03% |  |  | 0.11% |  | -0.12% |  | 99% | 100% | -0.02% |  | 0.08% | -0.23% | 99% | 104% | -0.01% | -0.06% | -0.28% | 97% | 100% |
| Class F | -0.01% |  | -0.06% |  | -0.03% |  | 97% |  | 95% | 0.00% |  | -0.10% | -0.08% | 97% | 95% | 0.00% | -0.11% | -0.18% | 95% | 95% |

# Conclusions

This contribution proposes to use the coordinate of last non-zero coefficient to determine whether transform index in MTS could be skipped. The parsing process could be greatly simplified. Meanwhile, simulation demonstrates there is no coding loss but with both encoder and decoder running time savings. It is suggested adopting this proposal to VVC.

# Reference

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, doc. JVET-J1010, April 2018.

- BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).