| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0333-v1 |
| --- | --- |

| Title: | CE9-related: Motion Vector Refinement in Bi-directional Optical Flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang Yue Wang, Pengwei Zhao, Dingkun Hong Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In Bi-directional Optical flow (BIO), motion vector between two reference blocks is derived at sub-block level and is used to improve the prediction of a coding unit. However, the derived motion vector is not further used to refine the signaled motion vectors. This contribution studies the impact of refining the signaled motion vectors with derived motion vectors in BIO. Specifically, for each prediction direction LX, the derived motion vector is scaled according to the Picture Count Order (POC) distance between the two reference pictures and the POC distance between the LX reference picture and the current picture, then it is added to the signaled motion vector. Simulation results reportedly show 0.63% BD-rate saving under Random Access configuration with minor running time increase.

# Introduction

In benchmark set (BMS) [1], BIO may be applied to a bi-predicted coding unit (CU) to further improve its prediction. For each sub-block in the CU, a motion vector is derived between its corresponding two reference sub-blocks as shown in Figure 1-1.

Figure 1-1. derived MV in BIO.

The derived motion vector, pointing from the sub-block in reference block 0 to the sub-block in reference block 1, denoted by (DMVx, DMVy), is used to further improve prediction of the current sub-block. However, the derived motion vector is not used to refine the motion vector of the sub-block.

# Proposed method

This contribution proposes to refine the motion vector of each sub-block by using the derived motion vector in BIO. Denote the POC distance (i.e., absolute POC difference) between the LX reference picture and the current picture as deltaPOCX, and denote (MVLXx, MVLXy) and (MVLXx’, MVLXy’) as the signaled and refined motion vector of the current sub-block, with X = 0 or 1. Then (MVLXx’, MVLXy’) is calculated as follows:

MVL0x'=MVL0x-deltaPOC0*DMVxdeltaPOC0+deltaPOC1

MVL0y'=MVL0y-deltaPOC0*DMVydeltaPOCX+deltaPOC1

MVL1x'=MVL1x+deltaPOC1*DMVxdeltaPOC0+deltaPOC1

MVL1y'=MVL1y+deltaPOC1*DMVydeltaPOCX+deltaPOC1

However, multiplication and division are required in the above equations. To address this issue, derivation of the refined motion vector is simplified as follows:

MVL0x'=MVL0x-DMVx+1≫1

MVL0y'=MVL0y-DMVy+1≫1

MVL1x'=MVL1x+DMVx+1≫1

MVL1y'=MVL1y+DMVy+1≫1

Proposed method is employed only when the current CU is predicted from a preceding picture and a following picture, and therefore only works in the Random Access configuration.

# Experimental results

The proposed method was tested on top of reference software BMS-2.1 [1], and simulations were performed following the common test conditions [2]. Following configuration is used: VTM configuration with BIO enabled. The results are shown in Table 1.

Table 1: Coding performance of Test #1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.32% | -0.43% | 101% | 100% |
| Class A2 | -1.03% | -0.92% | -1.02% | 99% | 98% |
| Class B | -0.57% | -0.42% | -0.46% | 101% | 101% |
| Class C | -0.52% | -0.34% | -0.46% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -0.48% | -0.56% | 101% | 102% |
| Class D | -0.80% | -0.50% | -0.72% | 101% | 104% |

# References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, July. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).