| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0341_r1 |
| --- | --- |

| Title: | CE3-related: CCLM coefficients derivation method without down-sampling operation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, a CCLM coefficients derivation method without down-sampling operation is proposed. In which, the maximum/minimum luma template sample values are obtained by checking the neighbouring samples of current luma block, directly. Also, corresponding chroma template sample values are obtained based on the position of the selected luma template samples. Only 1 row and 1 column neighbouring samples of current luma block will be checked. Simulation results reportedly show BD rate on Y, Cb and Cr components for AI configuration over VTM2.0.1 is 0.26%, 1.81%, and 1.18%, with 100% EncT, 100% DecT;

Tests description

CCLM coefficients derivation method using maximum/minimum luma value

Different from the model coefficients derivation method using the least square, a method using the maximum (max)/minimum (min) luma template sample value is proposed in JVET-K0204[1]. Here, the method is called max-min method, for simple. In the max-min method, the max luma template sample value and the min luma template sample value are obtained by checking the values of the samples from down-sampling (as shown in Fig.1) of the neighboring samples of current luma block. Also, the corresponding chroma sample value are obtained.

Figure 1: Template samples with down-sampling

Figure 2: LM_A mode

Then the CCLM coefficients are derived as (shown in Fig.2):

α=Ci-CjLi-Lj                                         (1)

β=Cj-α*Lj                                      (2)

Max-min method without down-sampling for template samples

In the existing max-min method, to get the luma template samples’ values, 2 rows and 3 columns of neighboring luma samples of current luma block are needed for down-sampling (which is the same as the least square method of CCLM in VTM2.0.1).

In the proposed method, only 1 row and 1 column of neighboring luma samples of current luma block are needed. The max luma value and the min luma value are obtained by checking the neighboring luma sample values, directly. Which means, only the first top row and the second left column will be used to get the template samples. To further reduce the complexity, only the luma sample of which position are align with chroma samples will be checked, as shown in Fig.3.

Figure 3: Template samples without down-sampling

Experimental results

The proposed tests are implemented based on BMS2.0.1, and tested under VVC CTC [2] with VTM configuration. The results are summarized in Tabel1 and Table2,

Table 1: test results in AI configuration

Table 2: test results in RA configuration

Conclusions

This proposal propose a CCLM coefficients derivation method without down-sampling is proposed. Simulation results reportedly show BD rate on Y, Cb and Cr components for AI configuration over VTM2.0.1 is 0.26%, 1.81 %, and 1.18 %;

References

- G. Laroche, J. Taquet, C. Gisquet, and P. Onno, “Non-CE3: On cross-component linear model simplification,” JVET- K0204, Jul. 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET- K1010, Jul. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).