| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0342 |
| --- | --- |

| Title: | CE3-related: Classification-based mean value for CCLM coefficients derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Fan Mu, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com mufan1@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, a coefficients derivation method based on classification-based mean value is proposed. In the proposed method, the luma template samples are classified into two luma classes by the mean value of them. Correspondingly, two chroma classes are obtained. Then two luma mean values of the two luma classes, and two chroma mean values of the two chroma classes are obtained, respectively. CCLM coefficients are derived by the two luma mean values and the two chroma mean values. Simulation results reportedly show BD rate on Y, Cb and Cr components for AI configuration over VTM2.0.1 is x.xx%, x.xx %, and x.xx %, xxx %, xxx %;

Tests description

Classification-based mean value for CCLM coefficients derivation

Figure 1: classification and mean value for CCLM mode.

As shown in Fig.1, Lmean is the mean value of the luma template samples. The luma template samples are classified into two luma classes by Lmean. Then, two luma mean values LLmean and LRmean are obtained. Correspondingly, CLmean and CRmean are obtained. The CCLM coefficients are derived as:

α=CLmean-CRmeanLLmean-LRmean                                 (1)

β=CLmean-α*LLmean                               (2)

This coefficients derivation method also can be used in other LM-related modes, like MMLM [1]. In this case, for each luma class, two sub-classes are further obtained by the class mean value. As shown in Fig.2:

Figure 2: classification and mean value for MMLM mode.

Then two sets of coefficients are obtained as below:

α1=CLLmean-CLRmeanLLLmean-LLRmeanβ1=CLLmean-α1*LLLmean                      (3)

α2=CRLmean-CRRmeanLRLmean-LRRmeanβ2=CRLmean-α2*LRLmean                      (4)

Experimental results

The proposed tests are tested under VVC CTC [2], and the results are summarized in Tabel1, Table2, Table3 and Table4.

CCLM

Table 1: test results of CCLM in AI configuration

Table 2: test results of CCLM in RA configuration

CCLM + MMLM

Table 3: test results of CCLM + MMLM in AI configuration

Table 4: test results of CCLM + MMLM in RA configuration

Conclusions

This proposal presents a CCLM coefficients derivation method, in which, the classification-based mean value are used. This method can also be used in MMLM. Simulation results reportedly show that, for CCLM, BD rate on Y, Cb and Cr components for AI configuration over VTM2.0.1 is x.xx%, x.xx %, and x.xx %; for CCLM+MMLM, BD rate on Y, Cb and Cr components for AI configuration over VTM2.0.1 is x.xx%, x.xx %, and x.xx %;

References

- J. Chen, E. Alshina, G. Sullivan, and J. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7),” JVET- G1001, Jul. 2017.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET- K1010, Jul. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).