| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0343-v1 |
| --- | --- |

| Title: | CE4: Adaptive multi parameter motion model (Test 4.1.17) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to add motion models that are a rotation, scaling and Affine for an inter prediction. The techniques are studied as test number 1.17 in CE4. This contribution reports the test results as follow the test condition of CE4. In test case-A 3/3/4 parameters model, simulation results reportedly show that the proposed technique achieves 0.57% and 0.2% BD rate loss for random access (RA) main 10 and low delay B (LB) Main10 configurations respectively. In test case-B 3/3/6 parameters model, simulation results reportedly show that the proposed technique achieves 0.10% and 0.11% BD rate loss for RA and LB respectively. In test case-C 3/3/4/6 parameters model, simulation results reportedly show that the proposed technique achieves 0.05% and 0.0% BD rate reduction for RA and LB respectively.

Introduction

This technique was proposed in JVET Call for Proposal [1] and it was planned to study in CE4 [2]. In the CE4, the proposed techniques are categorized and tested in CE4.1 that is Affine motion compensation. This contribution reports test results.

Proposed method

Alternative scaling and rotation model (JVET-K0124)

It’s tested to combine 3-parameter model with Affine model. The 3-parameter models predicts scaling or rotation. It makes a chance to reduce an overhead of motion information.

An encoder can choose a mode in five modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3, 4 (two MVs) and 6 (three MVs) for translate mode, scaling mode, rotation mode and two Affine modes.

The motion model index is defined and it is sent to the decoder to indicate a motion model.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

As mentioned above, a rotation mode is added in this technique. If the motion in a test sequence has rotation, the inter prediction can use a rotational translate. This rotation mode can predict the rotation and it can reduce the number of parameters than affine mode.

When the rotation mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1y). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1x) is implicitly calculated by below equation. Here, W is width in CU size.
v1x=v0x-v1y-v0y22W

A framework that switching between 3(scaling)/3(rotation)/4(simplified Affine)/6(Affine) is tested. In Ljubljana meeting, 4/6 parameter model was adopted. In this test, to further study to combine with 3-parameter model, it’s  implemented on the 4/6 parameter model. And it is divided three tests to check a trade-off in the combination.

Experimental results

The software to test is implemented based on VTM-2.0.1 and it is committed in Gitlab server [3]. Three tests are prepared to separately report as below.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.17 | a) 3(scaling)/3(rotation)/4 model switching b) 3(scaling)/3(rotation)/6 model switching c) 3(scaling)/3(rotation)/4/6 model switching | K. Kondo (Sony) |  |

Each testing item is evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in [4].

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX instruction is enabled.

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Test 1.17-A

The results of test 1.17-A are summarized in the Table 1.

Table 1 : Over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.41% | 0.40% | 0.31% | 112% | 100% |
| Class A2 | 1.50% | 1.07% | 0.92% | 112% | 102% |
| Class B | 0.48% | 0.33% | 0.26% | 112% | 101% |
| Class C | 0.10% | 0.10% | -0.04% | 109% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.57% | 0.43% | 0.32% | 111% | 100% |
| Class D | 0.06% | -0.12% | -0.03% | 109% | 101% |
| Class F (optional) | 0.73% | 0.64% | 0.59% | 112% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.28% | -0.03% | -0.08% | 125% | 101% |
| Class C | 0.10% | 0.16% | -0.18% | 116% | 102% |
| Class E | 0.21% | -0.13% | -0.54% | 144% | 99% |
| Overall | 0.20% | 0.01% | -0.23% | 127% | 101% |
| Class D | 0.08% | -0.07% | -0.03% | 116% | 102% |
| Class F (optional) | 0.72% | 1.30% | 1.17% | 126% | 101% |

Test 1.17-B

The results of test 1.17-B are summarized in the Table 2.

Table 2 : Over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.23% | 0.12% | 116% | 99% |
| Class A2 | -0.03% | -0.13% | -0.08% | 115% | 101% |
| Class B | 0.16% | 0.11% | -0.01% | 116% | 100% |
| Class C | 0.08% | 0.13% | 0.05% | 112% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.09% | 0.02% | 115% | 100% |
| Class D | 0.03% | 0.04% | 0.04% | 112% | 101% |
| Class F (optional) | 0.00% | -0.05% | -0.05% | 114% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | -0.07% | -0.09% | 131% | 101% |
| Class C | 0.07% | -0.03% | 0.04% | 120% | 104% |
| Class E | 0.11% | -0.22% | -0.45% | 153% | 99% |
| Overall | 0.11% | -0.10% | -0.14% | 133% | 101% |
| Class D | 0.04% | -0.05% | -0.19% | 120% | 101% |
| Class F (optional) | -0.11% | 0.05% | 0.49% | 131% | 102% |

Test 1.17-C

The results of test 1.17-C are summarized in the Table 3.

Table 3 : Over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.07% | -0.06% | 116% | 101% |
| Class A2 | -0.18% | -0.19% | -0.25% | 115% | 101% |
| Class B | -0.01% | -0.06% | -0.17% | 116% | 101% |
| Class C | -0.02% | 0.08% | 0.06% | 112% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.10% | 115% | 100% |
| Class D | -0.05% | -0.12% | -0.05% | 112% | 103% |
| Class F (optional) | -0.02% | 0.00% | -0.02% | 115% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.07% | -0.25% | 128% | 101% |
| Class C | 0.00% | 0.00% | -0.15% | 120% | 102% |
| Class E | -0.05% | -0.17% | 0.31% | 152% | 97% |
| Overall | 0.00% | -0.07% | -0.08% | 131% | 100% |
| Class D | -0.05% | 0.20% | -0.22% | 119% | 102% |
| Class F (optional) | -0.16% | 0.37% | 0.12% | 130% | 102% |

Conclusion

This contribution reported simulation results of CE4 test 1.17, which evaluates 3 / 4 / 6 parameter switching.  To reduce the parameter, this proposed technique adds two motion models which are scaling and rotation. Three test cases were tested that a 3-para (scaling) / 3-para (rotation) / 4-para (Affine), 3 / 3 / 6-para (Affine), and 3 / 3 / 4 / 6 switching as test names A, B and C. Test-C results show that the proposed method can provide 0.05% and 0.00% BD rate reduction, 115% and 131% encoding time, 100% and 100% decoding time for RA and LB.

References

[1] T. Suzuki, M. Ikeda and K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony” JVET-J0028, JVET 10th Meeting: San Diego, US, April 2018.

[2] Haitao Yang, Shan Liu and Kai Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding” JVET-K1024, JVET 11th Meeting: Ljubljana, SI, 10–18 July 2018

[3] https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM

[4] Frank Bossen, Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video” JVET-K1010, July 2018.

Patent rights declaration(s)

Sony corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).