| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0365-v1 |
| --- | --- |

| Title: | MS-SSIM as an additional metric |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen Martin Pettersson, Rickard Sjöberg, Per Wennersten | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com per.wennersten@ericsson.com |
| Source: | Huawei Technologies Co., Ltd Ericsson |  |  |

_____________________________

- Abstract

This contribution proposes to include the MS-SSIM metric as additional metric in VTM and make MS-SSIM Y mandatory in the CTC for SDR video. A patch for MS-SSIM integrated into VTM 2.0.1 and an updated excel template for the CTC for SDR video are provided.

Introduction

Several studies over the years have shown that PSNR does not correlate very well to perceptual quality of image and video data, since the metric is not designed with the human visual system (HVS) in mind. This may lead to suboptimal decisions in the standardization process.

Besides, some perceptual quality-oriented coding tools have been proposed in JVET meetings. They may cause a PSNR loss but in fact improve the overall visual quality. To facilitate the evaluation of coding tools in this category, e.g., comparing two tools with similar performance, a perceptual metric is needed.

Among the state-of-the-art image/video quality metrics, Multi-Scale Structural SIMilarity (MS-SSIM) has been widely used and has become a benchmark in image/video quality assessment studies. It consists of a simple formula and performs stably across image/video quality databases. It was reported in JVET-C0030 [1] that MS-SSIM correlated better with subjective data compared to PSNR in 7 public image/video quality databases. It was also demonstrated in JVET-F0064 [2] that MS-SSIM was stable for small changes in subjective quality, and MS-SSIM was adopted into the JEM and HM software at the same meeting.

Proposal

This contribution recommends including MS-SSIM as an additional metric in the VTM software. Since MS-SSIM was originally designed for gray-scale images, it is suggested to use MS-SSIM Y as a mandatory metric in the CTC for SDR video. Besides, some CEs or AhGs may use MS-SSIM Y for additional results.

A patch for MS-SSIM integrated into VTM2.0.1 is provided. The implementation is the same as that in JVET-F0064.

Suggested text changes to the CTC for SDR video

The following text changes are suggested for the CTC for SDR video document. Added text is marked in yellow. An updated excel template for the CTC for SDR video is provided with the contribution.

- Reported Metrics

The attached Excel sheets contain a reporting template in which bitrate, PSNR, MS-SSIM Y, coding time and BD-rate results are reported for the tested configuration against an anchor (VTM or BMS).

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE) and MS-SSIM Y is calculated as described in JVET-F0064, 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results. This behavior is built into the reference encoder and no external conversion program is required.

For the purpose of reporting encoding and decoding running times, the anchor and test should be simulated on the same platform, e.g. similar CPU cores, to have reliable time comparison. The Excel summary sheet contains fields to report attributes of the testing environment, including CPU type, compiler, and decoder configuration. Filling in these fields when making a report is highly encouraged.

In the attached Excel sheets, BD-rate is calculated using both piece-wise cubic and cubic interpolations, and the piece-wise cubic interpolation is used in the results summary.

Conclusions

This contribution proposes to include MS-SSIM into VTM and make MS-SSIM Y mandatory in the CTC for SDR video.

References

- H. Zhang, X. Ma, Y. Zhao, H. Yang, “Perceptual Quality Assessment Metric MS-SSIM”, JVET-C0030, May 2016.

- M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn, “MS-SSIM as an additional mandatory metric to PSNR for future video coding”, JVET-F0064, April 2017.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).